Section - VII

TECHNICAL  SPECIFICATION (TS)

Table of Clauses:

200	Materials	47

210	Wire  Ropes	47

220	Blasting  Materials	47

300	Steel Work	47

310	Supply and Fabrication of Steel Parts	49

311	Structural Steel	49

312	Reinforcement Steel	49

320	Supply of Thimbles	49

330	Supply of Bulldog Grips	50

340	Miscellaneous  Supply	50

341	Wire mesh Netting for LSTB Bridge	50

342	Sign Board		50

343	Bolts, Nuts & Washers	50

351	Hot Dip Galvanization of Steel Parts	50

400	Transportation	52

411	Materials and Equipment	52

412	Wire Ropes		52

500	Construction	52

510	Site  Clearance	52

520	All  Excavation	52

521	Soil		52

522	Soft Rock		53

523	Hard Rock (by Quarrying out)	53

524	Hard Rock (by Blasting)	53

525	Excavation in Shallow Water	53

530	Foundation  Excavation	53

531	Soil (ref. 521)	53

532	Soft Rock (ref. 522)	53

533	Hard Rock by Quarrying out (ref. 523)	53

534	Hard Rock by Blasting (ref. 524)	53

535	Hard Rock by Chiseling	53

536	Backfilling		53

540	Construction  of  Gabion	53

550	Concrete  Works	54

555	Formwork		57

560	Masonry and Mortar Work	57

561	Dry Stone Masonry or Pitching	57

562	Cement  Masonry (Incl. Pos. No. 563/64)	57

565	Cement Plaster	58

570	Erection and Finishing work	58

571/2	Erection of Towers/Truss Bridge	58

573/4	Cable Hoisting	58

575	Erection of Walkway	59

576	Protection from soil and debris	59

580	Surrounding  Works	59

590	Site Installation and Supervision	59

591	Site Installation and Site Supervision	59

600	Miscellaneous Works	60

610	Gabion  Works	60

611	Dismantling of Gabion Boxes	60

620	Concrete and Masonry Work	60

621/2	Dismantling of Masonry/Concrete Structure	60

630	Pretensioning of Wire Ropes	60

640	Walkway  Adjustments	60

641/2	Replacing/Adjustment of Suspenders	60

643/4	Replacing/Adjustment of Cross Beams	61

645/6	Dismantling/Refining of the Walkway	61

647/8	Dismantling/Replacement of Wire mesh Netting	61

650	Finishing Works	61

653	Rust Prevention by Repainting	61

660	Other Miscellaneous	61

Section - VII

Technical  Specification (TS)

# 200	Materials

## 210	Wire  Ropes

Wire Ropes will be provided by the Employer form its store.  As the wire ropes are procured centrally, the specification and manufacturer’s quality assurance has already been verified.  The concerned contractor/fabricator, however, shall verify the correct dimension in diameter, length, properly cut ends, and avoid the kinks while receiving the cable from the said store according to the approved check list.

## 220	Blasting  Materials

As these materials are sensible, destructive, and requires special care under security and expert supervision during handling, storing and its use. They are procured under expert guidance and stored in Military facilities, wherever possible, their specification as per the requirement has been earlier verified and should not be of great concern to the Contractor.  Its handling shall be strictly under the HMG law and regulations.

# 300	Steel Work

Bid and actual fabrication of the bridge parts shall be based on the "Terms of Steel works" given below in addition to relevant ISI Codes. In case of discrepancies between the two the "Terms of Steelworks" shall prevail.

Safety and health provisions shall be made according to IS: 818-1968, and shall entirely be the Contractor/ Fabricator’s responsibility.

The following “Terms of steel works” has to be strictly followed while performing the steel parts fabrication work.

a)	Straightening

All materials shall be straight and if necessary, before being worked shall be straightened and/or flattened by pressure, unless required to be of curvilinear form and shall be free from twists.

b)	Cutting

Cutting may be affected by shearing, cropping or sawing. Gas cutting by mechanically controlled torch is permitted for mild steel only. Gas cutting of high tensile steel is also permitted provided special care is taken to leave sufficient metal to be removed by machining so that all metal that has been hardened by flame is removed. No loads shall be transmitted into metal (except welded joints) through a gas cut surface.

c)	Holing and Punching

Holes for bolts or pins shall be drilled (not to be formed by gas cutting). Punching may be permitted for materials with thickness < 14mm, provided the holes are punched 3 mm less in diameter than the required size and reamed after punching to the full diameter. When holes are drilled or punched in one operation through two or more separable parts, these parts shall be separated after drilling or punching and the burrs removed. To obtain uniformity the use of templates and jigs is mandatory for holing.

d)	Assembly

The component parts shall be assembled in such a manner that they are neither twisted nor otherwise damaged and shall be so prepared that the specified cambers, if any, are provided.

e)	Bolting

Where necessary washers shall be tapered or otherwise suitably shaped to give the heads of  nuts and bolts a satisfactory bearing. The threaded portion of each bolt shall project through the nut at least two threads. In all cases where the full bearing areas of the bolt is to be developed, the bolt shall be provided with a washer of sufficient thickness under the nut to avoid any threaded portion of the bolt being within the thickness of the parts bolted together. Threading of rods and holes shall be done by machines.

f)	Welding

Welding shall be in accordance with any of the following standards as appropriate, and shall be conducted by appropriately qualified welders:

IS: 816-1989	:	Code of practice for use of metal arc welding for general construction in mild steel.

IS: 823-1964 &	:	Code of practice for use of welding in bridges and structures

IS: 1024-1979	:	subject to dynamic loading.

If there are any new editions of Indian Standard those should be used as references. The use of templates and jigs is mandatory for welding of assembly. Following special requirement in conjunction with galvanization has to be made. Sections should have corners cropped to allow free flow of zinc during galvanization.

Avoid narrow gaps between plates. Overlapping surfaces and back to back angles and channels should be avoided. Where small areas are unavoidable edges should be sealed by a continuous pore free weld to prevent penetration of pickle acids.

g)	LSTB (SBD) Standard Designs

The LSTB Standard Design and Drawings are integral parts of these technical specifications. Details for the steel works can be found in the set of Standard Steel Drawings, which will be made available to the Contractor after the contract is signed / awarded.

h)	Quality Requirements, Quality Control and Quality Assurance (Guarantee)

The quality control of the raw materials acquired, fabrication process, and that of ready made product is the responsibility of the prime contractor/fabricator, who should guarantee that all raw materials are of standard quality according to Indian Standards Institution and to the specifications given in the "Terms of Steel works" and fabrication of the bridge parts are correct according to LSTB Design and Drawings. The cost for all damages due to use of low quality materials has to be borne by the contractor/fabricator. At the time of delivery the parts and materials shall have to be free from any defects in material or workmanship in all respects and shall remain reasonably perfect till the end of maintenance period when a final check shall be carried out.

If any mistakes in the LSTB (SBD) Designs and Drawings are found by the workshop, The Employer’s Technical Section has to be informed immediately in writing. The changes in steel profile have to be done only with written permission from the Employer in case of proven non-availability of certain profiles in the market.

The cost of other materials thus proposed and used will be fixed according to their respective weight and the lower among the bid price and official cost of the Employer.

i)	Inspection, Tests and Final Check

The Contractor/Fabricator shall coordinate to provide access and other facilities for the inspection and, if required, tests of the works at any/or all stages during fabrication of the steel parts. Unless otherwise agreed this inspection should be carried out at the place of fabrication. The inspection shall be facilitated by a properly accredited person (quality control manager) available at the fabricator’s floor during all working hours together with a complete set of drawings and any further instructions which may have been issued.

Following steel parts have to be in assembled form during such inspection:  The parts shall be assembled after completion of the galvanization, on a flat, level and hard top place. Once dismantled, any damage to the galvanization of the steel parts has to be rectified before delivery.

- All anchorage parts

- Walkway deck with minimum of four cross-beams in assemblage

- Both towers, complete

- Windguy stay strut

- Complete truss

- Clamps and turnbuckles

Such inspection shall be made prior to delivery and shall be conducted so as not to interfere unnecessarily with the operation of other work.  Such inspections are supplementary and intended to clear the delivery only, and shall not be considered as a final acceptance.

The final check of  all steel parts will be conducted along with the final check of the bridge and the discrepancies found during such final check have to be rectified by the Contractor/Fabricator at its own cost. On a default situation, such works shall be conducted by the Employer and the cost incurred for such works shall be made a liability in the Contractor/Fabricator’s account (either from its retention money or from other payments and/or their property according to the prevailing rules and regulations).

All gauges and templates, measuring instruments etc. necessary to satisfy the inspector shall be supplied by the manufacturer. The inspector may at his discretion check the test results obtained at the manufacturers works by independent tests and should the material so tested be found to be unsatisfactory, the costs of such tests shall be borne by the manufacturer, and, if satisfactory, the costs shall be borne by the Employer. In any case the quality assurance of Principal Manufacturer has to be made available by the Contractor/Fabricator, whenever required by the Employer.

j)	Storing, handling and protection of steel parts (specially Threads)

All structural and reinforcement steel shall be stored and handled without subjecting them to excessive stresses and damage. The threaded parts including anchorage rods, clamps, suspenders etc. have to be protected with grease and jute immediately after completion.

k)	Labeling, Packing and delivery

Each member (or set of members) has to be labeled clearly indicating both the drawing number and the part number corresponding to the steel-part list and the drawing number  provided by the Employer, but only after galvanization.

Straight bars and plates shall be bundled. All bolts, nuts, washers and other small and loose parts shall be packed in cases or strong bags and labeled in order to prevent damage or distortion during transportation. The costs of packing and labeling shall be included in the bids. The Wire mesh netting has to be delivered in bundles. Each transport unit shall not exceed 45 kg in weight. For each delivery the workshop has to prepare a detailed list of the bridge parts to be delivered, containing the drawing numbers, part numbers, description of parts, quantities and weights.

## 310	Supply and Fabrication of Steel Parts

### 311	Structural Steel

Structural steel supplied by the workshop shall comply with the requirements of IS: 226-1975, specification for Structural Steel (Standard Quality). The dimensions for hot rolled materials are specified in IS: 800-1984 & 808-1989 (General construction in steel). Steel grade shall be standard quality FE 410, and should have following properties:

Yield stress ³ 250 N/mm2

Tensile strength ³ 410 N/mm2

Elongation ³ 23%

Permissible tolerance of physical dimension = ± 2.5%

### 312	Reinforcement Steel

Reinforcement steel supplied by the workshop shall be ribbed torsteel with high yield strength deformed bar of grade FE 415 confirming to IS: 1786-1985. All reinforcement shall be clean and free from loose mill-scales, dust, loose rust and coats of paint, oil or other coatings which may destroy or reduce bond. All reinforcement steel shall be cold bent. Points for anchor rods shall be formed by grinding.  Joining of reinforcement steel bars by welding is prohibited.

## 320	Supply of Thimbles

Thimbles supplied by the workshop shall conform to the standard specifications of Thimbles (IS: 2315-1978). The workshop is responsible that all thimbles can be fitted to the provided pins, steel parts etc. Thimbles should be hot dip galvanized with minimum zinc coat of 40mm.

## 330	Supply of Bulldog Grips

Bulldog grips supplied by the workshop shall confirm to the standard specifications of Bulldog grips (IS: 2361-1970). In addition it also should fulfill the following requirements:

The bridges must be drop-forged and suitably scored to grip a round strand rope of right hand lay having six strands. Bridge, U-bolts and nuts should be hot dip galvanized with minimum zinc coat of 40mm. The thread of U-bolt and nuts on it should withstand specific torque.

The bulldog grips should have minimum grip strength of :

13mm  ³ 2.7 tone,  26mm  ³ 6.6 tone,  32mm  ³ 8.3 tone,

36mm  ³ 9.0 tone, 40mm  ³ 9.7 tone,

Note: 	Thimbles, bulldog grips, nuts, bolts and washers have to be galvanized according to IS: 2629-1966,     and Recommended practice for Hot-dip Galvanizing of Iron and Steel.

## 340	Miscellaneous  Supply

### 341	Wire mesh Netting for LSTB Bridge

The wire used for fabrication of wire mesh netting shall be of 10 SWG (3.15 mm), conforming to IS: 280-1978, heavy coat galvanized according to IS: 4826-1979 (hot dip galvanized, minimum zinc coating of 270 gm/m2 and should pass uniformity of zinc coat and adherence test). In addition, G.I. wire should have tensile strength of 380-500N/mm2. The meshes shall be of chain link type, mesh size 60 x 60 mm. The width shall be 1.30 m for suspended bridge and 1.2 m for suspension bridge.

### 342	Sign Board

Only if necessary.

### 343	Bolts, Nuts & Washers

Bolts, nuts, & washers shall confirm the standard specification of  bolts, screws, nuts & washers (IS: 1363-1984 for hexagon head bolts) / (IS: 1367-1979/1980 conditions for threaded fasteners) / (IS: 6610-1972 for heavy washers) / (IS: 5369-1975 for plane & lock washers) /( IS: 5372-1975 for taper washers). Bolts, nuts & washers shall be hot dip galvanized unless otherwise stated.

### 351	Hot Dip Galvanization of Steel Parts

Galvanizing of steel parts shall be executed in accordance with IS: 2629-1966 (Recommended Practice for Hot-Dip Galvanizing of Iron & Steel) and IS 4759-1984 (specification for hot-dip zinc coating on structural steel).

a)	Galvanization Process

Galvanizing should follow the process as below :

Alkaline cleaning

Cleaning (Rinsing) with water

Acid Bath

Preparatory work

Cleaning (Rinsing) with water

Pre-flux treatment

Dry

Galvanization

Cooling with water

- Preparatory work

All welding slag should be removed from the steel parts to be galvanized. Acid traps should be avoided.

- Alkaline cleaning

Rust, oil and other contaminants should be removed from the steel by preliminary treatment with Alkaline cleaning in Sodium Hydroxide Solution of 10 ~ 15% concentration.

The concentration of the solution should be monitored at regular intervals and adjusted accordingly to have good cleaning effect.

- Rinsing

After Alkaline cleaning, steel should be flushed with running water.

- Acid Bath

The preliminary treated steel is then cleaned by Acid Bath in Hydrochloric Acid Solution of 50% concentration.

During the operation, close control of acid content in the solution is necessary. It is done by testing for acid (PH value = 1 to 3, measured by PH meter) and iron contents at regular intervals. The strength of the solution should be maintained by periodic addition of fresh concentrated acid. If the iron content in the solution is more than 100g/liter (density measured by hydrometer, maximum permissible value = 1.2), it should be changed by the new fresh solution.

- Rinsing

After Acid Bath steel should be flushed with running water.

- Pre-flux treatment

Acid bathed steel is dipped in a solution of Zinc Ammonium Chloride (ZnCl23NH4Cl) of 20 ~ 40% concentration. This solution is prepared by mixing 45% of Zinc Chloride (Zncl2) + 55% of Ammonium Chloride (NH4Cl).

The concentration of solution should be controlled at regular intervals. For this specific gravity of the solution is maintained (Hydrometer reading in range of 1.1 to 1.15 ) by adding required quantities of pre-flux chemicals in proportions.

- Dry

After pre-flux treatment, steel is dried for galvanization.

- Galvanization

The treated steel is dipped into the molted Zinc (zinc bath). Temperature of the molted Zinc should be maintained within the range of 4500 C to 4700 C. Appropriate dipping time also should be maintained.

- Cooling

After the hot dip, steel is cooled with water.

- Quality Checking of Galvanization

- Galvanizing Process

Check the galvanizing plant and its set up to be ensured that all the galvanizing process is available and maintained.

- Quality of Zinc

This can be checked by chemical testing of raw zinc used for the galvanization.  Zinc content should not be less than 98.5%.

- Quality of galvanization

By visual check the following:

-	Galvanized steel should be free from rust and black spots, acid traps, welding slags.

-	Galvanized coating should be free from ash, pimple and bulky deposition.

-	Galvanized coating should be continuous, smooth and evenly distributed as far as possible.

- Zinc coat should be adherent.  This can be checked by hammering or scratching.

- No zinc paint or almonium paint is applied to hide rust spots. This is very unhealthy practice, which frequently applied by the galvanizers.

- Zinc thickness can be measured by Deltascope (electromagnetic instrument for measuring coating thickness over ferrous material).  In our case zinc coat should not be less than 80 µm in any steel parts.

- Treatment of Threads:  Extra zinc on threads should be removed with a rotating wire brush immediately after galvanizing or with gas blow and cleaning with rotating brush if galvanization is already hardened.  Re-threading of the galvanized parts is not permitted.

# 400	Transportation

It is wholly the responsibility of the Contractor to control transportation activities and quality/quantity of any materials procured or received from the Employer’s stores, loading and off-loading of the same, and the delivery at designated site.

### 411	Materials and Equipment

Materials, such as cement and steel parts should be transported with due attention so as not to incur any damage to them either due to exposure to excessive moisture or mishandling.  The cement bags to be delivered at site should contain its full original weight viz. 50 kg. The Contractor shall entirely be responsible for the quality and quantity of materials received at the point of origin and their delivery at site in the same quantity and quality. No reasons whatsoever will be accepted for materials not reaching to site in proper condition and in required quantity.  Loss during transportation, if any is entirely on the Contractor’s account.

Other materials and equipment shall be transported properly so as to minimize the loss or damage during transportation.  Work being delayed because of equipment being damaged during transportation is Contractor’s concern which has to be considered during bidding and shall not be considered a reason for time extension.

In the case of blasting materials, the transportation, handling, storage and use of explosives shall be subject to the laws and regulations of the Government as well as to the instructions of the  Employer’s Engineer.  The Contractor shall maintain an inventory record of storage and withdrawal of all explosives and such information shall be duly communicated to the Engineer.  The Contractor shall provide such reasonable and adequate protective facilities as are necessary to prevent loss, theft or damage of explosives. The Engineer shall be informed promptly of any such loss. Security provision should be made accordingly at the Contractor’s cost.

### 412	Wire Ropes

Wire ropes supplied from Employer’s store shall be under direct responsibility of the Contractor till it is fitted in the bridge.  The Contractor must transport them immediately to site with proper handling so that cable is not damaged, kinked, or the loose ends opened.  It is the Contractors responsibility to check cutting, coiling, and the quantities of cable received from Employer’s store, their loading and unloading.

# 500	Construction

The construction works includes civil, mechanical, finishing works together with erection, running and maintenance of the camp.

## 510	Site  Clearance

The areas to be occupied by the permanent works and any borrow areas shall be cleared of trees, stumps, roots, vegetation, structures and other obstructions to the extent required by the design. All materials thus removed  during site clearance shall be the property of the Employer.

## 520	All  Excavation

All excavation has to be carried out according to design, designed cut slope , designed provision of shuttering, and the  safety has to be maintained. All excavation  has been classified as follows.

### 521	Soil

It is when soil can be removed by shovels, picks or which include big stones of a size which require moving by hand.

### 522	Soft Rock

It is when bed rock can  be removed by means of crowbars and picks without any blasting.

### 523	Hard Rock (by Quarrying out)

This includes quarrying by wedging.

### 524	Hard Rock (by Blasting)

The hard rock encountered in foundation excavation has to be removed by blasting it with industrial gelatin, detonator, and adequate length of fuse wire.

### 525	Excavation in Shallow Water

It is defined as excavation below the ground water level and includes all necessary measures for dewatering like water courses, ditches, rock pits but without provision and operation of pumps.

## 530	Foundation  Excavation

Excavation for the permanent construction such as foundation blocks shall be made to the lines, grades and dimensions shown on the drawings.  All necessary precautions (including control of blasting) shall be taken to preserve the material below and beyond the established lines of all excavation and also to the safety of workers and others.

When excavated to the required levels and dimensions, the surface has to be cleaned, prepared, and approved by the Engineer prior to the placing of concrete, masonry, or any other filling materials.

The classification of excavation are similar to the aforementioned and listed under following headings

### 531	Soil (ref. 521)

### 532	Soft Rock (ref. 522)

### 533	Hard Rock by Quarrying out (ref. 523)

### 534	Hard Rock by Blasting (ref. 524)

### 535	Hard Rock by Chiseling

Hard rock which can not be removed by wedging and due to other constraints can not be blasted, such surface has to be chiseled to acquire the required level and dimensions as per the design, i.e., for finishing the foundation surfaces.

### 536	Backfilling

Common material  generally shall be used for backfilling behind civil structures and  behind a filter bed of gravel fill as shall be shown in the drawing.  Fill material shall be placed in  position shown on the drawings and will be obtained from designated borrow areas, excavation on site or any other suitable area, as designated by the design and directed by the Engineer. Backfilling shall be of continuous compacted layers not exceeding 300 mm thickness. In case of retaining wall, proper filter materials for weep holes are to be provided in order to drain out water behind the wall.

All local material required for the construction of the work shall have to be obtained from borrow areas beyond a minimum haulage distance of 30 meters. The Contractor is responsible for  any kind of river bank erosion which ultimately may endanger the stability of foundations, as a result of carelessness in collecting local material from such critical areas.

## 540	Construction  of  Gabion

a)	Gabion Boxes

The gabion boxes shall be machine woven rectangular units made from double twist hexagonal mesh of heavy galvanized mild steel wire. All edges of the standard gabion including the diaphragms, if any, should be mechanically selvedged in such way as to prevent unraveling of the mesh and to develop the full strength of the mesh.

The wire used for manufacturing of the gabions shall be annealed mild steel complying with BS 1052:1990 or NS 169:2045 with an average strength of 380-500 N/mm2 and shall be galvanized before weaving as specified in BS 443:1982 of NS 163:2045.

The adhesion of the zinc coating shall be such that, when the wire is wrapped six turns around a mandrel of four times the diameter to the wire, it does not flake or crack to such an extent that any flakes of zinc can be removed by rubbing with the bare fingers. Further, the G.I. wire also should pass the uniformity test.

The combination of the mesh type and mesh wire diameter and ultimate failure load shall be as the table below:

| Mesh type (cm X cm) | Mesh wire diameter (mm) | Selvedged wire diameter (mm) | Ultimate tensile strength (kN/m) |
| --- | --- | --- | --- |
| 10 x 12 | 3.00 | 3.9 | 43.00 |
| 6 x 8 | 2.2 | 2.7 | 35.00 |

The tightness of the twisted joints should be such that a force of not less than 1.7 kN is required to pull one and separate it from other wire provided that each wire is prevented from turning and the whole process is done in one plane.  All the gabions of lengths more than 2.00 m should contain diaphragms spaced at a distance of not more than 1.00 m.

The fundamental technical specification of the gabion boxes of the required size should be as follows :

| Box size | Mesh type (cm X cm) | Mesh wire | Selvedged wire | Lacing wire | Zinc coating | No. of diaphragms |
| --- | --- | --- | --- | --- | --- | --- |
| 2 x 1 x 1 | 10 x 12 hexagonal | 3.0 mm | 3.9 mm | 2.4 mm | 275 gm/m2 | 1 |
| 3 x 1 x 1 | 10 x 12 hexagonal | 3.0 mm | 3.9 mm | 2.4 mm | 275 gm/m2 | 2 |
| 3 x 2 x 0.3 | 6 x 8 hexagonal | 2.2 mm | 2.7 mm | 2.2 mm | 240 gm/m2 | 2 |
| 4 x 2 x 0.3 | 6 x 8 hexagonal | 2.2 mm | 2.7 mm | 2.2 mm | 240 gm/m2 | 3 |
| 5 x 2 x 0.3 | 6 x 8 hexagonal | 2.2 mm | 2.7 mm | 2.2 mm | 240 gm/m2 | 4 |

b)	Laying & Packing of Gabion Boxes

Layout of gabion work shall be as per design/site condition and as instructed by the Engineer. Gabion boxes shall be laid in staggered manner. Back slope of 10:1 (V:H) shall be maintained in gabion wall. Gabion mattress shall be embedded and flushed with the river bed level. Before the crates are packed, the selvedges of one crate shall be bound to the selvedges of another adjacent crates.  When the crates are packed with boulders, the lids will be closed and the selvedges bound with binding wire.  The packing shall be with hammer dressed sound boulders, placed individually by hand, bedded on all sides on each other and bonded tightly.  No loose stones shall be inserted into the crate. Both end of the gabion walls of river bank protection shall be keyed with the river bank.

## 550	Concrete  Works

Concrete shall be composed of cement, sand, coarse aggregate and mixed with water as specified herein and shall be well mixed to an even consistency as required by this specification and/or as directed by the Engineer.  No concrete shall be placed in any section of the works until the foundation for that section has been completely dewatered, suitably prepared, and has been approved by the Engineer.

The below given specifications of concrete components are for all type of concrete classification that are most commonly used in SBD works. They are:

| Pos. No. | Description | Mark | Code |
| --- | --- | --- | --- |
| - 551 | Plain Lean Concrete 1:4:8 | M75 | All items |
| - 552 | Plain Mass Concrete 1:3:6 | M100 | according to |
| - 553 | Reinforced Cement Concrete 1:2:4 | M150 | ISC-456: |
| - 554 | Plumb Concrete with 60% PMC and 40% boulders |  |  |

In plumb concrete, the minimum gap of 100 mm between the adjacent boulders should be fully maintained and that the boulders are not in contact with each other.  And these gaps between the boulders are filled with concrete of designed mix and rammed adequately avoiding any voids. Each layer of boulders should be placed on 100 mm thick concrete mats.

Completed concrete structures should be sound without voids (honey combs), wall vertical to plumb and line and have overall finishing.

a)	Cement

Ordinary Portland cement conforming to IS 269-1989, without any warps or any sign of air setting shall only be used for all works under this contract.  Cement shall be transported and stored without exposing it directly to rain or excessive moisture and should be stacked on wooden platforms at least 5.0 cm above the ground.  It shall not be stored for a long time, which might set  the cement, reducing its strength. The Contractor shall build or maintain suitable store on site.  A result of laboratory test may be required by the Employer.

b)	Sand (conforming to IS 383:1970)

Collection

This item shall include quarrying, collecting, sieving, washing of sand, if required and haulage to site.

Quality requirement

Sand selected must be free from mica, loam, clay, mud and organic matter.

Washing

Contaminating  impurities like clay, loam, mud,  have to be washed.  The quality of river sand shall have to be approved by the Engineer before the haulage.

c)	Coarse Aggregate (conforming to IS 383:1970)

Collection

Gravel can be collected from river deposits, quarries or by breaking boulders into required size as defined by the design. Aggregate of unsuitable rock, such as mica, marl, and sand stone shall not be used.

Surface

Gravel with rough and triangular surface are preferred for good bondage. Flat and flaky particles are not to be used.

Purity

Gravel should be free from organic contaminants, deleterious substances like loam, clay, mud or stone dust and other organic matter.

Grading

Aggregate shall be well graded. The approximate % requirement of gravel size, may they be from river bed or broken shall be

5	-	10 mm	12.5	%

10	-	20 mm	25.0	%

20	-	40 mm	62.5	%

d)	Stone

Quality requirement

All stones shall consist of unweathered, hard, dense, durable, uncoated and sound rock fragments. The stones shall be thoroughly wetted by soaking in water immediately prior to placing.

Gabions and Rubble masonry shall be made of rock fragments which are  reasonably rectangular in shape.

Dry stone masonry and block stone masonry shall be made of all side dressed blocks with facing side made smooth, either out of boulders or out of stratified rock.

Size

For masonry, the size of stone shall be as per IS code specification i.e. max. length > 3H, B > ¾ thickness of wall.

The size of stone for gabion work shall be between 100 mm and 200 mm.

Stones for plumb concrete shall be rock fragments between 100 and 225 mm in size.

e)	Water

Water for washing aggregates, mixing, and curing concrete shall be fresh and shall be free from objectionable quantities of silt, organic matter, alkalis, salts and other impurities.

f)	Mixing and Placing

The proportions of cement, sand, coarse aggregate and water for the concrete mix of any classification shall be  as given in the Table below. Measurement shall be in volume.

|  | Concrete Grade |  |  |  |
| --- | --- | --- | --- | --- |
| Material Required (m3) | 1:4:8 Plain lean M - 75 | 1:3:6 Plain mass M - 100 | 1:3:6 40% Boulder | 1:2:4 Rein- forced |
| Gravel - 10 mm (m3) | 0.11 | 0.11 | 0.07 | 0.11 |
| Gravel - 20 mm m3) | 0.23 | 0.23 | 0.14 | 0.22 |
| Gravel - 40 mm (m3) | 0.55 | 0.55 | 0.33 | 0.52 |
| Sand (m3) | 0.47 | 0.47 | 0.28 | 0.445 |
| Boulders-225 mm (m3) | - | - | 0.50 | - |
| Water (aprx.) (Lt) | 150 | 150 | 100 | 200 |
| Cement (Kg) | 170 | 220 | 132 | 320 |

|  |  | Permissible Stress |  | N/cm2 |
| --- | --- | --- | --- | --- |
| Concrete Grade |  | 1:3:6 | 1:3:6 + 40% Boulder | 1:2:4 |
| Stress in | bending | 300 | 200 | 500 |
| Compression | direct | 250 | 150 | 400 |
| Stress in bond |  | 40 | 20 | 60 |
| Tension stress in bending |  | 30 | 20 | 50 |

Slump test shall be carried out to confirm workability and water cement ratio. Slump should not exceed 80 mm.

Concrete shall not be dropped freely from a height of more than 1.5m. Concrete shall be placed in horizontal layers not exceeding 25mm and shall be consolidated immediately to its full depth and worked into corners and angles of forms, all around the reinforcement and embedded parts. Consolidation shall be achieved according to the requirement.

Concreting shall take place during favorable weather conditions. Working joints shall be thoroughly maintained so as to keep the required design bond and strength. The working joints be roughened, cleaned and wetted before further work commences.

Concrete shall be mixed  on a cleanly prepared platform and  transported  from the mixing area to the area of placement as quickly as possible.

g)	Curing and Protection

Concrete shall be cured by maintaining the surface wet (using suitable and approved method) for at least 7 days. Concrete shall be protected from direct rain for 12 hours, from flowing water for 14 days. The contractor shall ensure that no structural and physical damage be inflicted on it.

h)	Reinforcement and Steel Parts

The contractor shall assemble and fix the steel parts and the reinforcement (conforming to 312) as shown on the drawings.  Reinforcement and steel parts stored in neat racks above the ground shall be cleaned of all loose rust, mill scale, dirt, grease, paint and other foreign substances and shall be maintained in a clean condition until embedded in concrete.

### 555	Formwork

Formwork shall be made from functional, straight and stout timber uniform enough to leave a smooth surface later on.

Formwork shall withstand the pressure resulting from the placement and vibration of the concrete,  maintained rigidly in position, and shall be sufficiently tight to  prevent the loss of mortar or the water content of the concrete.  The Contractor shall ensure the safety and quality during the work. Form work shall only be removed after the minimum stripping times which shall be:

-	Non load bearing members and side forms	:	3 days

-	Load bearing members up to 3m span 	:	10 days

-	Load bearing members up to 6m span 	:	20 days

-	Load bearing members above 6m span 	:	30 days

## 560	Masonry and Mortar Work

### 561	Dry Stone Masonry or Pitching

Stones must be laid with the largest possible areas of contact to provide sufficient friction.  Special attention shall be given to bonding stones of which at least five should be inserted per cubic meter of masonry.  Chips and sprawls of stones should be wedged into the work wherever necessary for its stability, but should be less than 10% by volume of the masonry.  The base of walls shall be stones with large bearing areas to provide efficient load transfer to the ground.  The top of the walls shall be covered with neatly dressed flag stones laid in such  a way that they cannot be dislodged by animals or people climbing over them.

### 562	Cement  Masonry (Incl. Pos. No. 563/64)

Cement mortar must be mixed on a  clean surface in a ratio of  one part of cement and four parts of sand (for Block stone and Rubble masonry 1:4) and one part of cement with six parts of  sand (for Rubble masonry 1:6), by volume.

One cubic meter of masonry has to contain:

|  | 1 : 4 |  | 1 : 6 |  |
| --- | --- | --- | --- | --- |
| Cement | 114 kg |  | 75 kg |  |
| Sand | 0.42 m3 | (loose) | 0.47 m3 | (loose) |
| Stones | 1.1 m3 |  | 1.1 m3 | (loose) |

Face stones shall not be narrower or shorter than its height. Joints  should not exceed 1cm in thickness. Stones shall be soaked in water prior to putting them in masonry. Stones shall not be less than 10cm in any direction, and should be carefully laid, hammered down into place with wooden mallets, and solidly bedded with mortar. Chips and sprawls shall be wedged in to avoid thick beds of joints and mortar.  Exposed faces shall be flush pointed and smooth. A proper bond shall be maintained with at least five bonding stones in a cubic meter of masonry. The whole structure shall be cured for 7 days.

|  |  | Permissible Stress N/cm2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Masonry |  | Rubble Masonry |  | Rubble Masonry |  | Block Stone Masonry 1:4 |  |  |  |
|  |  | 1:6 |  | 1:4 |  | Stratified blocks |  | Non-str. blocks |  |
| Slenderness ratio h/d |  | 0.5 | 2.0 | 0.5 | 2.0 | 0.5 | 2.0 | 0.5 | 2.0 |
| Stress in | Bending | 130 | 70 | 190 | 110 | 210 | 130 | 370 | 200 |
| Compression | Direct | 100 | 50 | 140 | 80 | 160 | 100 | 380 | 150 |
| Tension stress in bending |  | 13 | 7 | 19 | 11 | 21 | 13 | 37 | 20 |
| h=height of wall |  | d = thickness of wall |  |  |  |  |  |  |  |

### 565	Cement Plaster

Surface  mortar shall be applied to concrete and masonry as indicated in the drawings. Before application of mortar, surfaces shall be without efflorescence, clean, scored and well wetted.  Ordinary Portland cement shall be used.  Suitable sand has to be transported from the nearest possible source, sheaved to get the correct granular composition, and approved by the Engineer. Surface mortar shall be moist cured and protected from damage for at least 7 days.

In general, cement plaster (rendering) is executed in three  steps:  Render coat, leveling coat and finishing coat.  For structures with rough and absorbent surfaces, the render coat may not be required if so directed by the Engineer.  The render coat consists of a 3-5 mm thick, rich mortar paste with 600 to 800 kg cement per m3 of finished mortar and with sand 0-8 mm (mortar 1:2.5).  The surface shall remain rough to ensure a good bonding to the next coat.  The leveling coat should be 10 to 15 mm thick and shall be applied only after shrinkage cracks have appeared in the render coat.  Dosage: Cement 250-300 kg/m3, sand 0-6 mm (mortar 1:6).  The finishing coat shall provide an aesthetic finish and shall be applied to the leveling coat as soon as possible.  Thickness 2 to 5 mm. Dosage: Cement  500-600kg/m3,  sand 0-3 mm (mortar 1:3).  The average mortar shall contain in this case the cement to sand ratio of 1:4.

## 570	Erection and Finishing work

### 571/2	Erection of Towers/Truss Bridge

The Contractor shall be fully responsible for all the necessary safety precautions including the proper  temporary fixation of the towers (they are hinged) with the aid of cables anchored to specially designed anchorage blocks and the erection of the truss component of a steel truss bridge. The erection can be executed either with the aid of a bamboo scaffolding or with a suspended platform.

### 573/4	Cable Hoisting

This includes hoisting of all cables together with the placing and fixing of thimbles, bulldog grips and pretensioning.

The Contractor shall hoist the cable across the river, over onto the top of towers, further to anchorage blocks strictly according to a accepted method. The work shall include the correct fitting of all cable terminals either with thimbles and bulldog grips or with sockets. The work shall also include the pretensioning (proper setting of design sag) of main cables, windguy/windtie cables, spanning cables and sidestay cables along with the adjustment of stabilizing cables, diagonal stabilizing cables, the correct fitting of handrail and fixation cables as indicated in the drawings.  Safety of the workers, materials, and others should be of prime concern during this work, the responsibility for which entirely lies with the Contractor.

### 575	Erection of Walkway

The  Contractor  shall  be  fully  responsible for all the necessary safety precautions during the erection of the walkway.   The steel deck has to be properly fitted as specified in the drawings including suspender and cross beams.  The wire mesh netting shall be folded around to the handrail cable fixed to the fixation cables and suspenders with binding wire of 12 SWG @ an interval of 30 cm.

All clamps, bulldog grips, nuts etc. shall be retightened upon the completion of the bridge.

Permitted Tolerance for towers after erection of the deck shall be as follows:

-	Deviation from verticality	:	0.20%  of the tower height

-	Deviation from straightness	:	0.10%  of the tower height

Permitted deviations  from  the hoisting sag specified in the drawings shall be:

-	For suspension bridges	:	+ 0.04%  of the span

-	For suspended bridges 	:	+ 0.02%  of the span

### 576	Protection from soil and debris

All anchorages and foundations shall be cleaned of soil, mud and debris and all possible measures shall be taken to avoid steel parts, cable, etc. from being covered by soil, mud or debris.

## 580	Surrounding  Works

As soon as practicable, after cutting of a permanent slope, backfilling or the forming of an embankment slope, the Contractor shall encourage the growth of a vegetation cover by turfing or grass showing, transplanting the slope with ready made turf properly cut from another area, and afforestation (planting of trees and plants) as shown in the drawings or as directed by the Engineer. This work includes the care and protection of the plantations from any damage until the end of the period of maintenance.  Additional slope and river bank protection works, such as, wicker work fences, brushwood work, and terracing may be required if indicated in the design.

## 590	Site Installation and Supervision

This item shall cover the entire erection or hiring of camp, its regular running and maintenance along with the technical supervision of the whole work package.

### 591	Site Installation and Site Supervision

Camp Construction and Hiring of Camp :

A proper camp site above high flood level and safe from bank erosion shall be established.

The Contractor has to arrange an adequate camp and the necessary equipment for him and his own staff, along with a simple provision of lodging for the visiting technicians of the Employer, at his own cost.

Camp area should be sufficient for store room for storing cement and steel parts.  The rooms must be with waterproof roofing and compacted and moisture proof floor. No surface runoff or any other water should enter the cement store.

Land Rent /Compensation/ Acquisition :

The Contractor has to pay a negotiated land rent/compensation  based on the crops lost during the use of land for construction to the  land owner. Contractor shall pay them directly and its cost should be included within the camp item.  In case of permanent structure occupying a piece of land the acquisition/ “Muwabja” shall be according to the existing HMG practices and rules/regulations. The implementation of procedure for the land acquisition shall be the responsibility of the Employer. Written permission of land owner or VDC with its cost has to be incorporated in the cost estimate. Employer’s support is limited to make available such commitments only.

Supervision :

Supervision includes the supervision of fabrication of steel parts, transportation, construction and bridge erection by competent personnel. The entire supervision is the Contractor’s responsibility.

The Contractor shall appoint suitably qualified and experienced in the field technician as site staff (min. level overseer) for the purpose of daily supervision of works in each concerned phase.

The site supervisor has to devote full time on the site for the entire project period whereas an engineer of the Contractor has to visit the site for key supervision from time to time and according to the necessity.

Site supervisor/ engineer has to skill fully perform layout, measurement, quality control, test of material, monitor the execution of works as per drawing/ specifications/ schedule, and report the progress in due time to  the Employer.

The Contractor shall lay out the designed bridge at the construction site based on the existing survey pegs and reference points. After completion of general layout the Contractor shall set out all centering and levels and shall work the same with peg level.  Pegs shall be provided close to each structure.  The Contractor shall not commence any work until the setting out is complete and checked by the Engineer.

The Contractor shall assist any authorized inspector from the Employer who is inspecting the site.  All necessary information and documents must be provided on request and full cooperation on all matters should be given promptly.

# 600	Miscellaneous Works

(Applicable for maintenance works only)

## 610	Gabion  Works

### 611	Dismantling of Gabion Boxes

#### The existing gabion crates shall be dismantled before any new work could be carried out.  The lid of the crate shall be opened, stones properly removed and stacked for future use, if necessary.  The crates themselves have to disintegrated from the good ones that are still functional. Useless boxes shall be dismantled according to the instruction by the Engineer.

## 620	Concrete and Masonry Work

### 621/2	Dismantling of Masonry/Concrete Structure

In maintenance work, some part of masonry or/ and concrete has to be dismantled and an entirely new structure or a partial addition of masonry or/ and concrete has to be made.  Such cases require the dismantling of masonry and concrete structure.  This dismantling could be difficult, and shall be performed very carefully so as not to disturb other remaining structures and shall be as designed or instructed by the Engineer.

## 630	Pretensioning of Wire Ropes

The required pretension in any loose cable shall be achieved by pulling them, at first with the use of turnbuckles.  If the required pretension can not be achieved only by adjusting the turnbuckle,  the cable end shall be pulled with the help of a cable pulling machine until the designed pretension is achieved, the cable ends properly secured on the corresponding anchorages.  This has to be performed according to the Engineer’s advice and consent regarding the process and method to be use for the retightening of any cable to required pretensioning.

## 640	Walkway  Adjustments

### 641/2	Replacing/Adjustment of Suspenders

#### To replace the existing suspenders in order to install a new one, the clamps at the top cable and I or J bolt at the bottom shall be opened, the entire suspender properly dismantled and a new one fitted exactly at the required place.  While replacing or adjusting the suspenders  the verticality of the suspender as well as the clear span between two suspenders shall be maintained.

### 643/4	Replacing/Adjustment of Cross Beams

#### The replacement and/or adjustment of the cross beam shall be done as defined by the design. Adjacent walkway elements of the beam which has to be replaced or adjusted shall be opened, the beam then shall be adjusted or replaced with a new one.  The hook connecting the beam with the suspender shall be properly fitted so as to control the beam to beam distance, verticality of suspenders, and the alignment (parallel) of the beams themselves so that the standard pieces of the steel walkway deck fits in properly.

### 645/6	Dismantling/Refining of the Walkway

#### In case of maintenance of the walkway, or replacing the wooden walkway with a new steel deck the entire walkway shall be dismantled.  Existing walkway planks, nailing strips, and the cross flats shall be dismantled carefully.  The remaining cross beam shall be adjusted, cleaned and painted as designed.  New walkway either wooden or steel deck shall be properly fixed as per the design. This is equivalent to initial walkway fitting. This shall include removal of old nuts and bolts of wooden deck to be dismantled.

### 647/8	Dismantling/Replacement of Wire mesh Netting

#### Useless wire mesh netting shall be opened, unrolled  and removed from the bridge walkway area.  New wire mesh shall be properly fixed by using binding wire which shall be woven along the wire mesh and handrail/fixation cable. The wire mesh net has to be folded around the hand rail cable.

## 650	Finishing Works

#### For item Nos. 651-retightening of bulldog grips, nuts, etc.; 652-coaltar application; and repainting of steel parts, a reference shall be made to item No. 576/ 350. However, because of difficulty of painting surfaces without any access at site, it shall be done as mentioned in specific specification regarding maintenance of bridges.

### 653	Rust Prevention by Repainting

Surface to be painted or otherwise treated shall be dry, thoroughly cleaned and free from oil, grease and swarf removed properly by degreasing agents. Surfaces accessible to mechanical power tool operation shall be cleaned by employing power-operated wire brushes and inaccessible surfaces shall be cleaned by hand scraping, wire brushing, and by using coarse sandpaper, steel wool and emery paper.

Cleaned surface shall have a light metallic lustre. Inaccessible surfaces except the interior of sealed hollow structures shall receive the full protective treatment before assembly. Painting shall not be done prior to welding.  Welds and adjacent parent metal shall not be painted prior to deslagging. Parts to be encased in concrete shall not be painted or oiled (refer to standard drawing and steel parts lists to identify such parts).

Two base coats of "red lead primer" or “red oxide zinc chromate primer” and two finishing coats of "polyurethane" enamel paint has to be applied with the first base coat applied not more than 5 hours after cleaning of the parts. The minimum thickness of all 4 coats together shall not be less than 0.12 mm. The first base coat has to be applied with brush or roller by hand. Between the two applications of the base coats, a time span of at least 3 days has to be maintained.

Polyutherane enamel should be sky blue.

# 660	Other Miscellaneous

River Diversion :

During construction of structures in or adjacent to the river bed, the Contractor will have to control the river and divert it by constructing necessary cofferdams, channels, flumes drains, sumps and any other temporary protective work according to the design.

Approach Trails :

The extension of existing trails or construction of new trails shall be executed at the locations indicated on the drawings. Trail width will normally be 2.0 m but not less than 1.5 m. Appropriate measures shall be taken to protect the trails from erosion.