# 12.4.1	 Suspended Bridges

DESIGN OF MAIN AND HANDRAIL CABLE STRUCTURES [7 . 3]

Initial layout data		[7. 3. 6]

According to General Arrangement

## Foundation locations

On right bank, 7.75 m behind of axis point A

On left bank, 16.59 m behind of axis point B

## Nominal span,

S  = survey distance AB + 7.75 + 16.59

S = 132.10    m

= 107.76 + 7.75 + 16.59

## Design span between saddles,  ℓ

ℓ  = s + 2  0.25 = 132.1 + 0. 5	ℓ = 132.60  m

Cable elevations  [from topography]

Main cables	right bank, H1 = 110.85-0.25	H1 = 111.60  m

left bank, 	H2 = 105.85-0.25   	H2 = 105.60  m

Windguy cables right bank, upstream	=   102.60  m

##### downstream	= 102.30  m

##### Left bank,	upstream	=  97.50  m

##### downstream	=  96.40  m

### Different in main cable elevations between saddles, h

h = H1 – H2 = 110.6 – 105.6	h =   5.0  m

Check for h  [7. 3. 5 B]

ℓ14       	 o.k.!

i.e. the lowest point remains inside the bridge span.

### Pre-calculation of cable tensions

### Main and handrail cables

Approximate maximum cable tension

##### Tmax (approx.) = 11  S = 11   132.1	Tmax = 1450  kN

Number and diameter of cables

nm  = 4

Number of main cables

m  = 40

Diameter of main cable

=  2

Number of handrail cables

Diameter of handrail cable

H  = 26

## Permissible tension (all cables)     	Tperm = 1476  kN

#### Check for tension

Tperm  =  1476 kN   >  Tmax  =  1450 -->	o.k.

Sectional area (all cables)  [Table 7 . 3. 2]   	A = 3348  mm2

## Windguy cables

Windguy cable sag at mid-span  =  ℓ8

Take bw  = ℓ9 	bw = 14.7  m

With vertex close to the centre, bw ~fw = 14.7 m and there will not be any problem for the placement of Windguy cable foundation on either bank.

Approximate Windguy cable tension (one cable)

TL ~ TR	=w×ℓ28×bw1+2×fwV

=0⋅5×32.628×14.71+2×14.766.32	T  =  82  kN

#### Number and diameter of Windguy cables

nw =   2     nos

Number of windguy cables (both u/s and d/s)

Diameter of windguy cable

w =  26    mm

Permissible tension for one cable	Tperm =  129  kN

Tperm  =  129 kN  > T  =  82 kN      --> 	ok!

Modulus of elasticity  [table 4 . 2. 1]	E=110 kN/mm2

#### Loadings

- Hoisting load case, gh

Main and handrail cables	= 0.288 kN/m

- Dead load case, gd

hoisting load, gh	= 0.288 kN/m

walkway deck (steel)	= 0.370 kN/m

Walkway support (including hangers)	= 0.220 kN/m

fixation cables	= 0.010 kN/m

wiremesh netting	= 0.060 kN/m

windguy cables	= 0.050 kN/m

Windties (average)	= 0.030 kN/m

Total at dead load case	gd= 1.028 kN/m

- Full load case, gf

dead load, gd	= 1.028 kN/m

live load, P (=3.0 + 50/ℓ)	= 3.377 kN/m

Total at full load case	gf= 4.405 kN/m

Dead load sa, bd   [7. 3. 5]

bd (max) =	=  5.73  m

bd (min) =ℓ23−	=   4.52  m

With bd (max) =5.73 m;  full load sag, bf  1.22bd	=   6.99 m

β1f(max)	= 13.96 deg

Approximate maximum tension under full load [7. 3. 3]

Tmax   ~  gf.l28bf.cos β1f	 =  1427   1476 kN -> ok!

With bd (min) = 4.52 m;              bf = 1.22  4.52	=    5.51  m

β1f=arctan 4×5.51+5132.6	=  11.52 deg.

Tmax =4.405×132.628×5.51×cos11.52o	=  1804 kN    1476 kN  -->  Not ok!

#### Conclusion

- Even with the maximum sag, no next smaller diameter cable can be chosen.

- To obtain minimum sag, the main cable numbers should be increased, which is uneconomical, if the minimum freeboard can be obtained with larger sag.

- Within permissible tension, slight change either in h or bd or both can be made for better stability.

#### Revised h and bd

| Dead load sag | bd = 5.6 m |
| --- | --- |
| Different in elevation | h = 4.5 m |
| Approximate max. tension under full load | Tmax= 1466 kN |
| The left bank cable elevation shall be kept at | H2 = 105.6 m |
| The right bank cable elevation at saddle shall then be, |  |
| H1 = H2 + h = 105.6 + 0.5 | H1 = 110.1 m |

#### Cable inclination at saddle under dead load

at higher foundation,   β1d=arctan 4bd+hℓ

=arctan 4×5.6+4.5132.6	=  11.5o

at lower foundation,   β2d=arctan 4bd−hℓ

=arctan 4×5.6−4.5132.6	=    7.7o

Horizontal distance of lowest point of parabola from higher

Foundation	ed; 	ed = 79.62 m

Maximum sag, fd=bd+h2+h216.bd = 5.6+4.52+4.5216×5.6; 	fd =  8.08 m

#### Check for freeboard

The following lines are drawn in side elevation: –

the freeboard line at elevation= HFL+5.0 = 90.88+5.0

= 95.88 m

- the main cable parabola with its lowest point at horizontal distance of ed (=79.62 m) and vertical distance of fd (=8.08 m) from main cable saddle elevation at higher foundation.

- Approximate windguy cable arrangement in elevation for upstream or downstream, whichever is lower.

##### From the above layout, it is found that the freeboard line within water course clears all the cable alignments.

##### Approximate freeboard:    main cables	=  11.14  m

windguy cables                =   7.60  m

###### Length of dead loaded cables, Ld

Ld=ℓ×1+12hℓ2+83bdℓ2  =132.6 1+124,5132.62+835.6132.62

Ld =  133.31 m

###### Constant factor, C

C=64EA3ℓ3×Ld=64×110×33483×132.63×133.31	c=0.02528 kN/m4

Sag calculation  [7 . 3 . 7]

Hoisting load sag by iteration

1st iteration

[0]	b*=0.93×bd=0.93×5.6	=    5.21 m

[1]	=c×b*×b*2−bd2+b*bd×g

=0.02528×5.21×5.212−5.62+5.215.60×1.028	= 0.401 kN/m

[2]	new b*=bd+b*old−bd×gi−gdg*old−gdgi=gh=0.288kN/m

=5.6+5.21−5.6×0.288−1.0280.401−1.028	=     5.14 m

[3]	Δgi=gi−g*=0.288−0.425	=-0.113 kN/m

[4]	Δgi=0.113>0.01			Not ok !

2nd iteration

[0]	b*=new b*		=     5.14 m

[1]	g*=0.02528×5.14×5.142−5.62+5.145.60×1.028	= 0.302 kN/m

[2]	new b*=5.6+5.14−5.6×0.288−11.0280.302−1.028	=     5.13 m

[3]	Δgi=0.288−0.302		=-0.014 kN/m

[4]	Δgi=0.014>0.01			Not ok !

3rd iteration

[0]	b*=new b*		=      5.13 m

[1]	g*=0.02528×5.13×5.132−5.62+5.135.60×1.028	=  0.288 kN/m

[2]	new b*=5.6+5.13−5.6×0.288−11.0280.288−1.028	=      5.13 m

[3]	Δgi=0.288−0.288		=  0 kN/m

[4]	Δgi=0<0.01			Ok! Stop iteration

Hoisting load sag		bh =   5.13

Full load sag by iteration

1st iteration

[0]	b*=1.22×bd=1.22×5.6	=     6,83 m

[1]	g*=c×b*×b*2−bd2+b*bd×g

=0.02528×6.83×6.832−5.62+6.835.6×1.028	= 3.894 kN/m

[2]	newb*=bd+b*old−bd×gi−gdg*old−gd

=5.6+6.83−5.6⋅4.405−1.0283.894−1.028	=     7.05 m

[3]	Δgi=gi−g*=4.405−3.894	= 0.511 kN/m

[4]	Δgi=0.511>0.01		Not ok!

2nd iteration

[0]	b*=new b*		=    7.05 m

[1]	g*=0.02528×7.05×7.052−5.62+7.055.60×1.028	= 4.563 kN/m

[2]	newb*=5.6+7.05−5.6×4.405−1.0284.563−1.028	=     6.99m

[3]	Δgi=4.405−4.563		= 0.158 kN/m

[4]	Δgi=0.158>0.01			Not ok!

3rd iteration

[0]	b*=new b*		=    6.99 m

[1]	g*=0.02528×6.99×6.992−5.62+6.995.60×1.028	= 4.376 kN/m

[2]	newb*=5.6+6.99−5.6×4.405−1.0284.376−1.028	=     7,0 m

[3]	Δgi=4.405−4.376		= 0.029 kN/m

[4]	Δgi=0.029>0.01			Not ok!

4th iteration

[0]	b*=new b*		=    7,0  m

[1]	g*=0.02528×7.0×7.02−5.62+7.05.6×1.028	= 4,407 kN/m

[2]	newb*=5.6+7.0−5.6×4.405−1.0284.407−1.028	=    7,0  m

[3]	Δgi=4.405−4.407		= 0.002kN/m

[4]	Δgi=0.002<0.01			 ok! Stop iteration.

Full load sag		=  7,0 m

###### Maximum tension at full load

Tmax=gf×ℓ28×b×1+4×bf+hℓ2

=4.405×132.628×7.00×1+4×7.0+4.5132.62	= 1424 kN

###### Safety factor for cable tension

F=TbreakTmax=44281424=3.11>3.0		ok!

Calculation of final data

| Symbol | Formulas |  | Unit | Hoisting load | Dead load | Full load |
| --- | --- | --- | --- | --- | --- | --- |
| ℓ |  |  | m | 132.6 |  |  |
| h |  |  | m | 4.5 |  |  |
| AM | Initial Layout data |  | mm2 | 2764 |  |  |
| AH |  |  | mm2 | 584 |  |  |
| A |  |  | mm2 | 3348 |  |  |
| g |  |  | KN/m | 0.288 | 1.028 | 4.405 |
| b |  |  | m | 5.13 | 5.60 | 7.00 |
| Final results |  |  |  |  |  |  |
| 1 | arctan4×b+hℓ |  | deg | 10.7 | 11.5 | 13.8 |
| 2 | arctan4×b−hℓ |  | deg | 6.9 | 7.7 | 10.0 |
| e | ℓ2×1+h4×b |  | m | 80.8 | 79.6 | 77.0 |
| f | b+h2+h216×b |  | m | 7.63 | 8.08 | 9.43 |
| L | ℓ×1+12hℓ2+83bℓ2 |  | m | 133.21 | 133.31 | 133.66 |
| H | g×ℓ28×b |  | kN | 123.4 | 403.5 | 1383.1 |
| Tmax | Hcosβ1 |  | kN | 126 | 412 | 1424 |
| TM.max | Tmax×AMA |  | kN | 86 | 340 | 1175 |
| TM | Hcosβ2×AMA |  | kN | 103 | 336 | 1159 |
| TH.max | Tmax×AMA |  | kN | 22 | 72 | 248 |
| TH | Hcosβ2×AMA |  | kN | 22 | 71 | 245 |

##### Calculation of data for dead loaded cable parabola and walkway clearance

The equation of main cable parabola with its origin at lowest 
point is-

yi(d/f)=fd/fℓ×i×d2	with fd=8.08 m ;  ed = 79.60 m and

with ff=9.43 m;    ef = 77.00 m

Here, i = integer     = 1,2,3,4- etc.

Taking d=5.0 m, the ordinates (yi) of the parabola from its lowest point are calculated in the following table –

The ordinates for all load case are calculated only for the flat ground surface in front of main foundation where fully loaded cable may not have a minimum of 30 cm clearance above the ground level.

| ##### i | i.d (m) | Yid (m) | Yif (m) | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 5 | 0.03 |  | Origin at -– ed= 79.6 m from higher foundation at dead load case ef= 77.0 m from higher foundation at full load case |
| 2 | 10 | 0.13 |  |  |
| 3 | 15 | 0.29 |  |  |
| 4 | 20 | 0.51 |  |  |
| 5 | 25 | 0.81 |  |  |
| 6 | 30 | 1.15 |  |  |
| 7 | 35 | 1.56 |  |  |
| 8 | 40 | 2.04 |  |  |
| 9 | 45 | 2.58 |  |  |
| 10 | 50 | 3.19 | 3.98 | - cable anchorage at lower foundation at dead load case |
| 11 | 53 | 3.58 | -4.81 |  |
| 12 | 55 | 3.86 | 4.92 | - cable anchorage at lower foundation at full load case |
| 13 | 55.6 | - | 5.73 |  |
| 14 | 60 | 4.59 | 6.72 |  |
| 15 | 65 | 5.39 | 7.79 |  |
| 16 | 70 | 6.25 | 8.95 |  |
| 17 | 75 | 7.13 | 9.43 | - higher anchorage at full load case |
| 18 | 77 | - | - |  |
| 19 | 79.6 | 8.08 |  | - higher anchorage at dead load case |

###### Plot of main cable parabola

- Mark the lowest point of the parabola at [ed, fd] from higher foundation.

- With origin at the lowest point, plot the coordinates [(id), (yid)] from above table on either side of the origin upto the front of main foundation.

- Join then the plotted points to get main cable parabola at dead load case.

For full load case, only the points in front of either foundation are plotted to measure 1.

Compilation of Final Data  [7 . 3 . 8]; Cable Structures

Bridge No. & Name:  Design example	Date:  April 2004 checked by N. L. Joshi

- Initial Data (refer to 7 . 3  . 6 and GA)

| Nominal span, s |  | = | 132.10 | m |
| --- | --- | --- | --- | --- |
| Anchorage type (drum) |  | = | Drum | / |
| Main cable | nM | = | 4 | / |
|  | M | = | 40 | mm |
|  | AM | = | 2764 | mm2 |
| Handrail cable | nH | = | 2 | / |
|  | H | = | 26 | mm |
|  | AH | = | 584 | mm2 |
| Total Metallic Area | = AM + AH | = | 3348 | mm2 |
| Total break | =TM,break + TH,break TM,breakTH,break |  | 4428 | kN |
| Windguy cable | nW | = | 2 | / |
|  | W | = | 26 | mm |
| E-Module |  | = | 110 | kN/mm2 |
| Design span ℓ |  | = | 132.60 | m |
| h |  | = | 4.50 | m |
| bd |  | = | 5.60 | m |
| ed (from higher foundation) |  | = | 79.60 | m |
| fd (from higher foundation) |  | = | 8.08 | m |
| 1,d (at higher foundation) |  | = | 11.50 | deg |
| 2,d (at lower foundation) |  | = | 7.70 | deg |
| Cable anchorage elevations: |  |  |  |  |
| - Left Bank Windguy cable, upstream |  | =(appx) | 97.50 | m |
| Main Cables |  | = | 105.60 | m |
| Windguy cable, downstream |  | =(appx) | 96.40 | m |
| - Right Bank Windguy cable, upstream |  | =(appx) | 102.60 | m |
| Main Cables |  | = | 110.10 | m |
| Windguy cable, downstream |  | =(appx) | 102.30 | m |
| Approximate freeboard Main cables |  | = | 11.14 | m |
| Wind cables |  | =(appx) | 7.60 | m |
| Loads: |  |  |  |  |
| - Walkway steel deck |  | = | 0.37 | kN/m |
| - Live load P |  | = | 3.377 | kN/m |
| - Hoisting load gh |  | = | 0.288 | kN/m |
| - Dead load gd |  | = | 1.028 | kN/m |
| - Full load gf |  | = | 4.405 | kN/m |

- Data from Main Calculation

| Full Load: | bf | = | 7.0 | m |
| --- | --- | --- | --- | --- |
|  | Tmax | = | 14.32 | kN |
|  | Safety factor | = | 3.11 | / |

Comment ...........................................................................................................................

- Data to be Transferred to the General Arrangement

| Load Case | Load g (kN/M) | Tension Tmax (kN) | Sag b (m) | Horiz.Dist. e (m) | Lowest point Vert.Dist.f (m) | Elevation (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Hoisting | 0.288 | 126 | 5.13 | 80.8 | 7.63 | 102.47 |
| Dead Load | 1.028 | 412 | 5.60 | 79.6 | 8.08 | 102.02 |
| Full Load | 4.405 | 1424 | 7.00 | 77.0 | 9.43 | 100.67 |
| Live Load | 3.377 |  |  |  |  |  |

Data of Cable Structure

Also transfer all the remaining data and results displayed by frame

- Data to be Transferred to the General Arrangement

| Parameter | 1) Higher Foundation |  | 2) Lower Foundation |  |
| --- | --- | --- | --- | --- |
| TM,f | 1175 | kN | 1159 | kN |
| TH,f | 248 | kN | 245 | kN |
| f | 13.8 | Deg | 10 | deg |
| 1 | 12 | Deg | 9 | deg |

Cable Tension and Inclination of Full Load Case and Actual Front slope

Related standard drawing  [7 . 3 . 9]

| Drawing number | Drawing title |
| --- | --- |
| 01 | Walkway for 4 main cables |

# MAIN FOUNDATION DESIGN

## Main foundation, right bank, on rock

## Initial layout data [7 . 4 . 5]

## Characteristics of foundation

Type of bridge	:	Inclined Suspended

River bank	:	Right, higher foundation

Cable anchorage type	:	Drum type, without foot

Foundation	:	on rock

## Cable structure characteristics

Number of main cables	nM =	4

Main cable tension	TMf =	1175	kN

Handrail cable tension	THf =	248	kN

Cable inclination	f =	13.8	deg

Front slope of rock or stone pitching	1 =	12	deg

## Rock parameters

Rock at depth	=	0	m

Sliding friction angle	sl =	35	deg

Rock quality coefficient	k =	1.75

Ground bearing pressure	perm =	450	kN/m2

Front slope of rock or stone pitching	1 =	12	deg

Minimum embedded depth	t =	1	m

###### Foundation dimensions

| Foundation dimensions (m) | ##### minimum | maximum |
| --- | --- | --- |
| Back height H1 | 1.50 | 4.00 |
| Front height H2 | 0.80 | 4.00 |
| Width B | 5.00 | 8.50 |
| Length L | 2.90 | 5.00 |
| Back to C.G. distance of anchorage rods, S | 1.00 | - |

Trial values for foundation dimensions [7 . 4 . 6]

| Dimensions (m) | ##### Trial values | Remark |
| --- | --- | --- |
| Back height H1 | 2.50 | The dimensions compiled were found by iteration with regard to economical design |
| Front height H2 | 1.10 |  |
| Width B | 6.80 |  |
| Length L | 3.10 |  |

Active pressure height

As the slope behind foundation is rocky:         	 ha =  0

Unit weight of stone masonry [Table 4. 4. 4]	= 22 	kN/m3

Unit weight of concrete      [Table 4. 4. 4]	= 22 	kN/m3

## Additional load on top of foundation  [6. 2. 4]

(Dimension of retaining wall as given in General Arrangement)

A1=3.10×0.7×1.0×22		= 47.74 kN

A2=2⋅0.5×3.0−0.7×0.7×1.0×22		= 35.42 kN

a1=6.8−0..5×0.7		                            =   6.45 m

a2=6.8−0.7−133.0−0.7	                            =  5.33 m

A = A1+A2  = 47.74+35.42

=  83.16 kN

a=A1×a1+A2×a2A1+A2=47.74×6.45+35.42×5.3347.74+35.42

=  5.97  m

## Calculation of resultant loading forces and static moments

| Initial layout data |  |  | Remarks |
| --- | --- | --- | --- |
| Foundation type | :Drum anchorage on rock |  |  |
| Foundation dimensions | :back height | H1=2.50 m | H2  t (=1m) ok! |
|  | :front height | H2=1.10 m |  |
|  | :breadth | B = 6.80 m |  |
|  | :length | L= 3.10 m |  |
| Check – Bottom inclination | :α=arctanH1−H2B | α=11.6o | < 18o, ok |
| Cable tensions | : main cable | TM=1175kN |  |
|  | : handrail cable | TH= 248 kN |  |
|  | : cable inclination | = 13.8o m |  |
| Active earth pressure coefficient |  | ha=0 | fully rock |

Loading forces and moments

| Loading force |  |  |  | Lever arm |  | Moment Mf (kN-m) |
| --- | --- | --- | --- | --- | --- | --- |
| Formula |  | Vert. Rv (kN) | Horiz. RH (kN) | Formula | Dist. (m) |  |
| Load from foundation W1=0.5×H1+H2×B×L×γc |  | 834.8 | - | B3×2H1+H2H1+H2 | 3,84 | 3206.3 |
| W2=0.95×B×L×γc |  | 440.6 | - | 0.5B | 3.4 | 1498.0 |
| Load from foundation top Total load = A |  | 83.16 | - | a | 5.97 | 496.5 |
| ##### Load from cable tension TMh = Tm cos |  | - | 1141.1 | H2+0.7 | 1.8 | (-)2054.0 |
| THh = TH cos |  | - | 240.8 | H2+1.95 | 3.05 | (-)740.5 |
| TMv = TM cos |  | 280.3 | - | - | 0.25 | 70.5 |
| THv = TH sin |  | 59.2 | - | - | 0.25 | 14.8 |
|  |  | 1698.1 | 1381.9 |  |  | 2497.3 |

## Volume of foundation

Vol=W1+W2γC=834.8+440.622

Vol = 57.97 m3   kN

δR=arctanRHR=arctan1381.91698.1		R = 39.1  deg

B∗2=MFRV×1+tanℓ×tanδR

=2497.31698.1×1+tan11.6o×tan39.1o	B∗2	= 1.26  m

Check for B∗2

B∗2 = 1.26 m    B6 (=1.13)

## Safety factor against toppling   [6 . 4 . 4]

FT    =  1.9     1.5   		ok!

Stress distribution and number of anchor rods  [6 . 2 . 7]

"Uncracked" condition at inclined base

RV'	=RV×cosα+RH×sinα=1698.1×cos11.6o+1381.9×sin11.6o	= 1941.3 kN

B'	=Bcosα=6.8cos11.60	=  6.94  m

B*'2	=B*/2cosα=1.26cos11.60	=  1.29  m

e'	=B'2=B*'2=6.942−1.29	=  2.18  m

σmax.min.=RV'B'×L×1±6×e'B'=1941.36.94×3.1×1±6×2.186⋅94	max.=260.3 kN/m2

min.=(-)79.8 kN/m2

Since B*/2  B/3, anchor rods are necessary.

X'	=B'⋅σminσmin−σmax=6.94⋅(−79.8)(−79.8)−260.3	=   1.63  m

	==B'−B∗2−X'3=6.94−1.29−1.633	=    5.11 m

b	=B−S−B∗2=6.8−1.0−1.26	=    4.54 m

AS	=σmin×x'×L2σσcomb	=1194.4 mm2

N	=k×4×ASπ..d2

N  =     5 nos

Minimum number of anchor rods for 4 main cables	=    4  nos

## Ultimate bearing pressure

B*/2 (=1.26 m)  <  B/3  (=2.27 m)

σmax.ult=4×RV'3B*'×L		= 323.6 kN/m2

max.ult (=323.6kN/m2)  <  perm (= 450 kN/m2)                                            ok!

Safety factor against sliding [6 . 5 . 2 / 6 . 6 . 1]

RH	RH×cosα+RV×sinα=1381.9×cos11.6o−1698.1×sin11.6o	= 1012.2 kN

AS	=N×π4×252=5×π4×252		= 2454.0 mm2

Fsl	=tanφslRV'+ASσσcombRH'   [with anchor rods considered]

=tan35o×1941.3+2454×0,0751012.2	=1.52    1.5  ok!

Fsl	=tanφslRV'RH   [with anchor rods neglected]

=tan35o×1941.31012.2	=1.34    1.3  ok!

Compilation of Final Data  [7 . 4 . 7];  Main Foundation, R/B

Bridge No. & Name:  Design example 1	   Date:  April 2004 checked by N. L. Joshi

- and   B)  Initial Data (refer to 7 . 4 . 5)

C)	Main Calculation

| 1) | Load on top of foundation |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Total load | A = | 83.16 | kN |
|  | - Front to C.G. distance | a = | 5.97 | m |
| 2) | Soil/rock heights |  |  |  |
|  | - Active pressure height | ha = | - | m |
|  | - Rock height at back | hT = | full height | m |
|  | - Embedded depth | t = | 1.2 | m |
|  | - Depth of additional soil | t’ = | 0 | m |
| 3) | Soil parameters |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 = | - | deg |
|  | - Slope of soil baseline | B = | - | deg |
|  | - Length of influence | L*infl = | - | m |
|  | - Back slop of soil |  = | - | deg |
| 4) | Foundation dimensions |  |  |  |
|  | - Back height | H1 = | 2.5 | m |
|  | - Front height | H2 = | 1.1 | m |
|  | - Width | B = | 6.8 | m |
|  | - Length | L = | 3.1 | m |
|  | - Base inclination |  = | 11.6 | deg |
|  | - Distance to resultant force | B* / 2 = | 1.26 | m |
|  |  | L* / 2 = | 1.55 | m |
| 5) | Safety factors |  |  |  |
|  | - Sliding | FSL = | 1.52 | / |
|  | - Bearing capacity | FBC = | - | / |
|  | - Toppling | FT = | 1.90 | / |
|  | - Slope stability | FS = | - | / |
| 6) | Anchorage rods |  |  |  |
|  | Nos. as per calculation or minimum Nos. | N = | 5 | / |

C)	Additional Anchorage Rods

(from geological report)

As the rock is good, no additional anchorage is provided.

## Data to be transferred to the General Arrangement

All initial data and results displayed by the frame

## Related standard Drawings  [7 . 4 . 8]

| Drawing number | Drawing title |
| --- | --- |
| 61 61/2(26) 63(26) | Main cable anchorage for 4 main cables (capacity: 1220 kN) Main foundation for 4 main cables (related drawings: 61, 63(26)) Handrail cable anchorage (capacity: 260 kN) |

## Main foundation on soil, left bank  [7 . 4]

## Initial layout data [7 . 4 . 5]

## Foundation Characteristics

Type of bridge	:	Inclined Suspended

River bank	:	Left, lower foundation

Cable anchorage type	:	Drum anchorage

Foundation	:	on soil

## Cable structure characteristics

Number of main cables	nM =	4	nos

Main cable tension	TMf =	1159	kN

Handrail cable tension	THf =	245	kN

Cable inclination	f =	10	deg

Front slope of rock or stone pitching	1 =	9	deg

## Soil parameters

Sub-soil at depth	=	3.2	m

Friction angle of sub-soil 	1 =	30	deg

Unit weight of sub-soil	1 =	18	kN/m2

Friction angle of backfilling soil	2 =	22	deg

Unit weight of backfilling soil	2 =	17	kN/m2

Ground water at depth	=	           very deep

Ground bearing pressure	perm =	300	kN/m2

Minimum Embedded depth	t =	1.7	m

###### Foundation dimensions

| Foundation dimensions (m) | ##### minimum | maximum |
| --- | --- | --- |
| Back height H1 | 1.5 | 4.0 |
| Front height H2 | 1.2 | 4.0 |
| Width B | 6.2 | 9.5 |
| Length L | 2.9 | 5.0 |

Trial values for foundation dimensions [7 . 4 . 6]

| Dimensions (m) | ##### Trial values | Remark |
| --- | --- | --- |
| Back height H1 | 4.0 | The dimensions compiled were found by iteration with regard to economical design |
| Front height H2 | 2.5 |  |
| Width B | 7.0 |  |
| Length L | 4.7 |  |

Active pressure height	ha =  6.0     m

ha1  = 1.05   m

Embedded depth	t =  2.0     m

Inclination of front slope	 = 19.0    deg

Inclination of back slope	 = 11.0    deg

Unit weight of dry stone masonry	d = 20.0  kN/m3

Unit weight of cement masonry	m = 22.0  kN/m3

Unit weight of concrete	C = 22.0  kN/m3

Additional load on top of foundation  [6 . 2 . 4]

A	=12×0.7×1.05×4.7×22

A =  76.0  kN

a =   6.65  m

a	=7.0−12×0.7

Coefficient of horizontal active earth pressure [6 . 2 . 3]

λah=cos2φ21+sin(φ2+δ)×sin(φ−ψ)cosδ×cosψ2	with δ=23φ2

=cos2φ21+sin(22+23×22)×sin(22−11)cos23×22×cos112

ah = 0.4742

Base inclination

tanα		 = 12.1 deg

Check for 

 = 12.1 deg    15 deg		ok!

Calculation of Loading forces and moments

| Loading force |  |  |  | Lever arm |  | Moment, Mf (kN-m) |
| --- | --- | --- | --- | --- | --- | --- |
| Formula |  | Vert. Rv (kN) | Horiz. RH (kN) | Formula | Dist. (m) |  |
| Load from foundation W1 = 0.5(H1+H2) BLrC |  | 2352.4 | - | B3×2H1+H2H1+H2 | 3.77 | 8868.5 |
| W2=0.95×(B−1.2)×L×γC |  | 569.7 | - | 0.5B+0.6 | 4.1 | 2335.8 |
| Load from foundation top, Total load = A |  | 76.0 | - | a | 6.65 | 505.4 |
| ##### Active earth pressure Eah=λah×ha2−ha122×L×γ2 YEah=ha+2ha13×ha−ha1ha+ha1 |  | - | 661.1 | YEah−H1+H2 | 0.4 | (-)264.4 |
| ##### Eav=Eah×tan23×φ2 |  | 173.0 | - | B | 7.0 | 1249.5 |
| ##### Load from cable tension TMh = Tmcos |  | - | 1141.4 | H2+0.7 | 3.2 | (-)3652.5 |
| THh = THcos |  | - | 241.3 | H2+1.95 | 4.45 | (-)1073.8 |
| TMv = TMcos |  | 201.3 | - | - | 1.45 | 291.9 |
| THv = THsin |  | 42.5 | - | - | 1.45 | 61.6 |
|  |  | 3414.9 | 2043.8 |  |  | 8322.0 |

## Volume of foundation

Vol

Vol   =  132.8  m3   kN

Total vertical force	RV = 3414.9 kN

Total horizontal force	RH = 2043.8 kN

Total moments

Total positive moment	                           M+ = 13312.7  kN-m

Total negative moment	M- =  4990.7  kN-m

Algebraic sum of static moment about F;	MF = 8322.0  kN-m

Inclination of resultant

tanδR	R= 30.9 deg

B∗2=MFRV×(1+tanα×tanδR)=8322.03414.9×(1+tan12.1×tan30.9);B∗2	=  2.16 m

Check for B*/2

B*/2 = 2.16 m    B/4 (=1.75 m)	ok!

Factor of safety against sliding [6 . 4 . 4]

Fsl	Fsl=  1.70

Factor of safety against toppling [6 . 4 . 4]

FT	FT=  2.67

Ultimate bearing capacity and safety factor

against shear failure of the ground [6 . 5 . 4]

L*infl. = 4.3B* = 4.322.16	Linfl.= 18.6 m

From General Arrangement,

At L*infl. = 18.6 m	1 = B= 19.0 deg

q= 9.0 kNm2

##### Surcharge load, (1t + q) = (182 + 9)	= 45 kN/m2

Bearing capacity factors (for 1 = 30o);	Nq =18.4

N=18.1

Correction factor for

shape  B∗L∗ = 0.92  and 1 = 30o

Sq=153

S=0.63

depth  t∗B∗ = 0.46  and 1 = 30o

dq=1.13

d=1.00

inclination of load (R=30.9o and =12.1o)	iq=0.39

i=0.27

inclination of foundation base (=12.1o and 1=30o)	bq=0.78

b=0.72

inclination of base line (B=19.0o)	gq = g = 0.64

## Shear resistance of the ground

P∗B∗L∗(γ1t+q)NqSqdqiqbq'+12×γ1B∗NγSγdγiγbγ'×gq

=2×2.16×4.7×[45.0×18.4×1.53×1.13×0.39×0.78+12×18.0×2×2.16×18.1×0.63×1.0×0.27×0.72]×0.64

=  6778.7 kN

FBC	=  1.99            2.00  ok!

Ground bearing pressure  [6 . 2 . 7]

The case is:  eccentric load with B2

## On inclined base

RV'=RVcosα+RHsinα=3414.9×cos12.1o+2043.8×sin12.1o	=  3767.5 kN

B'	=   7.16 m

B∗2	=   2.21 m

σmax.=RV'B'×L1+3.1−B*'B'

=3767.57.16×4.71+3×1−2×2.167.16	=245.2 kN/m2 < perm (=300 kN/m2) ok!

Reinforcement   [7 . 4 . 6]

AS1=σmax×L0.414×1.2cosα−H2sinα2(H2×cosα+1.2×sinα−0.20)

=245.2×4.70.414×1.2cos12.1o−2.5sin12.1o2(2.5cos12.1o+1.2sin12.1o−0.20)	=  470 mm2

## Minimum reinforcement

AS1(min)=0.02 % of H2L =0.02×2500×4700100	= 2350 mm2

No. of 16 mm   reinforcement bars, N=11.7

Provide 12 nos  16 mm ribbed torsteel bars.

Compilation of Final Data  [7 . 4 . 7];   Main Foundation, L/B

Bridge No. & Name:  Design example 1	   Date:  April 2004 Checked by N. L. Joshi

A)  and  B)  Initial Data (refer to 7 . 4 . 5)

C)	Main Calculation

| 1) | Load on top of foundation |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Total load | A = | 76.0 | kN |
|  | - Front to C.G. distance | a = | 6.65 | m |
| 2) | Soil/rock heights |  |  |  |
|  | - Active pressure height | ha = | 6.0 | m |
|  | - Active pressure height | ha1 = | 1.05 | m |
|  | - Embedded depth | t = | 2.0 | m |
|  | - Depth of additional soil | t' = | 1.0 | m |
| 3) | Soil parameters |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 = | 19.0 | deg |
|  | - Slope of soil baseline | B = | 19.0 | deg |
|  | - Length of influence | L*infl. = | 18.6 | m |
|  | - Back slop of soil |  = | 11.0 | deg |
| 4) | Foundation dimensions |  |  |  |
|  | - Back height | H1 = | 4.0 | m |
|  | - Front height | H2 = | 2.5 | m |
|  | - Width | B = | 7.0 | m |
|  | - Length | L = | 4.7 | m |
|  | - Base inclination |  = | 12.1 | deg |
|  | - Distance to resultant force | B* / 2 = | 2.16 | m |
|  |  | L* / 2 = | 2.35 | m |
| 5) | Safety factors |  |  |  |
|  | - Sliding | FSL = | 1.70 | / |
|  | - Bearing capacity | FBC = | 1.99 | / |
|  | - Toppling | FT = | 2.67 | / |
|  | - Slope stability | FS = | - | / |
| 6) | Anchorage rods |  |  |  |
|  | Nos. as per calculation or minimum Nos. | N = | 12 | / |

C)	Additional Anchorage Rods		__

(from geological report)

## Data to be transferred to the General Arrangement

## All initial data and results displayed by the

frame

## Related standard Drawings  [7 . 4 . 8]

| Drawing number | Drawing title |
| --- | --- |
| 61 61/1(26) 63(26) | Main cable anchorage for 4 main cables (capacity: 1220 kN) Main foundation for 4 main cables {related drawings: 61 & 63(26)} Handrail cable anchorage (capacity: 260 kN) |

# Design of windguy cable structure  [9 . 4]

## General Arrangement

- Windguy arrangement is generally provided for LSTB (span > 120m) bridge.

- Placing of windguy anchorage foundations:

The foundation location should avoid

- dry gully (shown by concave contours) on right bank downstream side.

- steep slope on left bank downstream of the axis in order to get smaller 
angle   for better value of ground bearing capacity.

- the cable alignment should be above the freeboard line at elevation 95.88 m.

Since the right as well as the left bank upstream side has no problem for placement of foundations, the left bank downstream foundation, thus, requires to be fixed first.

From graphical determination,

CL   ~  11.70 m  ;         DL   ~   (-)  15.50  m

The upstream windguy parabola shall be arranged symmetrically to the downstream windguy parabola.

Initial layout data   [9 . 4 . 6]

## From cable structure analysis

ℓ	=	132.60 m

H1 	=	110.10 m

fd 	= 	8.08 m

r 	= 	79.60 m

## For suspended bridge

d	=	6.00 m

k	=	0.66 m

## Determination windguy parabola in plan

(Both upstream and downstream)

## Vertex of the parabola –

- Fix first cross beam at 0.40 m from saddle on right side.

- Fix vertex at 54th cross beam from right side

V = 0.40 + 53 1.20                                  V  =  64.0 m

Windguy cable parabola in plan is  [9 . 4 . 3]

yj'=fWV2×xj2+2.20

For prevailing topographical conditions, first fix the left bank down-stream windguy cable end.

j = 0

y'o= CL		yo= 11.7 m

xo = ℓ - V - DL	= 132.6 - 64.09 - 15.5	xo = 53.10 m

fW=V2Xo2yo'−2.20	= 6425312×(11.7−2.2)	fW= 13.80 m

## Calculation of hW and bW

hW=fW×ℓ−VV2−1=13.80×132.6−64642−1;	hW = 2.06 m

bW=hW2×V216fW×ℓ2−V2=2.062×64216×13.8×132.62−642	bW = 14.88 m

Check if	bW=ℓ8..........ℓ10

ℓbW; 	thus  bW  ~   ℓ9    	ok!

## Windguy cable tension

HW=wℓ28bW=0.5×132.628×14.88		HW= 73.85 kN

TR=HW×1+2×fV2=73.851+2×13.8642;

TR=   80.4 kN

TL=HW×1+2×f⋅ℓ−VV22=73.851+2×13.8×(132.6−646422

TL=  81.4 kN

Tmax=81.4kN

Number and diameter of windguy cable

nW=   1

Number of windguy cable (d/s or u/s)

W=  26 mm

Diameter of windguy cable

Permissible tension	    Tperm=129 kN

## Check for tension

Tperm (=129 kN)  Tmax (~ 81.4  kN)   	ok !

## Location of first windties

BR = V - (diR)= 64 - 68;

BR= 16.0 m

BL= 20.6 m

BL = ℓ - V - (diL) = 132.6 – 64.0 - 68;

## Calculation of R and L

αR=arctan2×fWV2(V−BR)

R=17.92 deg

=arctan2×13.8642(64−16);

αL=arctan2×fWV2(ℓ−V−BL)

L= 17.92 deg

=arctan2×13.8642(132.6−64−20.6);

## Calculation of CRO and CLO

CRO=fWV2×(V−BR)2+tanαR×BR+2,2

=13.8642×(64−16)2+tan17.92o×16.0+2.2;	CRO= 15.14 m

CLO=fWV2×(ℓ−V−BL)2+tanαL×BL+2.2

=13.8642×(132.6−64.0−20.6)2+tan17.92o×20.6+2.2;	CLO= 16.62 m

These data will be the same for both upstream and 
downstream windguy arrangements as they are arranged symmetrically.

## Windguy foundation layout data

The position of windguy cable anchorage foundations are 
determined by –

- locating windguy cable axes with the plot of CRO, CLO and 
 angles in the plan of General Arrangement.

- tracing theoretical windguy cable parabola.

- drawing cross-sections along the axis of four windguy cable ends.

- drawing freeboard line in the cross-section.

- taking into account the Limits and Recommendations given in 
Section 9. 4. 5.

Upstream:

HR= 102.6 m

HL= 97.5 m

DR= -13.0 m

DL= - 15.9 m

downstream:

HR= 102.3 m

HL= 96.4 m

DR= -13.5 m

DL= -15.5 m

## Calculation of CR and CL

##### For downstream windguy cable

CR=fWV2×(V−BR)2+tanαR×(BR+DR)+2.2

=13.8642×(64−16)2+tan17.92o×(16−13.5)+2.2

CR= 10.77 m

CL=fWV2×(ℓ−V−BL)2+tanαL×(BL+DL)+2.2

=13.8642×(132.6−64−20.6)2+tan17.92o×(20.6−15.5)+2.2

CL= 11.61 m

Calculation of windties lengths    [9 . 5]

downstream windguy arrangement

- Determination of parabola 4

y=a4x2+c4;	a4=fWV2=13.80642	a4 =0.003369

c4=2.2−k=2.2−0.66	c4 = 1.54 m

- Calculation ofΔhLP

ΔhL⋅P⋅=yL⋅P⋅×tanγ

yL⋅P⋅=a4(r−V)2+2.2−k

=0.0037×(79.6−64)2+2.2−0.66	yLP=2.37 m

tanγ=(H1−HR−fd)×(ℓ+DR+D)−(DR+r)×(HL−HR)(CR−k)×(ℓ+DR+DL)−(DR+r)×(CR−CL)

(ℓ+DR+DL)=(132.6−13.5−15.5)=103.6

(DR+r)=(−13.5+79.6)=66.1

tanγ	=  0.33

ΔhL⋅P⋅=2.37×0.33		ΔhL⋅P⋅=0.78 m

- Determination of parabola 1, 2 and 3

Parabola 1:	a1	a1=0.001275

c1=ΔhL⋅P⋅	c1 = 0.78 m

Parabola 2:  a2=HR+fd+ΔhL⋅P⋅−H1(r+DR)2

=102.3+8.08+0.78−110.1(79.6−13.5)2             =  0.000242

c2 = 0

Parabola 3:  a3=HL+fd+ΔhL⋅P⋅−H1(ℓ−r+DL)2

=96.4+8.08+078−110.1(132.6−79.6−15.5)2	=  - 0.003444

c3 = 0

## 4)	Calculation of windtie data  [9. 5. 7]

cci=Δhi2+yi2

Dwi=dcosβ×a42(d+2xi)2+1

Wherein:

xi=V−BR−(i−1)×d	=64−16−(i−1)×6=48−(i−1)×6

x'i=xi+r−V	=xi+79.6−64=xi+15.6

imax=(ℓ−B−BL)d+1	=132.6−16−20.66+1=17

yi=a4xi+2.2−0.66	=0.003358xi2+1.54

- for x’i  0

Δhi=aix'i2+ΔhL⋅P⋅−a2x'i

=0.001275x'i+0.78−0.00024x'i

=0.001033x'i+00.78

βi=arctan(2×a2×x'i)	=arctan(0.000484x'i)

- for x’i  0

Δhi=aix'i+ΔhL⋅P⋅−a3x'i

=0.001275x'i+0.78+0,003444x'i	=0.004719x'i2 + 0.777

βi=arctan(2×a3×x'i)	=arctan(0.006888xi')

Starting from right side (i=1) the windtie data are calculated continuously upto the left side (I = imax) as shown in the following tabular calculation.

## Tabular calculation of windtie lengths

| i | xi | X’i | Δhi/tani | c/ci (m) | Dwi (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 48 | 63.6 |  | 10.52 | 6.27 |
| 2 | 42 | 57.6 |  | 8.57 | 6.20 |
| 3 | 36 | 51.6 |  | 6.87 | 6.15 |
| 4 | 30 | 45.6 | Take values for x’i  o | 5.42 | 6.10 |
| 5 | 24 | 39.6 |  | 4.23 | 6.06 |
| 6 | 18 | 33.6 |  | 3.27 | 6.03 |
| 7 | 12 | 27.6 |  | 2.56 | 6.01 |
| 8 | 6 | 21.6 |  | 2.09 | 6.00 |
| 9 | 0 | 15.6 |  | 1.85 | 6.00 |
| 10 | - 6 | 9.6 |  | 1.88 | 6.01 |
| 11 | - 12 | 3.6 |  | 2.17 | 6.03 |
| 12 | - 18 | - 2.4 | Take values for x’i  o | 2.75 | 6.07 |
| 13 | - 24 | - 8.4 |  | 3.64 | 6.13 |
| 14 | - 30 | - 14.4 |  | 4.88 | 6.21 |
| 15 | - 36 | - 20.4 |  | 6.49 | 6.30 |
| 16 | - 42 | - 26.4 |  | 8.49 | 6.42 |
| 17 | - 48 | - 32.4 |  | 10.89 |  |
|  |  |  |  | 86.57 | 97.99 |

5)  Calculation of R and L

βR=arctan2a2×(BR−r)

=arctan2×0.000242(16−79.6)

R = -1.76o

βL=arctan2a3×(r−ℓ+BL)

=arctan2×(−0.003444)×(79.6−132.6+20.6) ;

L = 12.58o

6)	Calculation of ER and EL

ER=BR+DRcosαR×cosβR

=16+(−13.5)cos17,92o×cos(−1.76)

ER = 2.63 m

EL=BL+DLcosαL×cosβL

=20.6+(−15.5)cos17.92o×cos12.58o

EL = 5.49 m

7)	Calculation of total length of cables

lW(tot) = Dwi + ER + EL + overlapping length

= 97.99 + 2.63 + 5.49 +2  1.25	= 108.61 m

LT(tot) =  (c/ci + 0.6)

= 86.57 +17 0.6	=  96.77 m

## Upstream windguy cable structure

Calculation for upstream cable structure is not shown here, but can be made similar to the downstream windguy cable structure.

Compilation of Final Data      [9 . 4 . 8]; W.G.Cable Structures

Bridge No. & Name:  Design example 1	   Date:  April 2004 checked by N. L. Joshi

A)   Initial Data (refer to 9 . 4 . 6 and GA)

| Bridge type (suspended or suspension) ? |  | Suspended | / |
| --- | --- | --- | --- |
| Design span | ℓ = | 132.6 | m |
| Horizontal distance | r = | 79.6 | m |
| Windguy cable | nW = | 1 | / |
|  | W = | 26 | mm |
|  | AW = | 292 | mm2 |
|  | TW,break = | 386 | kN |
| E-Module | = | 110 | kN/mm2 |
| Cable anchorage elevation: |  |  |  |
| - Left bank Windguy cable, upstream | = | 97.5 | m |
| Windguy cable, downstream | = | 96.4 | m |
| - Right bank Windguy cable, upstream | = | 102.6 | m |
| H1 | = | 110.1 | m |
| Windguy cable, downstream | = | 102.3 | m |
| Freeboard Windguy cables | = | 7.5 | m |
| Loads: - Wind load | w = | 0.5 | kN/m |

B)   Data from Main Calculations

|  | Upstream | Downstream |  |
| --- | --- | --- | --- |
| Theoretical hW | = | 2.06 | m |
| bW | = | 14.88 | m |
| fW | = | 13.80 | m |
| TR | = | 80.40 | kN |
| TL | = | 81.40 | kN |
| Safety factor | = | 4.74 | / |
| Layout L | = | 17.92 | deg |
| R | = | 17.92 | deg |
| CLo | = | 16.62 | m |
| CRo | = | 15.14 | m |
| CL | = | 11.61 | m |
| CR | = | 10.77 | m |
| DL | = | -15.50 | m |
| DR | = | -13.50 | m |
| HL | = | 96.40 | m |
| HR | = | 102.30 | m |

## Data to be transferred to the General Arrangement

## Into plan

All results displayed by the frame

## Into longitudinal section

## Separate longitudinal section of all four cable ends

## Freeboard

## Windguy cable elevation (H). inclination () and tension (T).

|  | H (m) |  (deg) | T (kN) |
| --- | --- | --- | --- |
| Right bank: upstream downstream Left bank: upstream downstream | 102.6 102.3 97.5 96.4 | ..... -1.76 .... 12.58 | 80 80 81 81 |

## Into cable list

## Windguy cable:

## Upstream: 	 number	=	 1

diameter	=	26   	mm

Total cutting length, LW (tot)      = 108.61  m

## downstream: 	number	=	 1

diameter	=	26   mm

Total cutting length, LW (tot)      = 108.18  m  m

## Windties:	 diameter                            =   13  mm

Total length, LT (tot) [D/S]     =   97  m

Related standard drawings  [9 . 5 . 9]

| Drawing number | Drawing title |
| --- | --- |
| 11 | Windtie for windguy cable 13 mm  |

# Windguy cable foundation design  [9 . 6]

## Initial layout data [9 . 6 . 5]

| #### Foundation Characteristics River bank Side Foundation on | Right downstream Rock | Left downstream soil |
| --- | --- | --- |
| #### W.G. cable structure characteristics |  |  |
| Number of windguy cable nW = | 1 | 1 |
| Windguy cable tension TW = | 80.4 kN | 81.4 kN |
| Cable inclination  = | -1.76o | 12. 58o |
| Front slope 1 = | 0 | 14. 2o |
| #### Soil parameters |  |  |
| Sub-soil at depth = | - | 2. 5 m |
| Friction angle of sub-soil 1 = | - | 30o |
| Unit weight of sub-soil 1 = | - | 17 kN/m2 |
| Friction angle of backfilling soil; 2 = | - | 30o |
| Unit weight of backfilling soil; 2 = | - | 17 kN/m2 |
| Ground water at depth = | - | - |
| Ground bearing pressure perm = | - | 200 kN/m2 |
| #### Rock parameters |  |  |
| Rock at depth = | 0. 5 m | - |
| Sliding friction angle sl = | 35o | - |
| Rock quality coefficient k = | 1.75 | - |
| Ground bearing pressure perm = | 400 kN/m2 | - |
| Minimum embedded depth tmin = | 0. 5 m | 1. 0 m |

## Design parameters for 1 no  26 mm cable

| Foundation on | Rock |  |  | Soil |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Foundation dimensions (m) | ##### maximum | minimum |  | maximum | minimum |  |
| Width B= | 2.40 | 1.70 |  | 2.40 | 2.00 |  |
| Length L= | 3 | 1.20 |  | 3.50 | 1.50 |  |
| For = | - | 45o | 45o | - | 45o | 45o |
| Front height hT= | 2 | 0.4 | 0.6 | 2 | 0.4 | 0.6 |
| Front height H2= | - | HT + 0.4 |  | - | HT + 0.4 |  |
| C.G. distance S= | 0.75 |  |  | - |  |  |

Manual Calculation  [9 . 6 . 6]

The manual calculation are made for right and left bank downstream foundations only.

Pre-calculation data

| Foundation location | Right bank downstream on rock | Left bank downstream on soil |
| --- | --- | --- |
| Foundation dimensions (m) B L hT H1 H2 S | 2.0 1.5 0.5 1.5 1.1 0.75 | 2.5 1.5 1.2 2.0 1.6 - |
| Topographical conditions ha ha1 hp (t) hw t  1 |  | 3.7 1.7 0.8 0 0.7 20o 20o |
| Active earth pressure coefficient λah=cos2φ21+sin(φ2+δ)×sin(φ2−ψ)cosδ×cosψ2 with δ=23×φ2 |  | 0.29 |

Additional load on top of foundation

Left bank downstream

Soil	( = 17  kN/m3)

A1	=	0.5  (1.7+0.5) 1  1.5  17	=   28.05  kN

a1	=	13×2×1.7+0.51.7+0.5+1.5	=    2.1  m

Masonry retaining wall ( = 22  kN/m3)

A2	=	0.5 0.4 1.5 22	=   6.6 kN

a2	=	1.1+0.5  0.4	=    1.3 m

A	=	A1 + A2 = 28.05 + 6.6

A = 34.65 kN

a	=	28.05×2.1+6.6×1.328.05+6.6

a = 1.95 m

## Calculation of loading forces and static moments @ F

| Formulas |  | Right bank downs downstream |  |  |  | Left bank downstream |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Loads | Lever arm | RV (kN) | Rh (kN) | arm (m) | MF (kN-m) | RV (kN) | Rh (kN) | arm (m) | MF (kN-m) |
| Dead wt. of foundation W=H1+H22B × L ×γC | B3×2H1+H2H1+H2 | 85.80 | - | 1.05 | 90.20 | 148.5 | - | 1.3 | 192.50 |
| Load on top A | a | - | - | - | - | 34.65 | - | 1.95 | 67.57 |
| Earth pressure Eah=λah⋅ha2−ha122×γ2L YEah=ha+2ha13×ha−ha1ha+ha1 Eav=Eahtan | YEah-H1-0.1-H2 B | - - | - - | - - | - - | - 14.53 | 39.93 - | 0.38 2.50 | 15.17 36.33 |
| Cable tension load Th = Tcos Tv = Tsin | ht 0 | - 2.47 | 80.36 - | 0.5 0 | -40.181 0 | - -17.67 | 79.46 - | 1.20 0 | -95.35 0 |
|  |  | 88.27 | 80.36 | - | 50.02 | 195.18 | 119.39 | - | 216.22 |
| Volume |  |  |  |  |  |  |  |  |  |
| Vol = Wγc |  | 3.90 m3 |  |  |  | 6.75 m3 |  |  |  |
| Bottom inclination α=arctanH1+0.1−H2B |  | 14.04 deg |  |  |  | 11.31deg |  |  |  |
| Inclination of resultant δR=arctanRhRV |  | 42.32 deg |  |  |  | 31.45 deg |  |  |  |
| Location of resultant B∗2=MFRV×(1+tanα×tanδR) |  | 0.46 |  |  |  | 0.99 m |  |  |  |

## Right bank, foundation on rock

Safety factor against toppling

FT	= M+M−	ok!

Stress distribution and number of anchor rods [6 . 2 . 7]

“uncracked” condition at the inclined base

RV'	=RVcosα+Rhsinα

88.27cos14.04o+80.36sin14.04o	RV' =  105.12 kN

B’	=	B =    2.06 m

B'2	=	B'2=    0.48 m

e	=	e =    0.55m

σalignlmaxmin	=RV'B'L×1±6×e'B'

=105.122.06×1.5×1±6×0.552.06	Tmax  =   88.90 kN/m2

Tmin = - 20.91 kN/m2

Since B*/2    B/3  anchor rods are necessary

x' 	B'×σσmin−σmax

=2.06×(−20.91)−20.91−88.90	x =  0.39m

a	=B'−B*'2−x'3

=2.06−0.48−0.393	a =  1.45 m

b	=B−S−B∗2

=2.0−0.75−0.46	b =  0.79 m

AS	=σmin×x'×L2σt.perm⋅ab

=−20.91×0.39×1.52.23×1.450.79	AS= 0.49 cm2

N 	=	=  0.18

Minimum number of anchor rods (25 mm = 2)

Provide 2 nos 25 mm  vertical anchor rods.

Ultimate bearing pressure

σmax.ult.=4×RV'3×B*'×L=4×105.123×2×0.48×1.5	= 97.33  kN/m2

Check for max.ult.

σmax.ult.	97.33 kN/m2   <perm = 400 kN/m2	ok!

Safety factor against sliding  [6 . 5 . 2]

Rh'	=Rh×cosα−R×sinα

=80.36×cos14.04o−88.27×sin14.04o	= 56.55  kN

AS	=		= 981.7 mm2

Fsl	=tanφsl×RV'+AS×τtcombRh'   [with anchor rods considered]

=tan35o×105.12+981.7×0.07556.55	 = 2.60

Fsl	=tanφsl×RV'Rh'   [with anchor rods neglected]

=tan35o×105.1256.55	= 1.30

Check for Fsl

Fsl	=  2.60    1.50 [with anchor rods considered]	ok!

Fsl	=  1.30  =  1.30  [with anchor rods neglected]	ok!

## Left bank, foundation on soil

Safety factor against toppling  [6 . 4  . 4]

FT	= 	FT = 3.27

Safety factor against sliding

Fsl	= 	Fsl = 1.57

Ultimate bearing capacity and safety factor against shear
failure of ground [6 . 5 . 4]

L∗infl.	=B∗tan(45o+12φ1)×e0.5πtanφ1

=2×0.99×tan(45o+12×φ1)×e0.5πtan30o	=   8.49 m

From cross section in General Arrangement

AtL∗infl.	= 8.49 m,         t = 0.7 m	B=   20.0 deg

q	=12  t’  1 =12×0.7×17.0	q =  6.0 kN/m2

1 t	= 17.0 0.8	1 t= 13.6 kN/m2

Bearing capacity factors (for 1 = 30o)	Nq = 18.4

N = 18.1

Correction factors for

Shape		Sq = 1.58

(Note: if B*/L* > 1.0 – 1.5 take value for B*/L* =1.0)	S = 0.60

Depth		dq = 1.11

d = 1.0

Inclination of load (R = 31.45o  and  = 11.31o)	iq = 0.36

i = 0.24

Inclination of foundation base ( = 11.31o  and  1 = 30 o)	bq = 0.80

b = 0.74

Inclination of base line (B = 20 o)	gq  =  g = 0.62

Shear resistance of the ground

P=B∗L∗[(γ1t+q)×Nq×Sq×dq×iq×bq'×gq+12×γ1×B∗Nγ×Sγ×dγ×iγ×bγ'×gγ]

= 1.98 1.5[(13.6+6.0)18.41.58 1.110.360.800.62

+ 0.5171.9818.10.601.00.240.740.62]

P* = 395.20 kN

Safety factor against bearing

FBC	=	FBC =   2.02

Ground bearing pressure  [6 . 2 . 7]

RV'	RV×cosα+Rh×sinα

=195.18×cos11.31o+119.39×sin11.31o	RV'= 214.80 kN

B	=Bcosα=2.5cos11.31o	B =  2.55 m

B*'2	=  	B*'2= 1.00  m

e'	=	e =  0.28 m

Since	=

σalignlmaxmin	=RV'B'L×

=214.802.55×1.5×	σ=93.15 kN/m2

σ= 19.16 kN/m2

Upstream windguy cable anchorage foundations

Calculation for upstream windguy cable foundation is not shown here.  Calculation procedure given here for d/s windguy cable foundations can be followed for u/s windguy cable foundations too.

Compilation of Final Data  [9 . 6 . 7];    D/S W.G. Cable Foundations

Bridge No. & Name:  Design example 1	   Date:  April 2004 checked by N. L  Joshi

A)  and B)  Initial Data (refer to 9 . 6 . 5)

| C) | Main Calculation |  | Right bank | Left bank |  |
| --- | --- | --- | --- | --- | --- |
| 1) | Load on top of foundation |  |  |  |  |
|  | - Total load | A = | - | 34.65 | kN |
|  | - Front to C.G. distance | a = | - | 1.95 | m |
| 2) | Soil/rock heights |  |  |  |  |
|  | - Active pressure height | ha = | - | 3.7 | m |
|  | - Active pressure height | ha1 = | - | 1.7 |  |
|  | - Rock height at back | hrt = | - | 0 | m |
|  | - Embedded depth | t = | 0.3 | 0.8 | m |
|  | - Depth of additional soil | t' = | - | 0.7 | m |
| 3) | Soil parameters |  |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 = | - | 0 | deg |
|  | - Slope of soil baseline | B = | - | 20.0 | deg |
|  | - Length of influence | L*infl. = | - | 8.49 | m |
|  | - Back slop of soil |  = | - | 20 | deg |
| 4) | Foundation dimensions |  |  |  |  |
|  | - Height of cable anchorage | HT = | 0.5 | 1.2 |  |
|  | - Back height | H1 = | 1.5 | 2.0 | m |
|  | - Front height | H2 = | 1.1 | 1.6 | m |
|  | - Width | B = | 2.0 | 2.5 | m |
|  | - Length | L = | 1.5 | 1.5 | m |
|  | - Base inclination |  = | 14.04 | 11.31 | deg |
|  | - Distance to resultant force | B*/2 = | 0.46 | 0.99 | m |
|  | - Distance to resultant force | L*/2 = | 0.75 | 0.75 | m |
| 5) | Safety factors |  |  |  |  |
|  | - Sliding | FSL = | 2.60/ 1.30 | 1.57 | / |
|  | - Bearing capacity | FBC = | - | 2.02 | / |
|  | - Toppling | FT = | 2.24 | 3.27 | / |
|  | - Slope stability | FS = | - | - | / |
| 6) | Anchorage rods |  |  |  |  |
|  | Nos. as per calculation or minimum Nos. | N = | 2 | - | / |

C)	Additional Anchorage Rods

(from geological report)		As the rock is sound, no additional

anchorage rod is provided.

Check of results  [6 . 5 . 1 / 6 . 6 . 1]

| Foundation location | Right bank downstream on rock | Left bank downstream on soil |
| --- | --- | --- |
| -  18o on rock  15o on soil | 14.04o < 18o | 11.31o < 15o |
| B∗2  B6 on rock  B4 on soil | 0.46 m  0.33 m | 0.99 m  0.63 m |
| FT  1.5 | 2.24  1.5 | 3.27  1.5 |
| FBC  2 |  | 2.02 > 2.0 |
| max.ult.  perm | 98.20 kN/m2  400 kN/m2 | 93.15 kN/m2  200 kN/m2 |
| N  2 nos. | 2 nos 0k! |  |
| L−0.7N−1  0.75 m | 1.25  0.75m |  |
| Fsl  1.5 with anchor rods  1.2 without anchor rods | 2.60  1.5 1.30 = 1.3 | 1.57 0k! |

## Data to be transferred to the General Arrangement

All the initial layout data, remarks and results displayed

by the frame

Related Standard Drawings (for downstream foundations) [9. 6. 8]

| Drawing number | Number of drawings | Drawing title |
| --- | --- | --- |
| 41 | 2 | Windguy cable anchorage for 1 cable26 (capacity: 130 kN) |
| 41/1 | 1 | Windguy cable foundation for 1 cable (on soil) |
| 41/2 | 1 | Windguy cable foundation for 1 cable (on rock) |

# 12.4.2	suspension bridge

DESIGN OF MAIN CABLE STRUCTURES  [8 .  3]

Initial layout data  [8 . 3 . 6]

1)	Span, ℓ

approximate span,	ℓ = 142.0 m

integer	i	=     58

correct span	ℓ=2.4×i+2.2=2.4×58+2.2	=141.4 m

nS=ℓ−3.41.2=141.4−3.41.2	= 115 m

nc=ns+2=115+2	=   117

nd=nc+1=117+1	=   118

2)	Tower height, ht

ht (max)= 0.165  ℓ + 1.05 = 0.165  141.4  + 1.05	=24.38 m

ht (min)	= 0.110  ℓ + 1.05 = 0.110  141.4 + 1.05	=16.60 m

ht (rec)	= 0.145  ℓ + 1.05 = 0.145  141.4 + 1.05	=21.55 m

Available tower height nearest to ht (rec):   higher    	    lower

ht = 		22.73 m	20. 24 m

3)	Dead load sag, fd

fd (max)	= (ht  - 1.05) - 0.02  ℓ =	 18.85 m	16.36 m

fd (min)	= (ht  - 1.05) - 0.03  ℓ = 	17.44 m	14.95 m

fd (rec)	= (ht - 1.05) - 0.025 ℓ = 	18.15 m	15.65 m

4)	Calculation of live load, P

P                       =	3.6+60ℓ=3.6+60141.4	=4.024 kN/M

5)	Pre-calculation of approximate tensions

a)	Main cables

-	approximate full load,

-	gf(approx)=1.30+ℓ500+P

=1.30+141.4500+4.024	=5.607 kN/m

-	With ht = 22.73 m  and fd (rec) = 18.15 m

Tmax(approx.)=gf×ℓ28.4×fd×1+17.64⋅(fdℓ)2

=5.607×141.428.4×15.65×1+17.64×(15.65141.4)2

Tmax =  835 kN

-	 With ht = 20.24 m  and fd (rec) = 15.65 m

Tmax(approx.) =5.607×141.428.4×15.65×1+17.64×(15.65141.4)2

Tmax =  940 kN

Number and diameter of main cables

With nM = 4 and M = 36 mm

Permissible tension, Tperm = 987 kN   Tmax  =  940  kN

Therefore, adopt lower tower height.

By trial,  with next lower tower and maximum dead load sag,  increase in main cable section is required.

Safety factor for cable tension

Breaking load, 		Tbreak = 2960 kN

Safety factor against breaking  TbreakTmax=2960940=3.15

Since safety factor (=3.15)    3.0    slightly lower sag is possible.

Dead load camber, cd

cd	= ht  - 1.05 - fd  =  20. 24  - 1.05 - 15.65; 	cd =  3.54 m

Main cable inclination, f

f	=arctan4.2×fdℓ=arctan4.2×15.65141.4	= 24. 93 deg

b)	Windguy cables

-	With vertex at V = ℓ/2 = 141.4/2 = 70.7 m,	hW = 0

bW=fW=ℓ8  to  ℓ10

-	Take  bW = fW  ~  ℓ9  	= 15.70 m

Approximate windguy cable tension

TR  =  TL	~	w×ℓ28×bW×1+2×fWV2

i.e. Tmax. 	=	0.5×141.428×15.7×1+2×15.770.72	Tmax =  87.1 kN

With nW	=	1 (for upstream or downstream) & W	  =   26 mm

Tperm	=	129 kN      87.1  kN	ok!

6)	Final layout data

| Span | ℓ= 141.40 m |
| --- | --- |
| Tower height | ht = 20.24 m |
| Dead load sag | fd = 15.65 m |
| Dead load camber | Cd = 3.54 m |
| Camber span ratio, cd /ℓ | = 0.025 |
| Number of main cables | nM = 4 nos |
| Diameter of main cable | M = 36 mm |
| Sectional area of main cables (all cables) | AM = 2240 mm2 |
| Breaking load | Tbrea k= 2960 kN |
| Factor of safety | = 3.15 |
| Diameter of spanning cable | S = 32 mm |
| Number of windguy cable (u/s or d/s) | nW = 1 nos |
| Diameter of windguy cable | W = 26 m m |

7)	Young’s Modulus (for all cables) [Table 4 . 2 . 1] 	E=110 kN/mm2

8)	Backstay distances and cable anchorage elevations

according to General Arrangement with due consideration of topographical and geological conditions.

| Walkway elevation | HW = 84.0 m |
| --- | --- |
| Tower height | ht =20.24 m |
| Saddle elevation | (HW+ht)=104.24 m |

Backstay angle	f =24. 93 deg

Graphically -

-	left bank:	main cable elevation	= 93.53 m

| Saddle elevation | DL =18.75 m |
| --- | --- |

backstay distance

windguy cable, upstream	=  83.20 m

spanning cable	 = 83.75 m

windguy cable, downstream	=  83.30 m

-	right bank:	main cable elevation	=  97.40 m

| backstay distance | DR = 15.50 m |
| --- | --- |

windguy cable, upstream	=  83.60 m

spanning cable	=  83.75 m

windguy cable downstream	=  83.80 m

9)	Check for freeboard

- plot all the initial layout data in the cross-section of the General Arrangement.

- Draw all the cable alignments with respect to cable anchorage elevations.

Draw freeboard line at elevation H.F.L + 5. 0 = 79 + 5

= 84 m

within the river channel.

From the plot, it is found that all the cable alignments clear off the      freeboard line.

Approximate freeboard:	spanning cable 	=  5.3  m

Lowest windguy cable	= 5.0  m

- Loadings

Hoisting load case, gh

-	main cables		= 0.192 kN/m

Total at hoisting load case 	  gh  = 0.192 kN/m

Dead load case, gh

- Dead weights, gdd

- Hoisting load, gh		= 0.192 kN/m

- Walkway deck (steel) 	= 0.410 kN/m

- Walkway support		=  0.270 kN/m

- Handrail and fixation cables	=  0.030 kN/m

- Wiremesh netting		=  0.060 kN/m

- Suspenders (average)	=  0.170 kN/m

- Spanning cables		= 0.076 kN/m

- Windguy cables		= 0.050 kN/m

- Windties (average)	=  0.040 kN/m

Subtotal dead weights, gdd	= 1.128 kN/m

b)	Pretension in spanning cable

- Assumed approximate pre-tension at dead load case is 10 percent of dead weights.

- Gpd = 0.10  gdd = 0.10  1.128	= 0.113 kN/m

Total at dead load case, (gdd+gpd);	gd = 1.241 kN/m

Full load case, gf

- Dead weights	, gdd	= 1.128 kN/m

- Pretension	,	gpd	= 0

- Live load	,	P	= 4.024 kN/m

Total at full load case	gf= 5.152 kN/m

Sag calculation at hoisting and full load case by iteration [8 . 3 .7]

### Length of dead loaded main cables

Ld =ℓ×1+83×fdℓ2=141.4×1+83×15.65141.42	              Ld=146.02 m

Main cable tension at dead load case

Hd	=gd×l28×fd=1.241×141.428×15.65	       Hd = 198.18 kN

Td	Hd×1+16fdℓ2=198.18×1+1615.65141.42    	Td = 216.73kN

Calculation of a, b, and fo

a	=16×fdℓ×5−24×fdℓ2

=16×15.65141.4×5−24×15.65141.42    	   a = 8.334

b	=15−8×fdℓ2×5−36×fdℓ2

=15−8×15.65141.42×5−36×15.65141.42	b = 14.553

fo	~ arctan4.2×fdℓ

=arctan4.2×15.65141.4	              fo = 24.93 deg

## Tabular Calculation

| Step | Formula | Hoisting load g1=gh= 0.192kN/m | Full load g1 = gf= 5.152kN/m |
| --- | --- | --- | --- |
| 1 | H1=g×ℓ28×f1 T1=H1×1+16f1ℓ2 | 479.85fh Hh×1+fh35.352 | 12876.11ff Hf×1+ff35.352 |
| 2 | 15×ΔLa=15a×(2H1+T1)×Ld3⋅E×AM×g1−gdg1 | - 0.00389Hh - 0.00194Th | 0.00054Hf + 0.0002Tf |
| 3 | ba×ΔD1=ba×T1(DR+DL)E×AM×g1−gdg1 | - 0.00133Th + 0.000777 - 30.51/Hh2 | - 0.00184Tf |
| 4 | Δf=15⋅ΔL1a+ba×ΔD1 | - 0.00389Hh - 0.00327Th + 0.00777 - 30.51/Hh2 | - 0.00054Hf + 0.00454Tf |

Iteration

(a)	Hoisting load sag

- Primary  f1 = fh = 0.98  fd = 0.98  15.65 = 15.34 m

| Step | Iteration | 1st | 2nd |
| --- | --- | --- | --- |
| 1 | Hh=479.85fh Th=Hh×1+fh35.352 | 31.3 34.1 | 31.19 = Hh 34.01 = Th |
| 2 | -0.00389 Hh -0.00327 Th +0.000777 -30.51/Hh2 | - 0.1218 - 0.01115 + 0.000777 - 0.0311 | - 0.1213 - 0.1112 + 0.000777 - 0.316 |
| 3 | Δfh | - 0.2636 | - 0.2633 |
| 4 | New f=fd+Δfh | 15.386 | 15.387= fh |
| 5 | newfh−oldfh | 0.046 | 0  0.005 Stop iteration! |

For hoisting load case –

(b)	Full load sag

- Primary ff=1.05  fd=1.05  15.65=16.43 m

| Step | Iteration | 1st | 2nd |  |
| --- | --- | --- | --- | --- |
| 1 | Hf=12876.11ff Tf=Hf×1+ff35.352 | 783.70 864.21 | 781.98 862.65 | =Hf =Tf |
| 2 | - 0.00054Hf + 0.000454Tf | - 0.4232 + 0.3924 | - 0.4223 + 0.3916 |  |
| 3 | Δff | 0.8156 | 0.8139 |  |
| 4 | Newf=fd+Δff | 16.466 | 16.464 | =ff |
| 5 | newff−oldff | 0.036 | 0.002  0.005 Stop iteration! |  |

For full load case

Calculation of safety factor

Tmax = Tf		= 862.65 kN

Safety factor, F=	ok!

## Calculation of final data

(a)	Main Cables

Cable inclination at saddles

βf=arctan4×ffℓ=arctan4×16.46141.4

f=24.97 deg

Total length of main cable (at dead load) between anchorage

(including overlapping length for fixation)

LM

LM=  184  m

Anchorage location

hL=4×ffℓ×DL=4×16.46141.4×18.75	hL = 8.73 m

hR=4×ffℓ×DR=4×16.46141.4×15.5	hR = 7.22 m

Displacement of saddles for cable hoisting

ΔDL,R=Th×DL,RE×AM×gh−gdgh+gh2×DL,R324cosβfo×1Hd2−1Hh2

|  | Hoisting load case gh=0.192 kN/m Hh=31.19 kN/m Th=34.01 kN/m |
| --- | --- |
| Left bank: DL=18.75 m Right bank: DR=15.50 m | ΔDL = - 0.025 m ΔDR = - 0.018 m |

(b)	Spanning cables: dead load case

βcd=arctan4×cdℓ=arctan4×3.54141.4	cd=5.72 deg

Tsd	Tsd = 79.78 kN

hoisting load case

βch=arctan4×Cdℓ=arctan4×3.54141.4	ch = - 5.72 deg

Tsh=gSh×ℓ28×Ch=0.076×141.428×3.54	Tsh = 53.66 kN

Lsd=ℓ×1+83×Cdℓ2=141.4×1+83×3.54141.42	Tsd = 141.64 m

Compilation of Final Data  [8 . 3 . 8]; Main Cable Structures

Bridge No. & Name:  Design example 2  Date: April 2004 checked by N. L. Joshi

a)	Initial Data (refer to 7 . 3 . 6 and GA)

| Design span | ℓ | = | 141.4 | m |
| --- | --- | --- | --- | --- |
| Tower height | ht | = | 20.24 | m |
| Main cable | nM | = | 4 | / |
|  | M | = | 36 | mm |
|  | AM | = | 2240 | mm2 |
|  | TM,break | = | 2960 | kN |
| Spanning cable | n | = | 2 | / |
|  | S | = | 32 | mm |
|  | AS | = | 884 | mm2 |
|  | TS,break | = | 1170 | kN |
| Windguy cable | nW | = | 2 | / |
|  | W | = | 26 | mm |
| E-Module |  | = | 110 | KN/mm2 |
| Cable anchorage elevations: |  |  |  |  |
| - Left Bank Windguy cable, upstream |  | =(appx) | 83.20 | m |
| Spanning Cables |  | = | 83.75 | m |
| Windguy cable, downstream |  | =(appx) | 83.30 | m |
| Main cables |  | =(appx) | 93.53 | m |
| - Right Bank Windguy cable, upstream |  | =(appx) | 83.60 | m |
| Spanning cables |  | = | 83.75 | m |
| Windguy cable, downstream |  | =(appx) | 83.80 | m |
| Main Cables |  | =(appx) | 97.40 | m |
| Approximate freeboard Spanning cables |  | = | 5.30 | m |
| Windguy cables |  | = (appx) | 5.00 | m |
| Loads: |  |  |  |  |
| - Walkway steel deck | gpd | = | 0.41 (steel) | KN/m |
| - Pretension | P | = | 0.113 | KN/m |
| - live load |  | = | 4.024 | KN/m |
| - Hoisting load | gh | = | 0.192 | KN/m |
| - Dead load (including gpd) | gd | = | 1.241 | KN/m |
| - Full load | gt | = | 5.152 | KN/m |

b)	Data from Main Calculation

| Full loads: | ff | = | 16.46 | m |
| --- | --- | --- | --- | --- |
|  | f | = | 24.97 | deg |
|  | Tmax | = | 862.65 | KN |
|  | Safety factor | = | 3.43 | / |

Comment .....................................................................................................

c)	Data to be transferred to the General Arrangement

| Cables | Load case | Load, g (kN/m) | Tension, T (m) | Sag, f / Camber, C (m) | Elevation of Vertex (m) | Displacement of Saddles |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL(m) | DR(m) |
| Main | Hoisting | 0.192 | 34 | 15.39 | 88.85 | - 0.03 | - 0.02 |
|  | Dead Load | 1.241 | 217 | 15.65 | 88.59 | 0.00 | 0.00 |
|  | Full Load | 5.152 | 863 | 16.46 | 87.78 | / | / |
| Spanning | Hoisting | 0.076 | 54 | - 3.54 | 80.21 |  |  |
|  | Dead Load | 0.113 | 97 | 3.54 | 87.29 |  |  |
|  | Full Load | 0 | 0 | 2.73 | 86.48 |  |  |
|  | Live Load | 4.024 |  |  |  |  |  |

Data of Cable Structure

Also transfer all the remaining data and results displayed by the frame

Related Standard Drawings [8 . 3 . 9]

| Drawing Number | Drawing Title |
| --- | --- |
| 07 | Walkway |
| 08 | Steel Walkway Deck |

Standard Design Drawings: Walkway

# Load combinations with wind load [8 . 4]

(A)	Initial layout data  [8 . 4 . 8]

(1)	From cable structure analysis

Span	ℓ	=	141.4	m

Tower height	ht	=	20.24	m

Centre distance of tower leg	c/c1	=	4.0	m

Backstay cable inclination	f	=	24.97	deg

Main cables:	nM	=	4

M	=	36	mm

AM	=	2240	mm2

Spanning cables:	S	=	32		mm

AS	=	884		mm2

Modulus of elasticity	E	=	110		kN/mm2

Sag, fo:	-for loading case [A]	fd	=	15.65		m

-for loading case [B]	ff	=	16.46		m

Load, PMO:	-for loading case [A]	gd	=	1.241		kN/m

-for loading case [B]	gf	=	5.152		kN/m

Pre-tension in spanning cables:

-for loading case [A]	Pso	=	0.113		kN/m

-for loading case [B]	Pso	=	o

(2)	Wind load:	-for loading case [A]	w	=	0.5		kN/m

-for loading case [B]	w	=	0.167		kN/m

(3)	Breaking tension of spanning cables; 	TS,break	=	1170		kN

(4)	Calculation of initial cable lengths

| Loading case | A | B |
| --- | --- | --- |
| fo go PMO PSO Co =ht - 1.05 - fo LMO =l [1 + 8/3 (fo / l)2] LS0 =l [1+ 8/3 (Co / l)2] | 15.65 m 1.128 kN/m 1.241 kN/m 0.113 kN/m 3.54 m 146.02 m 141.64 m | 16.46 m 5.152 kN/m 5.152 kN/m 0 2.73 m 146.51 m 141.54 m |

Calculation of Wb

for loading case [A],  Wb	=0.116 + 0.00375 ht

=0.116 + 00375  20.24	=0.1919 kN/m

for loading case [B],  Wb	=13  0.1919	=0.0640 kN/m

(B)	Iteration procedure  [8 . 4 . 9]

Calculation of constant factors, C

| Loading case | [A] | [B] |
| --- | --- | --- |
| CMO=64×E×AM3×ℓ3×LMO CSO=64×E×AS3×ℓ3×LSO | 0.01273 0.00518 | 0.01269 0.005184 |

Iteration

| Step | Loading case | [A] |  |  | [B] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Iteration number | 1st | 2nd | 3rd | 1st | 2nd |
| (o) | First f1: case[A]; f1 ~ 1.002fd | 15.681 | 15.668 | 15.657 |  |  |
|  | Case[B]; ft ~ 1.001 ff |  |  |  | 16.476 | 16.460 |
|  | First x: case[A]; x ~ 0.015 l | 2.121 | 1.335 | 1.569 |  |  |
|  | Case[B]; x ~ 0.0025 l |  |  |  | 0.354 | 0.216 |
| (1) | 1=arcsinxf1+1,3 | 7.175 | 4.513 | 5.309 | 1.141 | 0.697 |
|  | 1=arctanxht+0,25−cosγ1×(f1+1,3) | 30.216 | 20.479 | 23.516 | 7.422 | 4.522 |
|  | C1=xsinα1 | 4.215 | 3.816 | 3.932 | 2.740 | 2.740 |
| (2) | PM1=CMO×f1×(f12−fo2)+ffo×PMO | 1.437 | 1.355 | 1.285 | 5.267 | 5.152 |
|  | PS1=CSO×C1×(C12−Co2)+CCo×PSO | 0.249 | 0.162 | 0.185 | 0.001 | 0.001 |
| (1) | PM = PM1 - PMO | 0.196 | 0.114 | 0.044 | 0.115 | 0 |
|  | g = -go+ (PM1cos1 - PS1cos1) | 0.083 | 0.071 | 0.018 | 0.113 | 0.001 |
|  | New f1 = fo+ (f1 - fo) ΔPM−ΔgΔPM | 15.668 | 15.657 | 15.654 | 16.460 | 16.460 |
|  | New x = XWb(PM1×sinγ1+PS1×sinα) | 1.335 | 1.569 | 1.562 | 0.216 | 0 |
| (4) | Δg |  0.02 |  0.02 |  0.02 |  0.02 |  0.02 |
|  |  |  |  | Stop iteration! |  |  |

Force diagram: for loading case [A]

Without  wind load	With wind load

Calculation of final data

| Symbol | Unit | Values for loading case |  | Formulas, Remarks |
| --- | --- | --- | --- | --- |
|  |  | A | B |  |
| PM1 PS1 1 1 f1 C1 | KN/m KN/m deg deg m m | 1.285 0.185 5.309 23.516 15.657 3.932 | 5.152 0.001 0.697 4.522 16.460 2.740 | Values from last iteration |
| Vtot | kN | 185.98 | 728.50 | PM1×ℓ2×cosγ1×1+ℓ×tanβf4×f1×cosγ1 |
| Hw | kN | 8.41 | 4.43 | PM1×ℓ2×sinγ1 |
| Gt | kN | 58.0 | 58.0 | From Table |
| P1 | kN | 26.95 | 353.30 | 12Vtot+Gt−ht×(HW+1.025×w×ht)CC1 |
| P2 | kN | 217.03 | 433.20 | 12Vtot+Gt+ht×(HW+1.025×w×ht)CC1 |
| PH | kN | 34.37 | 11.36 | HW+2.05×w×ht+PS1×ℓ2×sinα1 |
| TSV | kN | 5.22 | 0 | PS1×ℓ2×sinα1 |
| TSH | kN | 117.59 | 0.91 | PS1×ℓ28×C1 |
| TS | kN | 117.71 | 0.91 | PS1×ℓ28×C1×1+16×(C1ℓ)2 |
| FS | - | 9.94 | - | Tsbreak / TS |

Check of results

H	=	ℓ2  Wb + 2.05  ht   w - PH

[A]	=	0.5 141.4  0.1919 + 2.05  20.24 0.5 - 34.37	  = - 0.06 kN

[B]	=	0.5 141.4 0.333  0.1919+ 2.05  20.24  0.167 – 11.36	        =     0.09 kN

V	=	0.5 ℓ go + Gt+ 0.5 Vtot (P1  + P2   - TSV)

[A]	=	0.5 141.4 1.128 +58 + 0.5 185.98  (26.95 + 217.03 - 5.22)         	 = - 8.0 kN

[B]	=	0.5 141.4 5.152 + 58+ 0.5 728.5  (353.30 + 433.20)                  	=       0 kN

Check for tower capacity [Table 8 . 5 . 4]

Related Standard Drawings [8. 5. 5]

| Drawing Number | Drawing Title |
| --- | --- |
| 151 103 113 126 137 | Assembly drawing Base element drawing Intermediate element drawing Top element drawing ##### Saddle drawing |

# Design of walkway/tower foundation [8 . 6]

## Walkway/Tower foundation on rock, right bank

(A)	Foundation characteristics

River bank	:	Right

Foundation on	:	Rock;  without foot

(B)	Initial layout data  [8 . 6 . 5]

(1)	Loads on walkway/tower foundation

- Centre distance of tower legs	c/c1 =  4.0   m

- Centre distance of tower anchorage rods	c/c2= 550 mm

- Loads from tower:	(A)	(B)

Tower leg 1	P1 =   26.95 kN	353.30 kN

Tower leg 2	P2 = 217.03 kN	433.20 kN

Horiz. Load perpendicular to tower	PH =  34.37 kN	11.36 kN

- Loads from spanning cables;

Vertical	TSV = 5.22 kN	0  kN

Horizontal	TSH =177.59 kN		0.91 kN

- From survey and final geotechnical report:

Rock parameters

- Rock at depth	=  upto surface

- Sliding friction angle	SL =     40 deg

- Ground bearing pressure	perm = 400 kN/m2

- Minimum embedded depth		t =   1.0  m

- Design parameters  [Table 8 . 6 . 1]

| Foundation dimensions (m) | minimum | maximum |
| --- | --- | --- |
| Width B Length L ##### Total height (H+C) ##### Foot height C Foot width E | ##### 2.9 6.0 2.4 - - | ##### 5.0 9.0 10.0 - - |

(C)	Main calculation  [8 . 6 . 6]

(1)	Foundation dimensions  (by trial)

Width	        B = 2.9 m

Length		L = 6.0 m

Total height	(H+C)=2.4 m

Foot height	C  =  0

Foot width	E  =  0

(2)	Topographical conditions (from layout)

Active soil height at the back	ha  =  0

Soil height in the front	hp  =  0

Depth of ground water	hW  =  0

(3)	Unit weight of concrete	C = 22 kN/m3

(4)	Calculation of loading forces and moments

| Loading forces (kN) |  |  | Lever arm (m) for |  |  |  | Static moment (kN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Formula | Vert. | Horiz. | MX B |  | My L |  | Mx | My |
| Weight: W1=B L H c | 918.7 |  |  |  |  |  |  |  |
| Loads: case (A) P1 P2 TSV TSH PH | 26.95 217.03 -5.22 | 117.59 34.37 | H-0.25 | 2.15 | 0.5 C/C1 0.5 C/ C1 H | 2.0 2.0 2.4 | 252.82 | 53.90 434.06 82.49 |
| RV | 1157.46 |  |  |  |  |  | 252.82 | 462.65 |
|  | TSH | 117.59 |  |  |  |  |  |  |
|  | PH | 34.37 |  |  |  |  |  |  |
| 2.0 2.0 0 2.4 | 353.30 433.20 0 | 0.91 11.36 | H-0.25 | 2.15 | 0.5 C/C1 0.5 C/C1 0 H |  | 1.96 | - 706.60 886.40 27.26 |
| RV | 1705.20 |  |  |  |  |  | 1.96 | 187.06 |
|  | TSH | 0.91 |  |  |  |  |  |  |
|  | PH | 11.36 |  |  |  |  |  |  |

## Calculation of final data

|  | Formula/Symbol | Unit | (A) | (B) |
| --- | --- | --- | --- | --- |
| (5) | Calculation volume: |  |  |  |
|  | Vol =W122 | m3 | 41.76 |  |
| (6) | Location of resultant: ex =MXRy ex = | m | 0.22 | 0 |
|  | B∗2 B∗2= | m | 1.23 | 1.45 |
|  | ey ==MyRV ey = | m | 0.40 | 0.11 |
|  | L∗2 L∗2= | m | 2.60 | 2.89 |
|  | Check for; : L∗2 B*/2 B/3 ? L*/2 L/3 ? |  | Ok! Ok! | Ok! Ok! |
| (7) | Inclination of resultant: R =arctan(TSH+ΣEah)2+PH2RV2= | deg | 6.04 | 0.38 |
| (8) | Maximum ground bearing pressure: At B*/2B = And L*/2L = | - - | 0.42 0.43 | 0.50 0.48 |
|  | The Z – factor Z= | - | 1.36 | 1.07 |
|  | σmax.ult.=Z⋅RVB∗L∗ max.= | KN/m2 | 123.06 | 108.85 |
|  | Check for max.: max perm (=400 kN/m2) ? |  | Ok! | Ok! |
| (9) | Safety against toppling: FT checked by- |  | B*/2 and L*/2 | B*/2 and L*/2 |
| (10) | Safety against sliding: Fsl=tanφsltan(δR−α) Fsl= |  | 7.93 | 126.52 |
|  | Check for Fsl: Fsl 1.5 ? |  | Ok! | Ok! |

Compilation of Final Data  [8 . 6 . 7]; R/B, W/T Foundation

Bridge No. & Name:  Design example 2    Date: April 2004 Checked by N. L. Joshi

- And B)  Initial Data (refer to 8 . 6 . 5)

C)	Main Calculation

| 1) | Soil/rock heights |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Active pressure height at back | ha | = | - | m |
|  | - Rock height at back | Hra | = | - | m |
|  | - Active pressure height in front | Hp | = | - | m |
|  | - Rock height in front | hrt | = | - | m |
|  | - Depth of soil | t | = | - | m |
|  | - Depth of additional soil | t | = | - | m |
| 2) | Soil parameters |  |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 | = | - | Deg |
|  | - Slope of soil baseline | B | = | - | Deg |
|  | - Length of influence | L*infl. | = | - | m |
|  | - Back slope of soil |  | = | - | Deg |
| 3) | Foundation dimensions |  |  |  |  |
|  | - height | H | = | 2.4 | m |
|  | - height of foot | C | = | - | m |
|  | - Width | B | = | 2.9 | m |
|  | - Width of foot | E | = | - | m |
|  | - Length | L | = | 6.0 | m |
|  | - Distance to resultant force | B*/2 | = | 1.23 / 1.45 | m |
|  | and | L*/2 | = | 2.60 / 2.89 | m |
| 4) | Safety factors |  |  |  |  |
|  | - sliding | FSL | = | 7.93 / 126.52 | / |
|  | - Bearing capacity | FBC | = | - | / |
|  | - Toppling | FT | = | Ok | / |
|  | - Slope stability | FS | = | - | / |
| 5) | Anchorage rods |  |  |  |  |
|  | Nos. as per calculation | N | = | - | / |

D)	Additional Anchorage Rods

(from geological report) :  	as the rock is sound no additional

anchorage rod is provided.

## Walkway/Tower foundation on soil, left bank

(A)	Foundation characteristics

River bank	:	Left

Foundation on	:	Soil;  with foot

(B)	Initial layout data  [8 . 6 . 5]

(1)	Loads on walkway/tower foundation

- Centre distance of tower legs	 c/c1 =  4.00   m

- Centre distance of tower anchorage rods	c/c2= 550 mm

- Loads from tower:	(A)	(B)

Tower leg 1	P1 =  26.95 kN	353.30 kN

Tower leg 2	P2 = 217.03 kN	433.20 kN

Horiz. load perpendicular to tower	PH =  34.37 kN	11.36 kN

- Loads from spanning cables;

Vertical	TSV = 5.22 kN	0 kN

Horizontal	TSH =117.59 kN	0.91 kN

- From survey and final geotechnical report:

Soil parameters

- Sub-soil at depth                                                                   =   3.5  m

- Friction angle of sub-soil                                                     =    35 deg

- Unit weight of sub-soil                                                         =  19 kN/m3

- Friction angle of backfilling s                                              =    35 deg

- Unit weight of backfilling soil                                              =  18 kN/m3

- Ground water at depth                                                         =   3  m

- Ground bearing pressure	= 400 kN/m2

- Minimum embedded depth	                t =  1.4  m

- Design parameters  [Table 8 . 6 . 2]

| Foundation dimensions (m) | minimum | maximum |
| --- | --- | --- |
| Width B Length L ##### Total height (H+C) ##### Foot height C Foot width E | 2.9 6.0 2.4 1.2 1.0 | 5.0 9.0 10.0 - - |

(C)	Main calculation  [8 . 6 . 6]

(1)	Foundation dimensions (m)

B    = 2.9  m

Width

L    = 6.0  m

Length

(H+C) = 3.5  m

Total height

C    = 1.2  m

Foot height

E     = 1.0 m

- Foot width

(2)	Topographical conditions (from foundation layout)

ha     =  3.5          	m

hp       =  1.4	m

hra    =  0

hw    =  0.5	m

hrt    =  0

     =   32	deg

1    =  19	deg

(3)	Earth pressure coefficient

back side: ah	=cos2φ21+sin(φ2+δ)×sin(φ2−ψ)cosδ×cosψ2

With δ=23×φ2

=cos235o1+sin(35o+23×35o)×sin(35o−32o)cos2335o×cos3222

ah =  0.437

front side: aht 	=cos2φ21+sin(φ2+δ)×sin(φ2−ε1)cosδ×cosε12

=cos235o1+sin(35o+23×35o)×sin(35o+19)cos2335o×cos(−19o)2

aht =  0.1877

(4)	Unit weight of concrete	c  =  22	kN/m3

(5)	Calculation of loading forces and moments

| Loading forces (kN) |  |  | Lever arm (m) for |  |  |  | Static moment (kN-m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Formula | Vert. | Horiz | M x B |  | MyL |  | Mx | My |
| Weight: W1=B L H c | 880.4 | - | E / 2 | 0.5 | - | - | -440.2 | - |
| W2=(B+E) C L c | 617.76 | - | - | - | - | - | - | - |
| Ws=(hP – C - 0.5 E tan) E L 2 | 3.0 | - | B / 2 | 1.45 | - | - | 4.35 | - |
| Uplift : Wu=(B + E) hW L W | --117.0 | - | - | - | - | - | - | - |
| Earth pressure : |  |  |  |  |  |  |  |  |
| -back:Eah = ah ha22×L×γ2 | - | 289.1 | ha / 3 | 1.17 | - | - | 338.2 | - |
| Eav = Eah tan23×φ2 | 124.7 | - | (B+E) / 2 | 1.95 | - | - | - 243.2 | - |
| -front:Eaht = aht(hP−E×tanε)22×L×γ2 | - | -11.3 | ( hp-E tan  )/3 | 0.35 | - | - | -3.9 | - |
| Eavt=Eaht tan(23×φ2) | 4.9 | - | (B+E)/2 | 1.95 | - | - | 9.6 | - |
| Loads: case (A) P1 P2 TSV TSH PH | 26.95 217.03 -5.22 - - | - - - 117.59 34.37 | E/2 E/2 E/2 H+C-0.25 - | 0.5 0.5 0.5 3.25 - | 0.5 C/C1 0.5 C/C1 0 - H+C | 2.0 2.0 0 - 3.5 | -13.48 -108.51 -2.61 382.17 - | -53.90 434.10 0 - 120.30 |
| RV | 1752.52 |  |  |  |  |  | - 77.58 | 500.50 |
|  |  Eah | 277.80 |  |  |  |  |  |  |
|  | TSH | 117.59 |  |  |  |  |  |  |
|  | PH | 34.37 |  |  |  |  |  |  |
| Loads: case (B) P1 P2 TSV TSH PH | 353.30 433.20 0 - - | - - - 0.91 11.36 | E/2 E/2 E/2 H+C-0.25 - | 0.5 0.5 0.5 3.25 - | 0.5 C / C1 0.5 C / C1 0 - H + C | 2.0 2.0 0 - 3.5 | -176.65 -216.60 0 2.96 - | -706.60 866.40 0 - 39.76 |
| RV | 2300.26 |  |  |  |  |  | - 725.44 | 199.56 |
|  |  Eah | 277.80 |  |  |  |  |  |  |
|  | TSH | 0.91 |  |  |  |  |  |  |
|  | PH | 11.36 |  |  |  |  |  |  |

## Calculation of final data

|  | Formula/Symbol | Unit | (A) | (B) |
| --- | --- | --- | --- | --- |
| (6) | Calculation volume |  |  |  |
|  | Vol =W1+W2γc Vol. | m3 | 68.23 |  |
| (7) | Location of resultant eB = eX = MXRV eX = | m | - 0.04 | - 0.32 |
|  | B∗2= B+E2 B∗2= | m | 1.91 | 1.63 |
|  | eL = ey = MyRV ey= | m | 0.29 | 0.09 |
|  | L∗2 = L∗2= | m | 2.71 | 2.91 |
|  | Check for B∗2;L∗2: B*/2 B+E3 ? L*/2 L/3 ? |  | Ok! Ok! | Ok! Ok! |
| (8) | Inclination of resultant R=arctan(TSH+ΣEah)2+PH2RV2; R= | deg | 12.76 | 6.91 |
| (9) | Safety factor against sliding Fsl=tanφ1tanδR Fsl= Check for Fsl: Fsl 1.5 ? | - | 3.09 ok! | 5.77 ok! |
| (10) | Safety against toppling: | - | Checked by B*/2 and L*/2 |  |
| (11) | Maximum ground bearing pressure |  |  |  |
|  | Check: eBB+E+eLL = | - | 0.06  16 | 0.10  16 |
|  | Therefore, for load case (A) Z-factor = σmax.ult.=Z×RVB∗L∗ = | - KN/m2 | 1.03 87.18 | - - |
|  | for load case (B) σmax=RV(B+E)×L×1+6×eB(B+E)+eLL= | KN/m2 | - | 155.54 |
|  | Check : max perm? | KN/m2 |  400 |  400 |

| Formulas/Symbols |  | Unit | (A) | (B) |
| --- | --- | --- | --- | --- |
| (12) | Shear resistance of the ground L*infl. = 5.8B* L*infl.= | m | 22.16 | 18.91 |
|  | From General Arrangement; at L*infl.; B= | deg | 19 | 19 |
|  | q= | kN/m2 | 3.0 | 3.5 |
|  | Surcharge load = (hp - hw) 1 + hw (1 - W) + q = | kN/m2 | 20.6 | 22.6 |
|  | Bearing capacity factors (for 1 = 35o) Nq= | - | 33.3 | 33.3 |
|  | N= | - | 40.7 | 40.7 |
|  | Correction factors for: - Shape (B∗L∗=0.70 /0.56 and 1=35o) Sq= | - | 1.49 | 1.39 |
|  | S= | - | 0.72 | 0.78 |
|  | - Depth (tB∗= 0.37/0.43 and 1=35o) dq= | - | 1.09 | 1.10 |
|  | d= | - | 1.00 | 1.00 |
|  | - Inclination of load (R=12.76o/6.91o and =0) iq= i= | - - | 0.55 0.42 | 0.73 0.64 |
|  | - Inclination of found. base (=0 and 1=35o) bq= b= | - - | 1.00 1.00 | 1.00 1.00 |
|  | - Inclination of base line (B = 19o) gq = g = | - | 0.64 | 0.64 |
|  | P∗B.L.(hp−hw).γ1+hw(γ1−γw)+q×Nq×Sq×dq×iq×b'q | KN | 8399.8 | 10562 |
| (13) | ### Safety factor against ground shear FBC = P∗RV Check for FBC : FBC 2.0? |  | 4.8 ok! | 4.6 ok! |

Reinforcements :

- Section 1-1

M = 0.8max12E2L = 0.887.1812126.0	= 209 kN-m

H = C – 0.2	= 1.2  -  0.2                                                     = 1.0  m

As = Mη×h×σperm	= 2090.9×1.0×23.0×100	= 1011 mm2

ASmin= 0.02%  h  L = 0.0210-21.06.0106 	= 1200 mm2 >1011mm2

N1	=AS201=1200201 (16 mm)

N1 = 6 nos

n	 = ESEC	n = 10

	 = ASh⋅L=6×π4×1621.0×6.0×106	 = 2.0110-4

	=n×μ×1+2n×μ−1=10×2.01×10−41+210×2.01×10−4−1       = 0.614

#### Stresses

t	=1μ×(1−ξ3)×Mh2×L=1×1042.011−13×0.614×209×10−31.02×6.0

t   =       217.9 N/mm2

tperm = 230.0 N/mm2

c   =   0.14 N/mm2

c perm = 2 N/mm2

c	=2ξ×(1−ξ3)×Mh2×L=20.6141−13×0.614×209×10−31.02×6.0

#### Section 2-2

M	= TSH  (H-0.25) + ah[2,] (ha+hra−C)36×L×γ2

= 117.6 (6.0-0.25)+0.437(4.0+0+1.2)36×6.0×18	= 1782.2 kNm

h	= B - 0.5 = 2.9 - 0.5	=     2.4  m

As	= Mη×h×σperm=1782.20.9×2.4×23.0×100	=    3587 mm2

As (min) = 0.02% hL = 0.0210-2  2.4  6.0  106	=   2880 mm2

N2	=AS201=3587201 (16 mm)

N2 =    18 nos

n	=ESEC	n =       10

	=ASh×L=18×Π4×1622.4×6.0×106	 =  2.5110-4

	=n⋅μ⋅1+2n×μ−1=10×2.51×10−41+2×10410×2.51−1	=     0.0684

#### Stresses

t	=1μ×(1−ξ3)×Mh2×L=1×1042.51×1−13×0.0684×1782.2×10−32.42×6.0

= 210.2 N/mm2

tperm = 230.0 N/mm2

e	=2ξ×(1−ξ3)×Mh2×L=20.0684×1−13×0.0684×1782.2×10−32.42×6.0

= 1.5 N/mm2

tperm = 2.0 N/mm2

##### Related standard design drawings [8 . 6 . 8]

| Drawing Number | Bank | ##### Drawing Title |
| --- | --- | --- |
| 92 92/1 92/2 | Both Left Right | Walkway/tower anchorage (capacity: 390 kN) Walkway/tower foundation; c/c1=4,0m;c/c2=550mm Walkway/tower foundation; c/c1=4,0m;c/c2=550mm |

Compilation of Final Data  [8 . 6 . 7];  L/B, W/T Foundation

Bridge No. & Name:  Design example 2     Date: April 2004 checked by N. L. Joshi

  - and  B)  Initial Data (refer to 8 . 6 . 5)

C)	Main Calculation

| 1) | Soil/rock heights |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Active pressure height at back | ha | = | 3.5 | m |
|  |  | hra | = |  | m |
|  | - Active pressure height in front | hp | = | 1.4 | m |
|  | - Rock height in front | hrt | = | 0 | m |
|  | - Depth of soil | t | = | 1.4 | m |
|  | - Depth of additional soil | t | = | 0.32/0.37 | m |
| 2) | Soil parameters |  |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 | = | 19 | deg |
|  | - Slope of soil baseline | B | = | 19 | deg |
|  | - Length of influence | L*infl. | = | 22.16 / 18.91 | m |
|  | - Back slope of soil |  | = | 32 | deg |
| 3) | Foundation dimensions |  |  |  |  |
|  | - height | H | = | 2.3 | m |
|  | - height of foot | C | = | 1.2 | m |
|  | - Width | B | = | 2.9 | m |
|  | - Width of foot | E | = | 1.0 | m |
|  | - Length | L | = | 6.0 | m |
|  | - Distance to resultant force | B*/2 | = | 1.95 / 2.86 | m |
|  | and | L*/2 | = | 2.72 / 2.86 | m |
| 4) | Safety factors |  |  |  |  |
|  | - sliding | FSL | = | 3.09 / 5.77 | / |
|  | - Bearing capacity | FBC | = | 4.8 / 4.6 | / |
|  | - Toppling | FT | = | Ok | / |
|  | - Slope stability | FS | = | - | / |
| 5) | Anchorage rods |  |  |  |  |
|  | Nos. as per calculation | N | = | - | / |

# Design of main cable foundation [8 . 7]

Initial layout data  [8 . 7 . 5]

| Foundation characteristics - River bank - Foundation on | Unit | Right Rock | Left Soil |
| --- | --- | --- | --- |
| ### Main cable characteristics - Number of main cables nM = - Main cable tension TMf = - Main cable inclination f = - Tower leg centre distance c/c1 = | nos. kN deg m | 4 862.65 24.97 4.0 |  |
| ### Soil parameters - Sub-soil at depth = - Friction angle of sub-soil 1 = - Unit weight of sub-soil 1 = - Friction angle of backfill soil 2 = - Unit weight of backfilling soil 2 = - Ground water at depth - Ground bearing pressure perm = | m deg KN/m3 deg KN/m3 m KN/m2 | - - - 35 18 - - | 4.0 35 19 35 18 - 300 |
| ### Rock parameters - Rock at depth = - Sliding friction angle sl = - Ground bearing pressure perm = | m deg KN/m2 | 0.4 38 400 |  |
| ### Minimum embedded depth | m | 1.0 | 1.5 |

Design parameters  [Table 8 . 7 . 1]

| ### Foundation dimensions (m) |  | Minimum | Maximum |
| --- | --- | --- | --- |
| ### Width B = - Length L = - Back height H1 = - Front height H2 = |  | 4.9 6.4 3.3 1.5 | 5.5 8.4 5.5 4.5 |
| ### Front toe b = |  | 0.5 |  |

## Right river bank

Foundation on rock

Width

B     =  4.9   m

Length

L      =  6.4  m

Back height

H1    =  3.3  m

Front height

b      =  0.5  m

H2    =  1.5  m

- Front toe

From General Arrangement:

## ha  = 7.1  m

## ha1  = 5.0  m

## hra = 1.2  m

##  = 29  deg

Unit weight of concrete:	c = 22 kN/m3

Unit weight of gabion:	g = 20 kN/m3

## Additional load on top of foundation:

## For gabion arrangement shown in the section of General Arrangement:

A  = 307.0 kN

a   =  4.15  m

## Calculation procedure  [8 . 7 . 6]

## Weight of foundation and lever arm about F

W1	=0.5×(H1+H2+k+0.7)×B×L×γc

## =0.5×(3.3+1.5+0.4+0.7)×4.9×6.4×22	= 2035.3 kN

w1	=B3×2×(H1+0.1)+(H2+k+0.6)(H1+0.1)+(H2+k+0.6)

=4.93×2×(3.3+0.1)+(1.5+0.4+0.6)(3.3+0.1)+(1.5+0.4+0.6)	=   2.57 m

W2	=−b×(k+0.6)×L×γc=−0.5×(0.4+0.6)×6.4×22	= - 70.4 kN

w2	=b2=0.52	=   0.25 m

W3	=−0.5×b'⋅(k+0.6)×L×γc=−0.5×0.5×(0.4+0.6)×6.4×22	= -35.2 kN

w3	=	=    0.67 m

W4	=−0.5×(B−1)×0.1×L×γc=−0.5×(4.9−1)×0.1×6.4×22	= -27.5 kN

w4	=23×(B−1.0)+b'+b=23×(4.9−1.0)+0.5+0.5	=   3.6 kN

Total weight, Wtot	=  W1  - W2  - W3  - W4

= 2035.3 – 70.4 – 35.2 – 27.5	=     1902.2 kN

Volume of foundation

Vol.  =  86.46 m3

Vol	=Wtotγc=1902.222

Bottom inclination

	=arctanH1+0.1−1.0−H2B=arctan3.3−0.9−1.54.9	=     10.41 deg

Active earth pressure  [6. 2. 3]

ah	=cos2φ21+sin(φ2+δ)×sin(φ2−ϕ)cosδ×cosϕ2;δ=23×φ

=cos235o1+sin(35o+23×35o)×sin(35o+29o)cos23×35o×cos29o2	=      0.3777

Eah	=γah×ha2−ha122×L×γ2 = 0.3777×7.12−522×6.4×18	=    552.81 kN

YEah = ha+2ha13×ha−ha1ha+ha1 = 7.1+2×53×7.1−57.1+5                           =      0.99  m

Eav	=Eah×tanδ = 552.81×tan2×35°3                               =    238.46 kN

Loads due to cable tension

Tfh	= Tf cos f	= 862.65  cos 24.97o	=     782.0 kN

Tfv	= Tf sin f	= 862.65  sin 24.97o	=     364.2 kN

Resultant loading forces

RV	= Wtot + A + Eav – Tfv

= 1902.2 + 307 + 238.5 – 364.2	=    2083.5 kN

RH	= Eah + Tfh = 552.8 + 782.0	=    1334.8 kN

Static moment about F

Mf	= W1 w1  - W2 w2  - W3 w3  - W4 w4 + A a + Eav B

- Tfv b - Tfh  (H2 + K) - Eah  (YEah +  hra - H1 + H2 + K + 0.5)

= 2035.3 2.57 – 70.4 0.25 – 35.2 0.67 – 27.5 3.6

+ 307 4.15 + 238.5 4.9 – 364.2 05 -7 82.0  (1.5+ 0.4)

-552.8  (0.99 + 1.2 -3.3 + 1.5 + 0.4 + 0.5)	= 5152.4 kN-m

Inclination of resultant

R	=arctanRHRV	=    32.65 deg

Location of resultant

B∗2	=MFRV(1+tanα×tanδR)

=3689.02083.5×(1+tan10.41o×tan32.65o)	B∗2= 2.18 m

Check for B*/2

B∗2	= 2.18 m    B6   = 0.82 m	ok!

Safety factor against toppling

FT           =   M+M−      = 3.04      1.5	ok!

Calculation of ground bearing pressure

R'V	= RV  cos  + RH sin 

= 2083.46  cos10.41o + 1334.81  sin10.41 o	          R'V =   2290.31 kN

B'	= B  sec   = 4.9  sec 10.41 o	B' =    4.98  m

B∗2	= B∗2 sec   = 2.18  sec 10.41 o	B*'2 =   2.22  m

e'	= B'2 - 2.22	           e' =     0.27  m

max.min	=RV'B'L× = 2290.314.98×6.4×

max  = 95.2 kN/m2

min   =  48.5 kN/m2

Since    B2 > B∗2 >  B3

no anchorage rods are required for main cable foundation.

R'H	= RH cos  - RV  sin 

= 1334.81  cos10.41o + 2083.46  sin10.41 o	R'H =936.47 kN

Safety factor against sliding

Fsl	= tanφsl×RV'RH'=tan38o×2290.31936.47    	 Fsl = 1.91

Check for Fsl

Fsl	= 1.91         1.5	ok!

Additional  anchorage rods  [6 . 6 . 7]

From final geotechnical report

-	Rock type	:	Augen gneiss with quartz band

-	Weathering	:	Moderate

-	Rock quality	:	Highly fractured with open joints

Weakness planes/lines

-	Slope ~ bedding	:	320/55g (direction/amount of dip)

-	Intersection line 1	:	370/43g

-	Intersection line 2	:	360/39g

-	Intersection line 3	:	225/08g

-	Azimuth of bridge axis	:	318g

Provision of anchorage rods

-	The bedding plane 320/55g is nearly parallel to the bridge axis, i.e. , 318g, anchorage rods to stabilize this plane are provide in direction 120/45g i.e. perpendicular to 320/45g.

-	Since the intersection lines 1 and 2 are more or less in the same direction, anchorage rods perpendicular to their average direction i.e365/41g. , are provided, i.e.  in a direction of 165/59g.

-	As the bridge axis is oblique to the intersection lines 1 and 2, anchorage rods in direction 65/59g are also symmetrically provided.

-	Intersection line 3 is too lateral to the bridge axis and flat, no anchorage rod is provided to stabilize this line.

Number of anchorage rods

-	Nback	= B×L2×1.5=4.9×6.42×1.5	= 11 nos

-	Nfront	= 2×B×L2×1.5=2×4.9×6.42×1.5	= 21 nos

=32 (25mm) nos

-	Ntot	= Nback + Nfront = 11 + 21

Distribution of anchorage rods

-	direction 120/45g 	:	11 nos.

-	direction 165/59g 	:	10 nos.

-	direction 65/59g 	:	11 nos.

Compilation of Final Data  [8 . 7 . 7];  R/B, Main Cable Foundation

Bridge No. & Name:  Design example 2   Date: April 2004 checked by N. L. Joshi

  - And B)  Initial Data (refer to 8 . 7 . 5)

C)	Main Calculation

| 1) | Load on top of foundation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Total load | A | = | 307 | m |
|  | - Front to C.G. distance | a | = | 4.15 | m |
| 2) | Soil/rock heights |  |  |  |  |
|  | - Active pressure height | ha | = | 7.1 | m |
|  |  | ha1 | = | 5.0 | m |
|  | - Rock height at back | hra | = | 1.2 | m |
|  | - Embedded depth | t | = | 1.5 | m |
|  | - Depth of additional soil (if on soil) | t’ | = | - | m |
| 3) | Soil parameters |  |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 | = | 27 | deg |
|  | - Slope of soil baseline | B | = | - | deg |
|  | - Length of influence | L*infl. | = | - | m |
|  | - Back slope of soil |  | = | 29 | deg |
| 4) | Foundation dimensions |  |  |  |  |
|  | - Back height | H1 | = | 3.3 | m |
|  | - Front height | H2 | = | 1.5 | m |
|  | - Width | B | = | 4.9 | m |
|  | - Length | L | = | 6.4 | m |
|  | - Base inclination |  | = | 10.41 | m |
|  | - Distance to resultant force | B*/2 | = | 2.18 | m |
|  | and | L*/2 | = | 3.20 | m |
| 5) | Safety factors |  |  |  |  |
|  | - Sliding | FSL | = | 1.91 | / |
|  | - Bearing capacity | FBC | = | - | / |
|  | - Toppling | FT | = | 3.04 | / |
|  | - Slope stability | FS | = | - | / |
| 6) | Anchorage rods |  |  |  |  |
|  | Nos. as per calculation |  |  |  |  |

D)	Additional Anchorage Rods

(from geological report) :  	                       32	/

## Left river bank

Foundation on soil  [8 . 7 . 6]

Foundation dimensions (by trial)

B    =  4.9  m

Width

L    =  6.4  m

Length

H1  =  3.3  m

Back height

H2   =  1.5  m

Front height

b     =  0.5  m

- Front toe

From foundation layout:

## ha =   4.5  m

ha1 =  1.2  m

K = 0.4 m                       hp =   1.5  m

## b’ = 0.5 m                       hra =     0  m

## hw =      0  m

##  =  30 deg

1 = 33  deg

## Calculation procedure  [8 . 7 . 6]

## Weight of foundation and lever arm about F.

W1	=0.5×(H1+H2+k+0.7)×B×L×γc

## =0.5×(3.3+1.5+0.4+0.7)×4.9×6.4×22	= 2035.3 kN

w1	=B3×2×(H1+0.1)+(H2+k+0.6)(H1+0.1)+(H2+k+0.6)

=4.93×2×(3.3+0.1)+(1.5+0.4+0.6)(3.3+0.1)+(1.5+0.4+0.6)	=         2.57 m

W2	=−b×(k+0.6)×L×γc

=−0.5×(0.4+0.6)×6.4×22	=       -70.4 kN

w2	=b2=0.52	=         0.25 m

W3	=−0.5×b'×(k+0.6)×L×γc

=−0.5×0.5×(0.4+0.6)×6.4×22	=     -35.2 kN

w3	=b'3+b=0.53+0.5	=          0.67m

W4	=−0.5×(B−1.0)×0.1×L×γc

=−0.5×(4.9−1.0)×0.1×6.4×22	=    -27.46 kN

w4	=23×(B−1.0)+b'+b

=23×(4.9−1.0)+0.5+0.5	=       3.6 kN

Wtot	= W1 – W2 – W3  – W4

= 2035.3 – 70.4 – 35.2 – 27.5	=   1902.1 kN

Additional load on Foundation

Load of wall (Dry Stone Masonary):

A               =	0.5  6.4  1.2   20   	= 76.8 kN

Lever arm =  4.9  –  0.50.5	 = 4.65 m

Volume of foundation

Vol. =  86.46 m3

Vol	=Wtotγc=1902.122

Base inclination

	=arctanH1+0.1−1.0−H2B

=arctan3.3+0.1−1.0−1.54.9	=    10.41 deg

Active earth pressure  [6 . 2 . 3]

ah	=cos2φ21+sin(φ2+δ)×sin(φ2−ψ)cosδ×cosψ2;δ=23⋅φ

=cos235o1+sin(35o+23×35o)×sin(35o−30o)cos23×35o×cos30o2	=      0.3938

Eah =	0.5×γah×ha2−ha12×L×γ2=0.3938×0.5×4.52−1.22×6.4×18	=  426.7 kN

Vertical distance “YEah” from base (back side) of foundation block to the resultan force Eah;

YEah  =	1.33 m

Eav  =	Eah×tanδ=426.7×tan23×35o	       	  =  184.1 kN

Loads due to cable tension

Tfh	= Tf  cosf	=862.65  cos 24.97o	=   782.0 kN

Tfv	= Tf  sinf	=862.65  sin 24.97o	=   364.2 kN

Resultant loading forces

RV	= Wtot + A + Eav – Tfv

= 1902.1 +76.8 + 184.1 – 364.2	= 1798.8 kN

RH	= Eah + Tfh

= 426.7 + 782.0	= 1208.7 kN

Static moment about F

Mf	= W1 w1  - W2 w2  - W3 w3  - W4 w4 + Aa + Eav B

- Eah  (YEah – B tan) - Tf v  b - Tf h  (H2+ k)

= 2035.3 2.57 – 70.4 0.25 – 35.20 0.67 – 27.5 3.6+ 76.8 4.65  + 184.1 4.9

- 426.7 (1.33 - 4.9 tan10.41) - 364.2 0.5 - 782 (1.5 + 0.4)	= 4498.1 kN-m

Inclination of resultant

R	=arctanRHRV=arctan1208.71798.8	=     33.9 deg

Location of resultant

B∗2	=MFRV(1+tanα×tanδR)

=4498.11798.8×(1+tan10.41o×tan33.9o)	=         2.23 m

Check for B*/2

B∗2	= 2.23 m  >  B4=4.94   = 1.23 m	ok!

Safety factor against sliding

Fsl	= tanφ1tan(δR−α)=tan35otan(33.9o−10.41o))    	   =           1.61

Check for Fsl                	:   Fsl    = 1.61     >    1.5	ok!

Safety factor against toppling

FT	=M+M−=6489.71991.6

Check for FT	:  FT = 3.26  >   1.5	ok!

Ultimate bearing capacity and safety factor against

shear failure of the ground  [6. 5. 4]

L*infl. 	= 5.8 B*	 = 5.8  2  2.23	=       25.9  m

From General Arrangement

at L*infl.	= 25.9 m	B =    33.0  deg

Surcharge load 	= 1 t+q = 19 1.5 + 0	= 28.5 kN/m2

Bearing capacity factors (1 = 35)                                     Nq = 33.3

N  = 40.7

Correction factors for

Shape (B∗L∗ = 0.70 and 1 = 35o)	sq = 1.49

s = 0.72

Depth (tB∗ = 0.35 and 1 = 35o)	dq = 1.08

d =   1.0

Inclination of resultant (R = 33.9o &  = 10.41o)	iq = 0.29

i = 0.18

Inclination of foundation base (=10.41o & 1=35o)	bq = 0.77

b = 0.71

Inclination of base line (B = 33.0o)	gq  =  g = 0.41

Shear resistance of the ground

P=B∗L∗[(γ1×t+q)×Nq×Sq×dq×iq×bq'×gq+12×γ1×B∗Nγ×Sγ×dγ×iγ×bγ'×gγ]

=2×2.23×6.4×[28.5×33.3×1.49×1.08×0.29×0.77+12×19.0×2×2.23×40.7×0.72×1×0.18×0.71]×0.41	 =     5605.9  kN

FBC	=P∗RV=5605.91798.8	=               3.11

Ground bearing pressure  [6. 2. 7]

RV'	RV×cosα+RH×sinα

=1798.8×cos10.41o+1208.7×sin10.41o	=     1987.6  kN

B	=Bcosα=4.9cos10.41o	=           4.96 m

B*'2	=B*/2cosα=2.23cos10.41o  	=          2.27  m

e'	=B'2−B*'2=4.92−2.27	=             0.18 m

Since  B′2>B*'2>B'3

σmax	=RV'B'L×1+6×e'B'

=1987.64.96×6.40×1+6×0.184.96	= 76.2  kN/m2

Check for max

max	= 76.2 kN/m2 < perm = 300 kN/m2	ok!

Compilation of Final Data  [8 . 7 . 7];  L/B, Main Cable Foundation

Bridge No. & Name:  Design example 2  Date: April 2004 Checked By N. L. Joshi

A) And B)  Initial Data (refer to 8. 7. 5)

C)	Main Calculation

| 1) | Load on top of foundation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Total load | A | = | 76.8 | kN |
|  | - Front to C.g. distance | a | = | 4.65 | m |
| 2) | Soil/rock heights |  |  |  |  |
|  | - Active pressure height | ha | = | 4.5 | m |
|  |  | ha1 | = | 1.2 |  |
|  | - Rock height at back | hra | = | - | m |
|  | - Embedded depth | t | = | 1.5 | m |
|  | - Depth of additional soil (if on soil) | t | = | 0 | m |
| 3) | Soil parameters |  |  |  |  |
|  | - Front slope of soil (top of dry stone pitching) | 1 | = | 33 | deg |
|  | - Slope of soil baseline | B | = | 33 | deg |
|  | - Length of influence | L*infl. | = | 25.9 | m |
|  | - Back slope of soil |  | = | 30 | deg |
| 4) | Foundation dimensions |  |  |  |  |
|  | - Back height | H1 | = | 3.3 | m |
|  | - Front height | H2 | = | 1.5 | m |
|  | - Width | B | = | 4.9 | m |
|  | - Length | L | = | 6.4 | m |
|  | - Base inclination |  | = | 10.41 | deg |
|  | - Distance to resultant force | B*/2 | = | 2.23 | m |
|  | and | L*/2 | = | 3.2 | m |
| 4) | Safety factors |  |  |  |  |
|  | - Sliding | FSL | = | 1.61 | / |
|  | - Bearing capacity | FBC | = | 3.11 | / |
|  | - Toppling | FT | = | 3.26 | / |
|  | - Slope stability | FS | = | - | / |
| 5) | Anchorage rods |  |  |  |  |
|  | Not provided |  |  | - |  |

## Data to be transferred to the General Arrangement

All the initial layout data and the results displayed

by the frame

##### Related standard design drawings [8 . 7 . 8]

| ##### Drawing Number | Bank | ##### Drawing Title |
| --- | --- | --- |
| 49 49/1 | Both Both | Main cable anchorage for 4 main cables (capacity: 1220 kN) Main cable foundation for 4 main cables c/c1 = 4.0 m |

# Determination of suspender length [8 . 8]

Initial layout data      [8. 8. 4]

- Design span	l = 141.40 m

- Dead load sag	fd = 15.65 m

- Dead load camber	cd =  3.54 m

Main Calculation

Formula

- c/cn	= 4×(fd+cd)ℓ2×Xn2+1.3

With Xn	= 1.2  (n - 1)

With n	= 1 at mid-span to nmax continuously

- nmax		= ℓ−4.62.4+1=141.4−4.62.4+1	=  58  nos

- ln	= c/cn - 542

- jn	= INTEGERℓn−3501650

- lr	= ln - 1650 jn

- lrc	= lr + 180 for  12 mm suspenders

- = lr + 240  for  16 mm suspenders

- Wn12	= 1.625 jn + 0.888 10-3 lrc for 12 mm suspenders

- Sn12	= 0.069 jn + 0.0377 10-3 lrc 12 mm suspenders

- Wn16	= 3.286 jn + 1.58 10-3 lrc 16  mm suspenders

- Sn16	= 0.104 jn + 0.05 10-3 lrc 16  mm suspenders

Tabular calculation

| n | Xn (m) | Dia (mm) | c/cn (mm) | ln (mm) | Jn (nos) | lr (mm) | lrc (mm) | Wn (kg) | Sn (m2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 2 3 4 . . | 0 1.2 2.4 3.6 | 16 16 16 16 | 1300 1306 1322 1350 | 758 764 780 808 | 0 0 0 0 | 758 764 780 808 | 998 1004 1020 1048 | 1.6 1.6 1.6 1.7 | 0.03 0.03 0.04 0.04 |
| 21 22 . . | 24.0 25.2 | 16 12 | 3511 3738 | 2969 3196 | 1 1 | 1319 1546 | 1559 1726 | 5.7 3.2 | 0.18 0.13 |
| 55 56 57 58 | 64.8 66.0 67.2 68.4 | 12 12 12 12 | 17421 18023 18637 19262 | 16879 17481 18095 18720 | 10 10 10 11 | 379 981 1595 570 | 559 1161 1775 750 | 16.7 17.3 17.8 18.5 | 0.71 0.74 0.76 0.79 |

## Calculation of final data

- Total number of suspenders

N	=ℓ−3.400.60=141.4−3.40.6

N = 230  nos

Number of  16 mm suspenders from mid span,

n16  = lnt l7.2+1

= lnt 141.47.2+1

= lnt (19.64) + 1 = 20 + 1  = 21

- Total suspenders weight

W	=2×W1+4×n=258Wn

W = 1681.08 kg

- Total surface of suspender rods

S	=2×S1+4×n=258Sn

S = 66.39  m2

Data to be transferred to Standard Design Drawing [8 . 8 . 5]

- To the suspender list for each suspender

All the results except Xn from Tabular calculation.

- To the steel part list:

W = 1681.08 kg

S  = 68.72 m2

##### Related standard design drawings [8 .8 .6]

| ##### Drawing Number | ##### Drawing Title |
| --- | --- |
| 32 | Suspenders for 4 main cables |

# Design of stabilizing measures  [8 . 9]

Stabilizing cables   [8 . 9 . 1]

- fix first stabilizing cables at 8.90 m (d1= 8.90 m) from either tower saddle.

- Fix second stabilizing cables at 17.3 m (d2=17.3 - 8.9 = 8.4 m) from either tower saddle

Length of Stabilizing cables

- At d1 = 8.9 m

Main cable height	= 1.05+3.54+fdV2×ℓ2−d12

= 4.59+15.6570.702141.42−8.92	= 16.55 m

Length of first stabilizing cables, ℓ1=8.92+16.552	= 18.79 m

- At d2'= 17.3 m

Main cable height	= 1.05+3.54+fdV2×ℓ2−d2'2

= 4.59+15.6570.702141.42−17.32	= 13.52 m

Length of 2nd stabilizing cables, ℓ2=17.32+13.522	= 21.96 m

Check for cable inclination, 

 = arctan13.5217.30=38o  > 35  and < 45	= ok!

Data to be transferred to General Arrangement [8 . 9 . 3]

- Distances	:	d1 =  8.9  m

d2 =   8.4  m

- Cable lengths	:	l1 = 418.79 m

l2 = 421.96 m

Into cable list

- Total length = 4  18.79 + 4  21.96 + 16  0.4	=    170 m

##### Related standard design drawings [8 . 9 . 4]

| ##### Drawing Number | ##### Drawing Title |
| --- | --- |
| 22 | Stabilizing cable clamp for 4 main cables |

# Design of windguy cable structures [9 . 4]

Initial layout data  [9 . 4 . 6]

Span, l		l = 141.40 m

Elevation, 	H1 = 84.0 – 0.25                                       H1 =  83.75 m

Camber, 		                                                                   cd =   3.54 m

r = l/2 	  = 141.4/2                                                          r =  70.70 m

For suspension bridge;	d =    4.8 m

k =  0.735 m

Topographical and geological condition

- Check concave slope contours upstream of bridge axis on right bank.

- Slope is gentle and no geomorphological instability exists.

(survey reort).

Calculation procedure

Determination of theoretical parabola in plan

- Vertex, V	= ℓ2	V =   70.7 m

- Windguy cable sag at mid-span:

BW = fW	~ ℓ9	fW =  bW =  17.65 m

- HW	= w×ℓ28×bw	HW =  70.8 kN

TR = TL	= HW×1+2×fWV2

70.8×1+2×17.6570.702

TR = TL = 79.1 kN

Number and diameter of windguy cable

Number of windguy cable                                                nw = 1

Diameer of windguy cable                                               w = 26 mm

Permissible tension	Tperm = 129  kN

Safety factor, F =TbreakTmax=38679.1 =4.8  3.0	ok!

Windguy cables shall be provided symmetrically both upstream and downstream.

Location of first windties

BR	=	V - (d iR) = 70.7 – 4.8 13

BR = 8.3  m

BL	=	l – V - (d iL) = 141.4 – 70.7 - 4.8 14

BL = 3.5  m

Calculation of ’S

R	=	arctan2×fwV2×(V−BR)

=	arctan2×17.6570.72×(70.7−8.3)

R = 23.8  deg

L	=	arctan2×fwV2×(L−V−BL)

=	arctan2×17.6570.72×(141.4−70.7−3.5)

L = 25.4  deg

Calculation of CRO and CLO

CRO	=	fwV2×(V−BR)2+BR×tanαR+2.2

=	17.6570.72×(70.7−8.3)2+8.3×tan23.8o+2.2	= 19.61 m

CLO	=	fwV2×(ℓ−V−BL)2+BL×tanαL+2.2

=	17.6570.72×(141.4−70.7−3.5)2+3.5×tan25.4o+2.2	= 19.81 m

- Above data are plotted in the plan of the General Arrangement.

- The accurate axes of Windguy cable ends are located.

- Longitudinal sections along Windguy cable ends are drawn.

- From the plot, optimum Windguy foundation locations are determined as follows:

Foundation locations

|  | Right bank | Left bank |
| --- | --- | --- |
| Upstream Downstream | HR = 83.6 m DR = 4.8 m HR = 83.8 m DR = -5.6 m | HL = 82.5 m DL = 0 m HL = 82.6 m DL = 0 m |

Check for freeboard

- In the section of the General Arrangement, freeboard line at elevation 79.0+5.0=84.0 m and the Windguy cable alignments both upstream and downstream are drawn.

- From the plot, it is found that the Windguy cable on left bank passes slightly below (~20 cm) freeboard line.

- Therefore, the Windguy cable on left bank requires to be raised slightly.

Revised foundation locations

|  | Right bank | Left bank |
| --- | --- | --- |
| Upstream Downstream | HR = 83.6 m DR = 4.8 m HR = 83.8 m DR =-5.6 m | HL = 83.20 m DL = 1.5 m HL = 83.3 m DR = 1.5 m |

By trial, the windguy cable alignment clears off the freeboard line.

Calculation of  CR  and  CL

For upstream windguy cable

CR	=	fwV2×(V−BR)2+tanαR(BR+DR)+2.2

=	17.6570.72×(70.7−8.3)2+tan23.8o(8.3+4.8)+2.2

CR = 21.73  m

CL	=	fwV2×(ℓ−V−BL)2+tanαL(BL+DL)+2.2

= 17.6570.72×(141.4−70.7−3.5)2+tan25.4o×(3.5+1.5)+2.2

CL = 20.52 m

Calculation of windtie lengths  [9 . 5 . 7]

For upstream windguy arrangement

(1)	Determination of parabola 4

a4	=	fwV2	    a4 = 0.003531

c4	= 2.2 –  k = 2.2 - 0.735                                                     c4 = 1.465 m

(2)	Calculation of  hLP

hLP	=	yLP  tan 

yLP	=	a4 (r - V)2+c4

=	0.003531 (70.7-70.7)2 + 1.465	    = 1.465 m

tan 	=	(H1−HR−cd)×(ℓ+DR+DL)−(DR+r)×(HL−HR)(CR−k)×(ℓ+DR+DL)−(DR+r)×(CR−CL)

L + DR + DL  =  141.4 + 4.8 + 1.5	      = 147.7 m

DR + r	=	  4.8 + 70.7	                                   = 75.5 m

Tan 	=	(83.75−83.6+3.54)×147.7−75.5×(83.2−83.6)(21.73−0.735)×147.7−75.5×(21.73−20.52)       = 0.191

hLP	=	1.465  0.191                                                      = 0.28 m

(3)	Determination of parabolas (1), (2) and (3)

Parabola (1)	a1 	=   −cdr	 = - 0.000708

C1 	= hLP	= 0.28 m

Parabola (2)	a2 	= HR+(−cd))+ΔhLP−H1(r+DR)2

= 83.6−3.54+0.28−83.75(70.7+4.8)2	= - 0.000598

C2 	= 0

Parabola (3)	a3 	= HL+(−cd)+ΔhLP−H1(ℓ−r−DL)2

= 83.2−3.54+0.28−83.75(141.4−70.7+1.5)2	= - 0.000731

C3 	= 0

(4)	Calculation of windtie data

C/Ci	= Δhi2+yi2

DWi	= dcosβi×a42×(d+2xi)2+1

Where in –

xi	= V - BR - (i-1)  d = 70.7 - 8.3 - 4.8  (i-1)	= 62.4 - 4.8 (i-1)

xi’	= xi

imax	= ℓ−BR−BLd+1=141.4−8.3−3.54.8+1	=          28

yi	= a4  xi2 + c4 = 0.003531xi2 + 1.456	= 0.003531xi2 +1.465

for xi’    0

hi	=	a1  x’i2 -  hLP  -  a2  x’i2

=	- 0.000708  x’i2  +  0.28  +  0.000598  x’i2

 i	=	arc tan (2  a3  x’i) = arctan {2  (-0.000598)  x’i}

for xi’    0

hi	=	a1  x’i2 + hLP   - a3  x’i2

=	-0.000708  xi2+ 0.28 + 0.000731  xi2

 i	=	arc tan (2  a3  x’i) = arc tan {2  (- 0.000731)  xi’}

Starting from right side (i=1), the windtie data are calculated continuously upto
imax on left side as shown below:

Tabular calculation

| i | Xi = xi (m) | hi / tani | c/ci (mm) | Dwi (m) |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 | 62.4 57.6 52.8 48.0 43.2 38.4 33.6 28.8 24.0 19.2 14.4 9.6 4.8 0,0 -4.8 -9.6 -14.4 -19.2 -24.0 -28.8 -33.6 -38.4 -43.2 -48.0 -52.8 -57.6 -62.4 -67.2 | Take values for xi  0 Take values for xi  0 | 15.21 13.18 11.31 9.60 8.06 6.67 5.45 4.40 3.51 2.78 2.21 1.81 1.57 1.49 1.57 1.81 2.,21 2.78 3.51 4.40 5.46 6.68 8.06 9.60 11.31 13.18 15.22 17.41 | 5.23 5.16 5.10 5.05 5.00 4.96 4.92 4.88 4.86 4.83 4.82 4.81 4.80 4.80 481 4.82 4.84 4.86 4.89 4.92 4.96 5.00 5.05 5.11 5.17 5.29 5.30 |
|  |  |  | 190.45 | 134.18 |

(5)	Calculation of  R  and  L

R	= arc tan [2  a2  (BR - r)]

R = 4.27  deg

= arc tan [2  (- 0.000598)  (8.3 - 70.7)]

L	= arc tan [2  a3  (r – l + BL)]

L = 5.61  deg

= arc tan [2  (- 0.000731)  (70.7 - 141.4 + 3.5)]

(6)	Calculation of ER and EL

ER	= BR+DRcosαR×cosβR

ER = 14.36  m

= 8.3+4.8cos23.8o×os4.72o

EL	= BL+DLcosαL×cosβL

= 3.5+1.5cos25.4o×cos5.61o

EL = 5.56  m

(7)	Calculation of total lengths of cables

LW (tot)	=  Dwi + ER + EL + Overlapping length

= 134.18 + 14.36 + 5.56 + 2 1.70	= 157.50 m

Lt (tot)	=  (c/ci + 0.60)

= 190.45 + 28  0.60	= 207.25 m

## Downstream Windguy cable arrangement

Similar to the upstream Windguy cable arrangement manual calculation for downstream Windguy cable arrangement is also to be made.

Compilation of Final Data    [9 . 4 . 8];       U/S W.G.Cable Structures

Bridge No. & Name:  Design example 2  Date:  April 2004 Checked By N. L. Joshi

A)   Initial Data (refer to 9. 4. 6 and GA)

| Bridge type (suspended or suspension) ? |  | Suspension | / |
| --- | --- | --- | --- |
| Design span | ℓ = | 141.6 | m |
| Horizontal distance | r = | 70.7 | m |
| Windguy cable | nW = | 1 | / |
|  | W = | 26 | mm |
|  | AW = | 292 | mm2 |
|  | TW,break = | 386 | kN |
| E-Module | = | 110 | KN/mm2 |
| Cable anchorage elevation: |  |  |  |
| - Left bank Windguy cable, upstream | = | 83.20 | m |
| Windguy cable, downstream | = | 83.30 | m |
| - Right bank Windguy cable, upstream | = | 83.60 | m |
| H1 | = | 83.75 | m |
| Windguy cable, downstream | = | 83.80 | m |
| Freeboard Windguy cables | = | 5.00 | m |
| Loads: - Wind load | w = | 0.5 | KN/m |

B)   Data from Main Calculations

|  | Upstream | Downstream |  |
| --- | --- | --- | --- |
| Theoretical parabola hW = | 0 |  | m |
| bW = | 17.65 |  | m |
| fW = | 17.65 |  | m |
| TR = | 79.10 |  | kN |
| TL = | 79.10 |  | kN |
| Safety factor = | 4.80 |  | / |
| Layout L = | 25.40 |  | deg |
| R = | 23.80 |  | deg |
| CLo = | 19.81 |  | m |
| CLo = | 19.61 |  | m |
| CL = | 20.52 |  | m |
| CR = | 21.73 |  | m |
| DL = | 1.50 |  | m |
| DR = | 1.50 |  | m |
| HL = | 83.20 |  | m |
| HR = | 83.30 |  | m |

## Data to be transferred to the General Arrangement

## Into plan

All the initial layout data and the results displayed 
by the frame

## Into longitudinal section

## Separate longitudinal section of all four cable ends

## Freeboard

## Windguy cable elevation (H S), inclination ( S) and tension as given in the following table:

| Location | H (m) |  (deg) | T (kN) |
| --- | --- | --- | --- |
| Right bank: upstream downstream Left bank: upstream downstream | 83.6 83.8 83.2 83.3 | 4.27 ….. 5.61 ….. | 158.3 158.3 158.3 158.3 |

## Into cable list

## Windguy cable: 	Upstream:

## - number                           nW =    1

- diameter                                   W = 26 mm

Total cutting length;         LW    = 159 m

## Downstream:

## - number	nW =     1

- diameter                                  W=   26 mm

Total cutting length;        LW    =  ……m

## Windties: 	 - diameter                             = 13 mm

Total length;            LT (tot.)    = 208 m

Related standard drawings  [9 . 5 . 9]

| Drawing number | Drawing title |
| --- | --- |
| 10 | Windtie for windguy cable 26 mm  |

# design of Windguy cable foundation  [9 . 6]

## Initial layout data [9 . 6 . 5]

| #### Foundation Characteristics River bank Side Foundation on | Right upstream Rock | Left upstream soil |
| --- | --- | --- |
| #### W.G. cable structure characteristics |  |  |
| Number of windguy cable nW = | 1 | 1 |
| Windguy cable tension TW = | 79.1 kN | 79.1 kN |
| Cable inclination  = | 4.27o | 5.61o |
| Front slope 1 = | - | 17o |
| #### Soil parameters |  |  |
| Sub-soil at depth = | - | 4.65 m |
| Friction angle of sub-soil 1 = | - | 35o |
| Unit weight of sub-soil 1 = | - | 19 kN/m3 |
| Friction angle of backfilling soil; 2 = | - | 35o |
| Unit weight of backfilling soil; 2 = | - | 18 kN/m3 |
| Ground water at depth hW = | - | - |
| Ground bearing pressure perm = | - | 300 kN/m2 |
| #### Rock parameters |  |  |
| Rock at depth = | 0.20 m | - |
| Sliding friction angle sl = | 40o | - |
| Rock quality coefficient k = | 1.75 | - |
| Ground bearing pressure perm = | 400 kN/m2 | - |
| Minimum embedded depth | 0.5 m | 1.0 m |

## Design parameters for 1 no  26 mm cable [9.6.3]

| Foundation on | Rock |  |  | Soil |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Foundation dimensions (m) | ##### maximum | minimum |  | maximum | minimum |  |
| Width B = | 2.4 | 1.70 |  | 2.4 | 2.0 |  |
| Length L = | 3.0 | 1.2 |  | 3.5 | 1.5 |  |
| -For-  = | - | 6o | 6o | - | 6o | 6o |
| Front height hT = | 2.0 | 0.4 | 0.6 | 2.0 | 0.4 | 0.6 |
| Front height H2 = | - | hT + 0.4 |  | - | hT + 0.4 |  |
| C.G. distance s = | 0.75 |  |  | - |  |  |

Calculation procedure   [9 . 6 . 6]

- Determine the trial foundation dimensions with due consideration of economical design.

- Draw the cross-section at each Windguy cable anchorage end.

- Determine the topographical parameters.

- Calculate additional load on top of foundation, if applicable.

- Calculate active earth pressure, if required.

- Calculate the loading forces and static moments.

- Check the foundation for safety parameters.

- Compile the final data, data to be transferred to the General Arrangement and the Related Standard Design Drawings for all four foundations.

Detailed calculation procedure shown for windguy cable foundation, Suspended Bridge [page 316] can be followed for Suspension Bridge too.

If the foundation has to be designed on rock and the final geotechnical report suggests the provision of additional anchorage rods, the design procedure for additional anchorage rods as shown for right bank Main Cable Foundation, Suspension Bridge

[page 351] has to be followed.

# Design Example of deadman anchorage foundation [10. 4]

Initial layout data  [10. 4. 5]

| Foundation characteristics - River bank - Foundation on | Unit | Left Soil |
| --- | --- | --- |
| ### Main cable characteristics - Number of main cables nM = - Main cable tension TMf = - Main cable inclination f = - Tower leg centre distance c/c1 = | nos. kN deg m | 4 862.65 24.97 4.0 |
| ### Soil parameters - Sub-soil at depth = - Friction angle of sub-soil 1 = - Unit weight of sub-soil 1 = - Friction angle of backfilling soil 2 = - Unit weight of backfilling soil 2 = - Ground water at depth - Ground bearing pressure perm = | m deg KN/m3 deg KN/m3 m KN/m2 | 4.0 30 18 30 17 - 300 |

From General Arrangement

Back slope angle,      =  10.0  deg

Front slope angle,       =  10.0  deg

Active earth pressure  [6 . 2 . 3]

ah	=cos2φ21+sin(φ2+δ)×sin(φ2−ψ)cosδ×cosψ2;δ=23⋅φ

=cos230o1+sin(30o+23×30o)×sin(30o−10o)cos23×30o×cos10o2	=      0.32

Passive earth pressure

ph	=cos2φ11−sin(φ1−δ)×sin(φ1+ε)cosδ×cosε2;δ=−12⋅φ1

=cos230o1−sin(30o+12×30o)×sin(30o−10o)cos12×30o×cos10o2 		=   3.05

Earth pressure at rest

0 =  1 – sin 1   =   0.50

Design parameters  [Drawings 49/2]

| ### Foundation dimensions (m) | Formula | Values, m |
| --- | --- | --- |
| ### Anchorage Length A = - B = - Height H = - height h1 = - height h2 = -  h = - height h3  - active height ha = - passive height hp = | 4.125 + 0.71 n A cos  - 0.5 sin  A sin  + 0.5cos  + 0.3 1.41 + 1.59 tan ( - 26.56) Correct value is found by iteration H – h2 – 0.50 +  h h2 + h3 + 1.8 tan 4 h2 + h3 | 10.52 9.32 5.19 1.37 3.0 0.45 1.24 4.37 4.24 |

Eah =	γah×ha22×L×γ2                            =   311.67 kN

Eap  =	γph×hp22×L×γ1	       	  =  2960.91 kN

2E0L =  13 0 1 hp3 tan 1 λph+λah

=  0.33  0.5 18 4.24 3 tan 30  3.05+0.32       =         302.21  kN

Hult  = Eph – Eah + 2 E0L  = 2960.91 – 311.67 + 302.21                =    2951.45 kN

Loads due to cable tension

Tfh	= Tf  cosf	=862.65  cos 24.97o	                     =       782.0 kN

Tfv	= Tf  sinf	=862.65  sin 24.97o	                     =       364.2 kN

Weight of Foundation Block (refer Drawing)

WF = BF  L  h2  C  =  1.8  6.0  3.0  22                                =     712.80 kN

Weight of Earth over Foundation Block

WE = BF  L  h3  2  =  1.8  6.0  1.24  17                                =    227.66 kN

Safety factor against sliding

Fsl	=   Hult+WF+WE−Tfvtanφ1Tfh    =

=  2951.45+712.8+227.66−364.2tan30°782.0                             =     4.2

Check for Fsl                	:   Fsl    = 4.2    >    3.5	ok!

## Data to be transferred to the General Arrangement

All the initial layout data and the results displayed

by the frame

##### Related standard design drawings

| ##### Drawing Number | Bank | ##### Drawing Title |
| --- | --- | --- |
| 49/2 | Left | Main cable deadman anchorage foundation capacity: 1220 kN, c/c1 = 4.0 m 4 main cables (related drawing 49) |