12.	Appendix

- Table of Contents

12.1	Cable Statics	269

12.1.1	Parabolic Cable	269

12.1.2	Catenary	270

12.2	Geometry of the Parabola	271

12.2.1	General Case	271

12.2.2	Vertex at the0Intersection of the Coordinate Axes	271

12.3	List of Standard Drawings	272

12.3.1	Standard Working Assembly Drawings	272

12.3.2	Standard Structural Drawings	273

12.3.3	Special Design Drawings	275

12.4	Design Examples	276

12.4.1	Suspended Bridge	276

12.4.2	Suspension Bridge	276

12.5	Cost Estimate Norms/Formats (21 Pages)

12.6	Survey Form and Checklist (42 Pages)

12.1	Cable Statics

12.1.1	Parabolic Cable

A cable hanging between the supports A and B, carrying a load uniformly distributed along the horizontal, will form a parabola. Cables of suspension bridges may be assumed to be loaded in this way, since the weight of the cables is less than the weight of the walkway. Denote by q , the load per unit length (measured horizontally) and express in kN/m . Choosing coordinate axes originating at the lowest point, C , of the cable, the magnitude, W , of the total load carried by the portion of the cable, extending from C to the point, D , of coordinates x and y , can be expressed as:

W	 =   q  x	kN

- Drawing the corresponding force triangle, the following relations can be obtained

T· cos =   To	T·sin  =   W	kN

T  =   		kN

a  =   arctan WTo		deg

- The relationship defining the magnitude and direction of the tension force at D become

T  =   	kN

=   arctan q⋅xTo	deg

The distance from D to the line of action of the resultant W is equal to half the horizontal distance from C to D.  To sum up the moments about D:

MD  =   0 ------- >	q · x ·  – To · y  =   0	kNm

and solving for  y,	y  =   · x2	m

- This is the equation for a parabola with a vertical axis and its vertex at the origin of coordinates.

- The curve formed by cables loaded uniformly along the horizontal is thus a parabola.

The length of the cable from its lowest point, C , to its support, B , may be obtained by differentiating the Integral of the arc and using the binomial theorem, resulting in the formula:

lB  =   xB · [ 1 + ( )2 – ( )4 + . . .]

The series converges for values of the ratio, yB / xB , less than 0.5 ; in most cases, the ratio is much lower than this and only the first two terms of the series need be taken into consideration.

12.1.2	Catenary

A cable hanging between the supports A and B, carrying a load uniformly distributed along the cable itself, will form a catenary. Cables hanging under their own weight are loaded in this way. Denote by q the load per unit length (measured along the cable) and express it in kN/m. The magnitude, W , of the total load carried by a portion of cable of length, s , extending from the lowest point, C , to a point D is:

W  =   q · s 	kN

- The relationship defining the magnitude of the tension force at D become

T = 										     kN

The distance from D to the line of action of the resultant, W , is not known, as the distance , s , is measured along the cable itself. Refer to the technical literature for the complete formula of the catenary.

The error introduced, by assuming a parabolic shape for cables hanging under their own weight, is small if the cable is tightly stretched.

For the calculation of standard bridges, sufficient accuracy can be achieved by using the formula for the parabola instead of the formula for the catenary.

12.2	Geometry of the Parabola

12.2.1	General Case

- A: Vertex (horizontal tangent)

General formula	:   y 	=    a · x2 + b · x + c

tangent in x	:   tan 	=      = 2 a  x + b

Given: Vertex A (xA , yA) and point B (xB, yB)

Determine a, b, and c with:

yA	=   a · xA2 + b · xA + c

yB	=   a · xB2 + b · xB + c

tan	=   2 a  xA + b = 0

12.2.2	Vertex at the Intersection of the Coordinate Axes

A :  Vetex (horizontal tangent)	Example: xB	=   l/2, and   yB  =   f

General formula	:  y	=   a · x2	a	=

tangent in x	:  tan =   2 a · x	y	=   · x2

Given	:  Point B (xB , yB) , and	tan	=    · x

a  =   	tan	=

12.3	List of Standard Drawings

12.3.1	Standard Working Assembly (Steel) Drawings

| Drawing Title (Bridge type) | Drawing Number | ## Specifications |
| --- | --- | --- |
| Walkway (support) | 01 | For 4 main cables, suspended bridge |
|  | 02 | For 6 main cables, suspended bridge |
|  | 03 | For 8 main cables, suspended bridge |
|  | 04 | For 10 main cables, suspended bridge |
|  | 05 | For 12 main cables, suspended bridge |
|  | 07 | For suspension bridge |
| Walkway Steel Deck | 06 | Steel walkway deck for suspended bridge |
|  | 0DM | Steel walkway deck for suspended bridge (Maintenance) |
|  | 08 | Steel walkway deck for suspension bridge |
|  | ON | Steel walkway deck for suspension bridge ((Maintenance) |
| Windties | 10 | For windguy cable Ø 26 mm |
|  | 11 | For windguy cable Ø 32 mm |
|  | 12 | For windguy cable Ø 36 mm |
|  | 13 | For windguy cable Ø 40 mm |
|  | 14 | For double windguy cable Ø 32 mm |
|  | 15 | For double windguy cable Ø 36 mm |
|  | 16 | For double windguy cable Ø 40 mm |
| Windguy Cable Clamp | 17 | For cable Ø 32 mm |
|  | 18 | For cable Ø 36 mm |
|  | 19 | For cable Ø 40 mm |
| Diagonal Stabilizer (Suspension) | 20A | For 4 main cables |
|  | 20 | For 6 main cables |
|  | 21 | For 8 main cables |
| Stabilizing Cable Clamp (Suspension) | 22 | For 4 main cables |
|  | 23 | For 6 main cables |
|  | 24 | For 8 main cables |
| Suspenders (Suspension) | 31 | For 2 main cables |
|  | 32 | For 4 main cables |
|  | 33 | For 6 main cables |
|  | 34 | For 8 main cables |
| Sidestay Cable An- chorage (Suspension) | 40 | For 1 sidestay cable Ø 26 mm, capacity 130 kN |
| Windguy Cable Anchorage | 41 | For 1 windguy cable Ø 26 mm, capacity 130 kN |
|  | 42 | For 1 windguy cable Ø 32 mm, capacity 195 kN |
|  | 43 | For 1 windguy cable Ø 36 mm, capacity 305 kN |
|  | 44 | For 2 windguy cables Ø 32 mm, capacity 390 kN |
|  | 45 | For 2 windguy cables Ø 36/40 mm, capacity 610 kN |

| Drawing Title (Bridge type) | Drawing Number | ## Specifications |
| --- | --- | --- |
| Main Cable Anchorage (Suspension) | 48A | ## For 4 main cables Ø 26/32 mm, ## capacity 520 / 780 kN, c/c1 = 3.50 m |
|  | 48B | ## For 4 main cables Ø 36/40 mm, ## capacity 1220 kN, c/c1 = 3.50 m |
|  | 49 | ## For 4 main cables Ø 36/40 mm, ## capacity 1220 kN, c/c1 = 4.00 m |
|  | 50 | ## For 6 main cables Ø 36/40 mm, ## capacity 1830 kN, c/c1 = 4.00 m |
|  | 51 | ## For 8 main cables Ø 36/40 mm, ## capacity 2440 kN, c/c1 = 4.00 m |
| Tower -stay Cable | 52 | ## For tower erection |
| Main Cable Anchorage (drum-type) (Suspended) | 60 | ## For 2 main cables, capacity 610 kN (For Suspension) |
|  | 61 | ## For 4 main cables, capacity 1220 kN |
|  | 62 | ## For 6 main cables, capacity 1830 kN |
| Handrail Cable Anchorage (Suspended) | 63(26) | ## For 2 handrail cables Ø 26 mm, capacity 260 kN |
|  | 63(32) | For 2 handrail cables Ø 32 mm, capacity 390 kN |
|  | 63(36) | For 2 handrail cables Ø 36 mm, capacity 495kN |
| Main Anchorage (open-type) (Suspended) | 64 | ## For 8 main cables and 2 handrail cables Ø 40 mm, capacity 2440 and 610 kN |
|  | 65 | ## For 10 main cables and 2 handrail cables Ø 40 mm, ## capacity 3050 and 610 kN |
|  | 66 | ## For 12 main cables and 2 handrail cables Ø 40 mm, ## capacity 3660 and 610 kN |
|  | 67 | ## Saddle and Accessories for 8 main cables |
|  | 68 | ## Saddle and Accessories for 10 main cables |
|  | 69 | ## Saddle and Accessories for 12 main cables |
| Cable Rock Anchorage | 70 | ## For 1 windguy cable Ø 26 mm, capacity 90 kN |
|  | 71 | ## For 1 windguy cable Ø 32 mm, capacity 180 kN |
|  | 72 | ## For 1 windguy cable Ø 36/40 mm, capacity 280 kN |
| Walkway/Tower Anchorage (Suspension) | 90 | ## For spanning cable Ø 26 mm, capacity 260 kN, ## c/c1 = 3.50 m, c/c2 = 38.3 cm |
|  | 91 | ## For spanning cable Ø 32 mm, capacity 390 kN, ## c/c1 = 3.50 m, c/c2 = 48.8 cm |
|  | 92 | ## For spanning cable Ø 32 mm, capacity 390 kN, ## c/c1 = 4.00 m, c/c2 = 55 cm |
|  | 93 | ## For spanning cable Ø 36/40 mm, capacity 610 kN, ## c/c1 = 4.00 m, c/c2 = 56.6 cm |
| Tower (Suspension) | 100 - 107 | ## Tower base element |
|  | 110 - 117 | ## Tower intermediate element |
|  | 120 - 130 | ## Tower top element |
|  | 135 - 138 | ## Tower saddle |
| Tower (Suspension) | 140 | ## Guide to LSTB standard towers |
|  | 145 - 160 | ## Assembly of tower |

12.3.2	Standard Structural (Construction) Drawings

| Drawing Title (Bridge type) | Drawing Number |  | Specifications |  |
| --- | --- | --- | --- | --- |
| Sidestay Cable Foundation (Suspension) | 40/1 |  | 1 cable 0 26 mm |  |
| Windguy Cable Foundation | 41/1 |  | 1 cable Ø 26 mm, on soil |  |
|  | 41/2 |  | 1 cable Ø 26 mm, on rock |  |
|  | 41/3 |  | 1 cable Ø 26/32/36 mm, on soil (with foot) |  |
|  | 42/1 |  | 1 cable Ø 32 mm, on soil |  |
|  | 42/2 |  | 1 cable Ø 32 mm, on rock |  |
|  | 43/1 |  | 1 cable Ø 36/40 mm, on soil |  |
|  | 43/2 |  | 1 cable Ø 36/40 mm, on rock |  |
|  | 44/1 |  | 2 cables Ø 32 mm, on soil |  |
|  | 44/2 |  | 2 cables Ø 32 mm, on rock |  |
|  | 45/1 |  | 2 cables Ø 36/40 mm, on soil |  |
|  | 45/2 |  | 2 cables Ø 36/40 mm, on rock |  |
| Main Cable Foundation (Suspension) | 48A/1 |  | 4 main cables Ø 26/32 mm, c/c1 = 3.50 |  |
|  | 48B/1 |  | 4 main cables Ø 36/40 mm, c/c1 = 3.50 |  |
|  | 49/1 |  | 4 main cables Ø 32/36/40 mm, c/c1 = 4.00 |  |
|  | 50/1 |  | 6 main cables Ø 36/40 mm, c/c1 = 4.00 |  |
|  | 51/1 |  | 8 main cables Ø 36/40 mm, c/c1 = 4.00 |  |
| Main Cable Deadman Foundation (Suspension) | 48A/2 |  | 4 main cables Ø 26/32 mm, c/c1 = 3.50 |  |
|  | 48B/2 |  | 4 main cables Ø 36/40 mm, c/c1 = 3.50 |  |
|  | 49/2 |  | 4 main cables Ø 32/36/40 mm, c/c1 = 4.00 |  |
|  | 50/2 |  | 6 main cables Ø 36/40 mm, c/c1 = 4.00 |  |
|  | 51/2 |  | 8 main cables Ø 36/40 mm, c/c1 = 4.00 |  |
| Tower-stay cable Foundation for Tower Erection (Suspension) | 52/1 |  | 1 cable on one side for tower height ≤ 25.23 m |  |
|  | 52/2 |  | 2 cables on two sides for tower height ≤ 25.23 m |  |
|  | 52/3 |  | 2 cables on one side for tower height  27.73 – 35.21m |  |
| Main Foundation (drum-type) (Suspended) | 61/1(26) |  | 4 main cables Ø 32/36/40 mm and 2 handrail cables Ø26 mm on soil, related drawings 61 & 63(26) |  |
|  | 61/2(26) |  | 4 main cables Ø 32/36/40 mm and 2 handrail cables Ø26 mm on rock, related drawings 61 & 63(26) |  |
|  | 61/1(32) |  | 4 main cables Ø 32/36/40 mm and 2 handrail cables Ø32 mm on soil, related drawings 61 & 63(32) |  |
|  | 61/2(32) |  | 4 main cables Ø 32/36/40 mm and 2 handrail cables Ø32 mm on rock, related drawings 61 & 63(32) |  |
|  | 62/1(26) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 26 mm on soil, related drawings 62 & 63(26) |  |
|  | 62/2(26) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 26 mm on rock, related drawings 62 & 63(26) |  |
|  | 62/1(32) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 32 mm on soil, related drawings 62 & 63(32) |  |
|  | 62/2(32) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 32 mm on rock, related drawings 62 & 63(32) |  |
|  | 62/1(36) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 36 mm on soil, related drawings 62 & 63(36) |  |
|  | 62/2(36) |  | 6 main cables Ø 36/40 mm and 2 handrail cables Ø 36 mm on rock, related drawings 62 & 63(36) |  |
| Drawing Title (Bridge type) |  | Drawing Number |  | Specifications |
| Main Foundation (open-type) (Suspended) |  | 64/1 |  | 8 main cables, on soil, related drawings 64 & 67 |
|  |  | 64/2 |  | 8 main cables, on rock, related drawings 64 & 67 |
|  |  | 65/l |  | 10 main cables, on soil, related drawings 65 & 68 |
|  |  | 65/2 |  | 10 main cables, on rock, related drawings 65 & 68 |
|  |  | 66/1 |  | 12 main cables, on soil, related drawings 66 & 69 |
|  |  | 66/2 |  | 12 main cables, on rock, related drawings 66 & 69 |
| Walkway/Tower Foundation (Suspension) |  | 90/1 |  | Spanning cable Ø 26 mm, without foot, c/c1 = 3.50, c/c2 = 38.3 cm |
|  |  | 90/2 |  | Spanning cable Ø 26 mm, with foot, c/c1 = 3.50, c/c2 = 38.3 cm |
|  |  | 91/1 |  | Spanning cable Ø 32 mm, without foot, c/c1 = 3.50, c/c2 = 48.8 cm |
|  |  | 91/2 |  | Spanning cable Ø 32 mm, with foot, c/c1 = 3.50, c/c2 = 48.8 cm |
|  |  | 92/1 |  | Spanning cable Ø 32 mm, without foot, c/c1 = 4.00, c/c2 = 55.0 cm |
|  |  | 92/2 |  | Spanning cable Ø 32 mm, with foot, c/c1 = 4.00, c/c2 = 55.0 cm |
|  |  | 93/1 |  | Spanning cable Ø 36/40 mm, without foot, c/c1 = 4.00, c/c2 = 56.6 cm |
|  |  | 93/2 |  | Spanning cable Ø 36/40 mm, with foot, c/c1 = 4.00, c/c2 = 56.6 cm |
| Staircase (Suspension) |  | 94/1 |  | For medium to unfavorable soil conditions |
|  |  | 94/2 |  | For good soil conditions |

12.3.3	Special Design Drawings

| Drawing Title (Bridge type) | Drawing Number | Specifications |
| --- | --- | --- |
| Windguy Clamp for Direct Windguy Cable | 25 | For windguy cable Ø 26 mm, main cable Ø 36/40 mm (Working and Assembly Drawing) |
| Spacing Clamp for Main Cables (Suspension) | 28 | For 4 main cables Ø 32 mm (Working and Assembly Drawing) |
| Combined Main Cable and Windguy Cable Foundation (Suspension) | 49/3 | 4 main cables Ø 32/36/40 mm, c/c1 = 4.00 m, 2 windguy cables Ø 36/40 mm, c/c1 = 2.00 m, (Structural Drawing), related drawings: 49 and 45 |
| Main Cable Tunnel Foundation (Suspension) | 60/3 | 2 main cables (Structural Drawing), related drawing : 60 |
| Staircase Combined with Main Foundation (Suspended) | 60/4 | 2 main cables (Structural Drawing), related drawings: 60 and 63 |
| Walkway and Tower Foundation | 90/3 | Structural Drawing |
| Walkway Foundation (Suspension) | 91/3 | Spanning cable Ø 26/32 mm (Structural Drawing) |
| Walkway and Windguy Cable Anchorage | 91/4 | For Spanning cable Ø 32 mm and windguy cable Ø 32 mm (Structural Drawing) |
| Windguy Stay Strut (Working and Assembly Drawings) (Suspension) | 175-175/1 | Length 4.50 m, for windguy cable Ø 26 mm, |
|  | 176-176/1 | Length 6.50 m, for windguy cable Ø 32 mm |
|  | 177-177/1 | Length 8.30 m, for windguy cable Ø 36 mm |

12.4	Design Examples

On the following pages, two design examples, are compiled.

One for a suspended bridge, refer to

12.4.1	Suspended Bridge

and, one for a suspension bridge, refer to

    - Suspension Bridge

The calculations have been carried out according to the procedures given in this manual. In cases where similar calculation has to be carried out several times, they are presented for one example only. E.g., the calculation of the windguy arrangement is only carried out for the suspended bridge. Reference has to be made to this example for the windguy arrangement of the suspension bridge. The "General Arrangements" related to the two design examples are contained in:

LSTB Technical Manual Volume C, Standard Design Drawings,  Design Examples.

  - Cost Estimate Norms/Formats

12.6	Survey Form and Checklist