Survey FORM

| Bridge Number : ____________ | Name : ________________________ |
| --- | --- |
| Span* & Type : ____________ | River : ________________________ |
| Coordinates |  |
| N : ________________________ | E : ________________________ |
| Map No. : ________________________ | Altitude : ________________________ |
| Left Bank | Right Bank |
| Village : ________________________ | Village : ________________________ |
| District : ________________________ | District : ________________________ |
| Region : ________________________ | Region : ________________________ |

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Surveyed by: |  |  |  |
| Checked by: |  |  |  |
| Approved by: |  |  |  |

* approximate span and type of bridge

Table of Content

1. 	Introduction

2. 	Socio-Economic Data

2.1 Traffic Counting/Expected Time Saving

2.2 Poverty Orientation

3. 	Feasibility Survey

3.1 Site Selection

3.2 Local Support of the Project

3.3 Type of River

3.4 Existence of Local Bridge or Crossing

3.5 Availability and Cost of Construction Materials

3.6 Labor District Rates

3.7 Accessibility, Distances

4. 	Geological and Geotechnical survey

4.1 Rocky Bank and Slope

4.2 Soil Bank and Slope

4.3 Soil Test Pit Dimension

4.4 Sampling

5. 	Topographical Survey

5.1 Description of Pegs and Bench-Marks

5.2 Triangulation and Elevation of Pegs and Bench-Marks

5.3 Tacheometric Survey

6. 	Preliminary Design

6.1 High Flood Level

6.2 Sketch of the bridge and the bridge site

6.3 Preliminary Design Data

7. 	Photographs

8. 	Compilation of Data and Reporting

8.1 Feasibility and site selection

8.2 Geological Survey and Determination of Design Parameters

8.3 Topographical Survey, preliminary design and photographs

# 1.	Introduction

This Survey Check List is prepared for experienced consultants and TBS/SBD engineers. The surveyor should be very familiar with the standard design, “Survey, Design and Construction of Trail Suspension Bridges for Remote Areas” developed by Trail Bridge Section/Suspension Bridge Division and Helvetas. In addition, the surveyor must have the basic knowledge in the art of engineering geology. He must be able to judge whether a site is feasible or geologically risky. In the latter case, an Engineer Geologist is necessary to carry out a detailed geological study. In such exceptional cases, the Engineer Geologist of TBS/SBD will provide the additional required information.

Prior to visiting the site, the “Preliminary Survey Report” prepared by the Engineer Geologist, TBS/SBD has to be studied. Without such report, no survey should be carried and the works will also not be accepted by the Trail Bridge Section/Suspension Bridge Division.

The following Manuals and Check-Lists are required for executing a survey according to the requirements of TBS/SBD:

- Survey and Design and Construction of Trail Suspension Bridges for Remote Areas: Volume A: Design, revised version

- Survey and Design and Construction of Trail Suspension Bridges for Remote Areas: Volume B: Survey

- Check-List No. 1	:	Preliminary Study For Alternative Sites

- Check-List No. 2	:	Slope study & Site Selection

- Check-List No. 3	:	Rock Investigation

- Check-List No. 4	:	Geological Plane Investigation (GPI)

- Check-List No. 5	:	Transit Cross-Profile

- Check-List No. 6	:	Soil Investigation

- Check-List No. 7	:	Triangulation

- Check-List No. 8	:	Summary Of Triangulation And Elevations

- Check-List No. 9	:	Tacheometry

- Check-List No. 10	:	Design Parameters

The surveyor is requested to follow the instructions very precisely. In case of an incomplete or wrong survey, refer in particularly to Chapter 7 below, The office will strictly not accept and approve such submitted jobs.

# 2.	Socio-Economic Data

## 2.1	Traffic Counting/Expected Time Saving

Traffic counting has to be conducted at the traditional crossing point/s for at least three consecutive days.  If, in case the proposed crossing site is not being used by the traffic due to unavailability or risky river crossing facility (bridge, ferry, cable car, temporary bridge) the traffic study has to be conducted at the nearest crossing site at the upstream or downstream of the river.  The average  daily traffic of the proposed crossing site should be interpreted based on the origin and destination survey with proper justification.

Expected time saving after bridge construction should be estimated on the basis of  detouration of the crossing point at present.

Date and day of traffic survey: ________________ time (From ………… To ……….)

Human Traffic

|  | Traffic category (numbers) |  |  |  |  |  | Expected |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S. | Porters | Travellers |  | Origin | Destination | Remarks | time saving |
| N. |  | Male | Female |  |  |  | after bridge construction |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |
| 9. |  |  |  |  |  |  |  |
| 10. |  |  |  |  |  |  |  |
| 11. |  |  |  |  |  |  |  |
| 12. |  |  |  |  |  |  |  |
| 13. |  |  |  |  |  |  |  |
| 14. |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |
| Total |  |  |  |  |  |  |  |

Animal Traffic

| S. N. | Traffic category (numbers) |  | Origin | Destination | Remarks | Expected time saving after bridge construction |
| --- | --- | --- | --- | --- | --- | --- |
|  | Porters | Animals |  |  |  |  |
| 1. |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |
| 9. |  |  |  |  |  |  |
| 10. |  |  |  |  |  |  |
| 11. |  |  |  |  |  |  |
| 12. |  |  |  |  |  |  |
| 13. |  |  |  |  |  |  |
| 14. |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |
| Total |  |  |  |  |  |  |

Summary of Traffic Survey Results

| S. N. | Date | Daily traffic (numbers) |  |  |  |  | Expected time saving after bridge construction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Goods traffic |  | Non-goods traffic |  |  |  |
|  |  | Porter | Pack animals | Travellers |  | Animals |  |
|  |  |  |  | Male | Female |  |  |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |
| Average Daily Traffic |  |  |  |  |  |  |  |

Note: 	Cattle, mules, buffaloes, horses: 1 unit

Goats, sheep: ½ unit

## 2.2	Poverty Orientation

Identify number of people and households who are below the poverty line (those who have daily per capita income less than US $ 1) within the influence area of the proposed bridge.  Investigate the possible positive or negative impact on these people due to the construction of the proposed bridge.  Recommend measure to minimize negative impact on the poor.

Household with daily per capita income less than US $ 1	=

Household with daily per capita income            > US $ 1	=

Total household	=

Possible Negative impacts on the poor:

Possible Positive impacts on the poor:

Recommend Measures to minimize negative impacts on the poor:

# 3.	Feasibility Survey

## 3.1	Site Selection

Carry out feasibility studies of at least three alternative sites in concordance with the “Preliminary Survey Report”.

For all three sites do the following:

- Fill in Check List No. 1: Preliminary Study for Alternative Sites

- Draw a sketch and a cross section of each alternative site, include the trail system and trails to be build in future, existing and planned roads, present and future river crossings, distribution of population, etc.

- Take photographs i.e. an overall view and both banks

Select the best site by considering criteria like trail system, span, type of bridge, bank and slope conditions, structure of rocks, etc.

Decide whether a detailed geological investigation is required. If not, continue with the detailed survey. If yes, an Engineer Geologist is necessary to carry out the detailed geological and topographical survey.

## 3.2	Local Support of the Project

Do the local people agree with the selected site? Interview at least three beneficiaries, VDC and DDC members, etc.

| No |  | Name |  | Address |  | Function |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |

In addition, get a letter of confirmation regarding the site selection from the DDC Chairman.

## 3.3	Type of River

Report the nature of the river at the crossing point based on the following simple classification:

|  | Major | - Unfordable throughout the year |
| --- | --- | --- |
|  | Medium | - Fordable at some places during dry season with difficulties or with temporary bridges |
|  | Minor | - Easily fordable during dry season |

## 3.4	Existence of Local Bridge or Crossing

Describe the existing bridge or crossing (if any):

Bridge Type	:	____________________	Span (m)	:	_________

Year of Construction	:	____________________	Free-Board (m)	:	_________

Actual condition of Bridge	:	________________________________

Constructed by	:	________________________________

Organization responsible for maintenance	:	________________________________

Replacement 	  :	          Necessary	        Not necessary

Attach a sketch, make photographs (overall view, relevant details).

## 3.5	Availability and Cost of Construction Materials

| Cost Estimate Pos. No. | Item Description | Rates |  | Haulage Distance | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | NC | Unit | meter |  |
| 550/60 | Sand |  |  |  |  |
| 540/50/60 | Rubble, Boulders |  |  |  |  |
| 555 | Wood for Form work |  | m3 |  |  |
| 571/72 | Bamboo |  | pc |  |  |
| 571/72 | Dori |  | kg |  |  |

## 3.6	Labor District Rates

| Cost Estimate Pos. No. | Item Description | Rates |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | NC | Unit |  |
| 500/600 | Skilled Labor |  | md |  |
| 500/600 | Unskilled Labor |  | md |  |
| 410 | Porter |  | md |  |

## 3.7	Accessibility, Distances

Existing distances:

| Nearest | Name | Distances |  |
| --- | --- | --- | --- |
|  |  | Miles | Porter days |
| Roadhead (Trucks) |  |  |  |
| Roadhead (Tractors) |  |  |  |
| Airport |  |  |  |

From which source are this information ? 	___________________________________

Will in future a new road be build or planned? If yes, give the following information:

| Nearest | Name | Distances |  |
| --- | --- | --- | --- |
|  |  | Miles | Porter days |
| Roadhead (Trucks) |  |  |  |
| Roadhead (Tractors) |  |  |  |

Planned date of completion: 	______________________________________________

From which source are this information? 	____________________________________

# 4.	Geological and Geotechnical survey

Study carefully and fill in the respective Check-Lists as indicated in the “Preliminary Survey Report”.

## 4.1	Rocky Bank and Slope

- Check List No. 2:	Slope Study & Site Selection, for each bank

- Check List No. 3:	Rock Investigation, for each rock sample

- Check List No. 4:	Geological Plane Investigation,
	for exposed rock around the anchorage blocks and banks

- Check List No. 5:	Transit Cross Profile, for geological difficult banks

- Check List No. 6:	Soil Investigation,
	for open test pits, if the rock is covered by soil

## 4.2	Soil Bank and Slope

- Check List No. 2:	Slope Study & Site Selection, for each bank

- Check List No. 6:	Soil Investigation, 
	for each main anchorage block and tower foundation

## 4.3	Soil Test Pit Dimension

The minimum length and breadth of the pit should be 0.80 x 1.50 meter.

The depth for soil anchorages must be at least  2.00 meter for tower foundations and main anchorages and 1.50 meter for windguy anchorages or at least up to the estimated foundation base or below to the rock face. For highly weathered and fractured rock surfaces, the depth must be up to the fresh rock.

## 4.4	Sampling

In order to be able to determine the soil and rock parameters the following samples are essential and must therefore be transported back to the head office:

Rock	:	One fresh sample from each anchorage and bank for each different type of rock. And, one sample for different grade of weathering.

Soil	:	One mixed sample  from top to bottom and, one sample1 from the bottom layer of the pit for each pit.

# 5.	Topographical Survey

## 5.1	Description of Pegs and Bench-Marks

Draw a sketch and describe the pegs by indicating type of peg, distances, angles and azimuths.

| Right Bank |  |  |  |
| --- | --- | --- | --- |
| Point |  | Marking | Elevation |
| Left Bank |  |  |  |
| Point |  | Marking | Elevation |

Indicate distances, azimuths, angles, north direction and flow of river.

## 5.2	Triangulation and Elevation of Pegs and Bench-Marks

Refer to Check-List Nos. 7 and 8.

- The base distance “d” must be at least 20 % of the distance D

- The distance “d” must be accurately measured horizontally. Measure it several times and take the mean

- The triangulation has to be made twice, if the difference of D is mote than 0.25%, repeat it

- If D < 150 meter: The triangulation can be made in one section

- If D > 150 meter: The distance d has to be divided into two sections. Ref. Volume B: Survey, page 189

- Make all the calculation before continuing with the tacheometric survey

## 5.3	Tacheometric Survey

Refer to Check-List No. 9.

- Based on the preliminary design, refer to paragraph 5 below, the area to be covered by the detailed survey can be determined

- The staff must be kept in a vertical position. A slight inclination from the vertical will result in a large error in the distance

- Special attention has to be paid to the area near the proposed anchorage blocks, at breaking points of the terrain, paddy fields, existing local crossings, drainage, trails, houses, big boulders, etc.

- Tacheometric reading has to be made for triangulation points, reference points and bench marks for cross checking purpose

- The location and elevation of the soil test pits and rock outcrops have to be measured

- Set up the theodolite on permanent pegs only

# 6.	Preliminary Design

Refer to LSTB Technical Manual,Volume B: Survey, chapter 6.5 for detailed information.

## 6.1	High Flood Level

Since in remote areas hydrological data are normally not available, the design flood level has to be based on the highest ascertainable historic flood level by inquiry among local inhabitants and commented by observations at the site by flood marks on the river banks, presence of forests, etc.

## 6.2	Sketch of the bridge and the bridge site

Measure a profile in the axis line A-B by using the tacheometric method. Draw a sketch of the site in scale 1:500 in plan and cross section with all important information.

## 6.3	Preliminary Design Data

Suspended Bridge:	Span range	:	20 - 350 meter

Span intervals	:	span = 1.2 i

Dead load sag	:	fd = span/23

Suspension Bridge:	Span range	:	30 - 280 meter

Span intervals	:	span = 2.40i  + 2.20

Camber	:	cd = 3 % of span

Dead load sag	:	fd = 12 % of span

Anchorages/Foundations:	The approximate position must be determined.

Wind bracings:	The approximate layout including the location of the anchorages must be determined for spans exceeding 50 meters.

Free-Board:	Determine the elevation of the bridge including the wind bracings in such a way that the minimum free-board of five meter is maintained.

Special Design:	In special topographic conditions, modifications of the standard bridges may be required. There are different possibilities of adopting the design to the specific site conditions like tunnel anchorages, suspension bridges with one tower, etc. For further details refer to LSTB Technical Manual, Volume A: Design.

# 7.	Photographs

For the Bridge Record of the District and the internal checking propose good photographs are very essential. Therefore, a survey without sufficient and good photos will not be accepted by the office.

The following photographs have to be taken:

- Overall view, anchorages, walkway, etc. of the existing crossing, if any

- Overall view of the bridge site from downstream and upstream

- Overall view of the right and left bank

- Position all foundations and anchorages

- Soil test pits and locations where rock samples are taken. Do not forget to put the staff or a scale in visible position for references

If one picture does not cover the necessary area, take several pictures from the same spot with sufficient overlapping.

# 8.	Compilation of Data and Reporting

The survey report has to be in concordance with the following structure and it must contain the following information:

## 8.1	Feasibility and site selection

- Complete Check List No. 1:	Preliminary Study for Alternative Sites for all sites

- Draw a sketch and a cross section of each alternative site, include the trail system and trails to be build in future, existing and planned roads, present and future river crossings, distribution of population, etc.

- Describe each alternative site separately and compile the photographs

- Give a detailed justification, pros and cons, of the selected site

- Describe the existing trail system and compare it with the situation after the new bridge will be built

- Make your own firm judgment with regard to the feasibility of the proposed bridge site in terms of socio-economic as well as technical as aspects.

## 8.2	Geological Survey and Determination of Design Parameters

- Complete Check List No. 2:	Slope Study & Site Selection, for each bank

- Complete Check List No. 3:	Rock Investigation, for each rock sample

- Complete Check List No. 4:	Geological Plane Investigation, for exposed
	rock around the anchorage blocks and banks;

- Complete Check List No. 5:	Transit Cross Profile, for geological difficult
	banks

- Complete Check List No. 6:	Soil Investigation, for each open test pit

- For rocky banks carry out a stability analysis according to Volume B: Survey, chapter 5.41

- Submit all soil and rock sample, ref. Chapter 3.4 above. Give reference numbers where geological data and/or samples were collected

- Fill in Check List No. 10:  Design Parameters

- Make a report with comments on the detailed geological survey with emphasis on the geological limits and possible failures of the slopes and banks

## 8.3	Topographical Survey, preliminary design and photographs

- Complete Check List No. 7: Triangulation

- Complete Check List No. 8: Summary of Triangulation and Elevations

- Complete Check List No. 9: Tacheometry

- Complete the preliminary bridge design and give recommendation for the design

- Plot the contour line map and a section along the bridge axis in scale 1: 200 on a white paper with 0.60 meter width. Indicate permanents pegs, bench-marks, location of soil test pit, existing crossings, trails, etc

- Compile all photographs. Give a comment, indicating all the features of the picture

Take at least 1.5 kg per sample for visual classification and at least 2.5 kg for laboratory test i.e. sieve analysis, atterberg limits and shear test.  Fill soil sample tag for each sample and bind it to the sample bag.